Reducing Greenhouse Gas Emission of Construction Equipment at Construction Sites: A Field Study Approach

Nina Szamocki1; Min-Koo Kim, M.ASCE2; Changbum R. Ahn, M.ASCE3 and Ioannis Brilakis, M.ASCE4

1Part-Qaulified Patent Attorney, Haseltine Lake LLP, 120 Redcliff Street, Bristol, The United Kingdom. E-mail: n.szamocki@gmail.com

2Assistant Professor, Department of Building and Real Estate, The Hong Kong Polytechnic University, Hung Hom, Kowloon, Hong Kong. (corresponding author) E-mail: minkoo.kim@polyu.edu.hk

3Associate Professor, Department of Construction Science, Texas A&M University, 3137 TAMU, College Station, TX 77843-3137, USA Email: ryanahn@tamu.edu

4Laing O’Rourke Reader, Department of Engineering, University of Cambridge, Trumpington Street, Cambridge, The United Kingdom. E-mail: ib340@cam.ac.uk

Abstract

Construction operations of construction equipment generate significant greenhouse gas emissions including carbon emissions and diesel exhaust emissions. While improving operational efficiency of construction activities implemented by construction equipment could result in significant emission reduction, there is still a lack of practical knowledge on the impact of operational adjustments applicable in real-world operations. The objectives of this study are to identify common inefficiency of real-world construction equipment activities and quantify emission saving potentials from operation-level adjustments of construction activities. In order to achieve the goals, three construction sites were used as case studies to identify common sources of excessive emissions on construction sites as well as possible opportunities for their reduction. As the means of quantifying potential gas emission reduction, a simplistic method that estimates equipment cycle time and can be easily replicated in practice is introduced. It was found from the case studies that a typical problem on a construction site is unnecessary idling of equipment. In addition, findings of this study indicate that the integration of careful planning on construction equipment activities with appropriate equipment selection could contribute to the reduction of carbon gas emissions as well as savings of construction cost. It is envisaged that this study contributes to the body of knowledge by 1) providing new findings in common inefficiency of construction operation in construction sites through field studies, and 2) quantifying the potential gas emission reduction with the simplistic method.

Key words: Sustainable development; gas emissions; construction operations, efficiency analysis, construction equipment, field data

Introduction

The building and construction sector has drawn much attention regarding sustainable development because buildings and infrastructure account for a large share of energy consumption and greenhouse gas (GHG) emissions [United Nations Environment Programme Sustainable Buildings & Construction Initiative (UNEP SBCI) 2018]. Amongst the scientific community, there is a consensus that the average global temperature is rising and this is due to an increase of greenhouse gases including carbon dioxide (CO2) in the atmosphere. Such an increase of global temperature gives an unacceptable risk of dangerous changes to the climate and as a result, the members of the European Union (EU) have committed to cutting emissions by 20% by 2020 and 40% by 2030 from the 1990 levels in order to prevent an average temperature increase of 1.2 °C from today’s temperature [EU 2018b]. Cleary every industry in the EU is being put under pressure to become more energy efficient and the construction industry is no exception.

The processes associated with constructing buildings and civil infrastructures generate significant environmental effects. In particular, gas emissions from construction equipment and construction-related transportation highly contribute to local air quality degradation and global warming [Ahn and Lee 2013a]. According to a report [CEMA & CECE 2011], construction equipment is responsible for 2% of emissions within the transport sector in the EU, which all equates to on-site construction activities being responsible for approximately 0.5% of total emissions in the EU. In addition, it is estimated that over 10% of the total costs of construction are attributed to fuel costs from on-site activities and that fuel use accounts for 34% of the operation costs of the average excavator [CECE 2014]. Although GHG emissions from construction activities are relatively small compared to the embodied emissions and operational emissions in the project life cycle, its significance in terms of the intensity and amount of total GHG emissions from the construction industry has been recognized and emphasized by many previous studies [Ahn et al. 2010, Ahn and Lee 2013a and Ahn et al. 2013b]. For example, GHG emissions from construction activities are mostly direct emissions from equipment, which are hard to control, compared to other indirect GHG emission sources during material manufacturing and building operation phases. They are emitted within a relatively short period (several months to several years), while embodied emissions and operational emissions are generated for a long period (several decades). In addition, although construction emissions from one project may not be that large, aggregated emissions from a sheer number of construction projects make the construction industry the third largest contributor among all the industry sectors. In this regard, reducing the gas emissions of construction operations at construction sites is important by identifying and quantifying the inefficiency of current construction operations. It is commonly said from the construction equipment industry that there is a natural incentive to provide fuel efficient equipment, as fuel is a significant proportion of the operating costs in construction [CECE 2014]. For this reason, most construction equipment fleet owners are now concerned with reducing their fuel use and improving fuel efficiency to reduce fuel expense.

Improving the operational efficiency of equipment operation has been considered as the cost effective alternative to improving fuel efficiency and reducing GHG emissions from equipment operations. However, there is a lack of empirical studies examining possible modifications that can be made in daily operation of equipment fleet and quantifying the impact of such modifications of equipment operation. In this context, this paper looks into on-site construction processes and possible operational adjustments for emissions reductions and conducted three different field studies to understand a phenomenon of this issue rather than trying to solve this problem. Specifically, this paper aims to: (1) identify common inefficiency of construction activities on sites; and (2) quantify the carbon emission reduction potential for the problems of different types of construction operations identified during the construction phase.

The major contribution of this paper is to quantify the emission saving potential from slight adjustments of real-world equipment operations, for example, matching hauling equipment with loading equipment, adjusting loading actions of a loader, selecting a nearer dumping site, and reducing idling. While it is well known that such adjustments to address operational efficiency in equipment operation could bridge emission reduction to some extent, it has not been attempted to quantify how much emissions can be actually reduced by minor operational adjustments that could be easily implemented in real-world operations. This study fills such a knowledge gap by using the data captured from real-world observations in three case studies and quantifying potential emission reductions using the industry standard practice in estimating equipment operation productivity.

The paper is organized as follows. Research background and review on previous related studies are presented in the next section. The methodology of this study including the procedure for the calculation of gas emissions from construction equipment is then described. Next, three field case studies are described and the findings from the field studies are presented, followed by discussions on the findings of the case studies. Finally, it concludes with a brief summary and future work of this study.

Background and Previous Studies

Current Gas Emissions Regulations

There are existing emissions regulations for light-duty vehicles such as cars and vans, and non-road mobile machinery. The implementation of light-duty vehicles is slightly different from that for non-road mobile machinery. The regulations for light-duty vehicles are given for carbon dioxide emissions and are limited in terms of gCO2/km [EC 2018a]. On the other hand, the regulations for non-road mobile machinery including construction equipment impose limits on emissions of carbon monoxide (CO), hydrocarbons (HC), nitrogen oxides (NOX) and particulate matters in terms of gCO2/kWh [EC 2018a]. This difference in limitation is due to the large variety in size and power of non-road mobile machines and the difference in purpose of cars and construction equipment; while the main aim of a car is to travel distances, whereas construction equipment may be doing useful work whilst standing still and therefore must be regulated without considering distance travelled. For that reason, the regulations for the non-road mobile machines are set for ranges of engine power, where an engine to be installed in non-road mobile machinery is classed according to its potential power output. This also takes into account the differences in weight since some equipment must necessarily be more powerful and therefore heavier than other equipment.

Previous Related Studies

There has been many research efforts studied the environmental impacts of construction project activities. Life-cycle assessment (LCA) is one of the popular methods to evaluate the degree of environmental impact of products or services [Hong et al. 2014; Ortiz et al. 2010] and several studies have used LCA as a tool to quantify the environmental impact of buildings [Suh and Lippiatt 2012; Lippiatt and Bovles 2001; Facanha and Horyath 2005; Lee et al. 2009]. In addition, LCA can show various types of environmental impacts including CO2 emissions of a building during its whole life cycle. Most of these studies, however, have been focused on the maintenance and operation phases to identify and quantify the impact on CO2 gas emissions since these phases account for the largest part of energy consumption during the life cycle of facilities.

Over the last several years, there have several studies aiming to estimate CO2 emissions during the construction phase. Hong et al. (2014) developed a model to assess the CO2 emissions in the construction phase and a case study was conducted on the three different processes of material manufacturing, transportation and on-site construction to investigate the applicability of the developed model. Sandanayake et al. (2016) also developed a model to estimate the emissions at foundation construction and a process-based quantitative method was proposed for calculating the emissions coming from materials, transportation and equipment usage. In addition, Trani et al. (2016) predicted on-site fuel consumption and the resulting carbon dioxide emissions due to earthworks-related processes during the planning stage of residential projects. The prediction was made based on information contained in construction project documents and the definition of equipment load factors. On top of that, a research group presented a methodology for incorporating the analysis of operational efficiency into quantifying the amount of exhaust emissions from construction operations [Ahn and Lee 2013]. The same research group also proposed an integrated framework for estimating and monitoring gas emissions of construction operations [Ahn et al. 2013]. Although these previous studies attempted to estimate the amount of gas emissions, the estimations rest on the planned activity data of construction equipment, which may not provide an accurate estimation of the actual gas emission. To measure the actual gas emissions of construction equipment, Kim et al. (2015) presented a direct measurement method to quantify the carbon emission of construction equipment onsite at a construction project. They used a flow velocity measurement sensor to measure CO2 emission of back-hoe, dump truck, dozer, and rollers in real time based on flow velocity measurement sensors and compared the actual data with theoretical equations.

Recently research efforts on reducing gas emissions of construction operations have been made. One of them, Lewis et al. (2012) evaluated the impact of idling on fuel use and CO2 emissions of diesel construction equipment and conducted a field study to collect field data from heavy-duty diesel equipment to quantify the impacts of engine idling on the fuel use and gas emissions. Gallivan (2010) investigated mitigation measures for construction activity operations. In the report, as the means to reduce CO2 emissions, three recommendations of 1) reducing equipment activity, 2) improving fuel economy, and 3) using alternative technologies were suggested. In addition, Kim et al. (2012) quantified the carbon emissions of construction equipment based on the design documents of the road construction projects by considering the capacity of different type equipment, and a guidance was made to improve equipment productivity and reduce gas emissions.

Gaps in Knowledge and Objectives

There have been several studies on gas emissions of construction equipment during construction operations with the ultimate purpose of reducing gas emissions. From the prior studies, improving the operational efficiency of equipment operation has been considered as the cost effective alternative to improving fuel efficiency and reducing CO2 emissions from equipment operations. However, there is a lack of empirical studies examining possible modifications that can be made in daily operation of equipment fleet and quantifying the impact of such modifications of equipment operation. Although some recent studies have started to find current efficiencies of construction activities operated by construction equipment and then suggest mitigation measures for reducing gas emissions of construction equipment, they has not attempted to quantify how much emissions can be actually reduced by minor operational adjustments that could be easily implemented in real-world operations. The uniqueness of this paper lies in the identification and quantification of the impact of slight adjustments of real-world equipment operations on the gas emission savings at construction sites for the first time. This study fills such a knowledge gap by using the data captured from real-world observations in three case studies and quantifying potential emission reductions using the industry standard practice in estimating equipment operation productivity. This paper aims to: (1) identify common inefficiency of construction activities on sites; and (2) quantify the CO2 emission reduction potential for the problems of different types of construction operations identified during the construction phase.

Current Efficiency of Construction Equipment in EU

Since the existing emissions regulations take power into account for construction equipment, it would seem the most effective measure of efficiency in terms of carbon emissions should also be grams of pollutant per kWh (gCO2/kWh). Table 1 presents the average efficiencies of different types of construction equipment at varying levels of usage in terms of their CO2 emissions. Note that these values were calculated for this study using fuel consumption tables in [Caterpillar Performance Handbook 2010] which give fuel consumption of most Caterpillar products. This is because it was the only available source for equipment fuel consumption for a variety of equipment supplied in the EU. They are given at various levels of usage, which are low (load factor of 20-40%), medium (load factor of 40-60%), and high (load factor of 60-80%). For each piece of equipment, the fuel consumption (l/hr) was calculated in terms of l/kWh by dividing by the rated power (in kW) of that machine [Caterpillar Performance Handbook 2010]. This was then converted into gCO2/kWh by multiplying by 2640 gCO2/l assuming diesel engines [Ecoscore 2018]. Each piece of equipment was classed into one of the categories given in Table 1 and the average emissions per kWh for each equipment type were calculated for low, medium and high usage levels. These values were then converted to fuel cost to illustrate the difference in the operation cost per kWh for a car compared to all types of equipment. It is clearly found from Table 1 that the difference in cost between the average of construction equipment type and the average car varies from between almost 5 times as much (€0.14/kWh compared to €0.03/kWh) to 10 times as much (€0.31/kWh compared to €0.03/kWh). This indicates that there is an urgent need for a guidance on reducing fuel cost and gas emissions and improving the efficiency of construction equipment.

Table 1 CO2 emissions of typical construction equipment types with different level of usage.

| Type | gCO2/kWh |  |  | €/kWh |
| --- | --- | --- | --- | --- |
|  | Low | Med | High | Fuel cost |
| Track Tractor | 379 | 459 | 609 | 0.24 |
| Motor Grader | 336 | 440 | 564 | 0.23 |
| Skid Steer Loader | 410 | 533 | 656 | 0.28 |
| Multi Terrain Loader | 413 | 536 | 660 | 0.28 |
| Track Loader | 366 | 475 | 585 | 0.25 |
| Articulated Truck | 208 | 278 | 377 | 0.15 |
| Load Haul Dump Truck | 208 | 370 | 508 | 0.19 |
| Excavator | 319 | 433 | 554 | 0.23 |
| Pipelayer | 216 | 278 | 345 | 0.15 |
| Scraper | 451 | 592 | 758 | 0.31 |
| Telehandlers | 258 | 477 | 604 | 0.25 |
| Compactor | 368 | 483 | 591 | 0.25 |
| Paver | 388 | 476 | 651 | 0.25 |
| Truck | 199 | 266 | 333 | 0.14 |
| Wheel loader | 208 | 300 | 416 | 0.16 |
| Track Loader | 467 | 579 | 688 | 0.30 |
| Cars | - | 51 | - | 0.03 |

Methodology

This section provides an overview of the research to identify common inefficiency problems of construction activities on sites and quantify the CO2 emission reduction potential for the problems. Figure 1 shows the research procedure, consisting of four primary processes 1) onsite observation on operations of construction equipment, 2) estimation of CO2 emission, and 3) estimation of potential CO2 emission reduction.

It is important to note that the focus of this paper is to quantify emission reduction potential from slight adjustments of real-world equipment operations in a way that can be easily interpreted to the industry people. While simulation methods such as discrete-event simulation may result in more reliable and detailed estimates on equipment cycle time considering all the constraints, this study intends to use commonly used methods to estimate equipment cycle time in field so that such estimations can be easily replicated in other real-world operations.

Onsite Observation on Operations of Construction Equipment

First, in order to identify common inefficiency of construction activities operated by construction equipment, field studies were conducted. Three UK construction sites were chosen in this study, which are (1) a site in West Cambridge run by Morgan Sindall Inc., (2) one in Elephant and Castle, London run by Laing O’Rourke Inc., and (3) one in Trumpington, Cambridge run by Coinford Inc. Details of these sites are presented in a later section.

Estimation of CO2 Emission

The basic premise of this study is that gas emissions from onsite equipment usage are directly associated with the fuel consumption of the equipment. That is, energy consumption increases in direct proportion to the working hours of construction activities of construction equipment.  Based on the aforementioned premise, calculation of gas emissions was conducted in three stages: 1) working hour estimation, 2) fuel consumption calculation, and 3) gas emissions calculation.  Note that in this study construction activities of excavators and dumpers are focused since they are the most popular construction equipment at construction sites.

The first step for the estimation of CO2 Emission is to estimate the working hours of equipment required for completing construction activities. Here, the ‘working hours’ is the total time in hours for excavators and dumpers to complete the task of moving the required materials. Note that the definitions of technical terms including ‘mins per dumper cycle’ and details of how to calculate the total time are presented in Appendix I.

Once the total time of construction equipment is calculated, fuel used for the activities is then calculated using the following formulas:

Fuel used (l) for 1 dumper in actual case = ideal fuel consumption + (mins per dumper cycle ×# of dumper cycles60 ×low excavator fuel consumption) + (dumper travel time ×# of dumper cycles60 ×low dumper fuel consumption)

For 2 dumpers in actual case,

if mins per dumper cycle < dumper travel time, then =

(dumper travel time ×# of dumper cycles60 ×medium dumper fuel consumption) + (mins per dumper cycle ×# of dumper cycles60 ×(medium excavator+low dumper fuel consumption))+

((dumper travel time-mins per dumper cycle) ×# of dumper cycles60 ×low dumper fuel consumption),

Otherwise = (dumper travel time ×# of dumper cycles60 ×medium dumper fuel consumption) + (mins per dumper cycle ×# of dumper cycles60 ×(medium excavator+low dumper fuel consumption))+

((mins per dumper cycle- dumper travel time) ×# of dumper cycles60 ×low dumper fuel consumption)

Note that ideal CO2 gas emissions given the ideal fuel used to complete the task are presented in Appendix I. Finally, CO2 gas emissions given the fuel (l) used to complete the task are calculated by multiplying by 2640 gCO2/l assuming diesel engines.

Estimation of Potential CO2 Emission Reduction

Once the fuel consumption used for the activities is completed, potential savings for CO2 gas emissions are estimated using the ideal and actual CO2 gas emissions as follows:

| Emission savings= actualCO2 emission-ideal CO2 emissionactualCO2 emission×100% | (1) |
| --- | --- |

In the next sections, the research procedure of this study described in this section is presented for each case study in detail.

Case Study

In order to determine the common inefficiencies associated with construction equipment activities, and to quantify their potential for emissions reductions, three construction sites were used as case studies. In fact, construction sites are notoriously difficult to access because of the obvious problems of access, permissions, safety and etc. Based on the reasons that 1) the primary purpose of the field study is to find common inefficiencies in construction operations without specific targets of study and 2) a field study at construction sites has difficulties in getting access permission from site managers, the construction sites were chosen based on the proximity to the researchers and the approval from site managers.

Scenario Development of Equipment Operations

For the analysis of construction operation inefficiency, in the case studies, ‘simulations’ with different size of excavators and dumpers were conducted based on the collected inefficiency of construction activities. A set of scenarios with varying size of excavators and dumpers were developed based on the observations from the three different construction sites to compare and quantify the potential capacity of gas emissions caused by construction activities. Table 2 shows the 12 scenarios. The excavators were chosen from the Caterpillar Handbook [Caterpillar Performance Handbook 2010] so that the fuel consumptions estimated in Table 1 can be used. The choice of 3, 6 and 9 tonne dumpers came from the case studies in which they are mostly commonly used. Note that since the Caterpillar Handbook does not contain dumpers of those sizes, Thwaites dumpers were used for the estimation of fuel consumption and their fuel consumption was estimated by taking their rated power and using the averages of gCO2/kWh calculated in Table 1. The definitions of the columns of the tables in this section are given in Appendix I.

Case Study 1: West Cambridge, Morgan Sindall

This site was located in West Cambridge and consisted of an area of approximately 7500m2(100m × 75m). Figure 1 shows the site map (not to scale) of West Cambridge, which consists of two areas (Area 1 and Area 2) where construction equipment were working. Since pathway was also a road, Area 1 was the only accessible area in the site for material to be dumped. A U50-3α Kubota excavator was positioned in Area 1 and was transferring material into a 3 tonne dumper which travelled along pathway 1 to Area 2 and back along the same route. If pathway 1 was blocked, then it travelled both ways down pathway (pathway 2). It was estimated that 200m3 of material was moved in total.

Onsite observation on operations of construction equipment

The process of moving material from one area to another was studied for an hour from Area 1. When the excavator was working to transfer material to the dumper, it was working at medium capacity as explained earlier and at low capacity while it was idle. Likewise with the dumper, it was noted as working at medium capacity while it was travelling and at low capacity while it was idle. It can be seen from Table 3 that, on average during the process of transporting material from Area 1 to Area 2, it took 2 minutes for the excavator to fill the dumper and 5 minutes for the dumper to travel to Area 2 and back. While the dumper was being filled, it remained on idle rather than being turned off, and the same was true for the excavator while the dumper was travelling. This indicates a potential for emissions reductions by minimising unnecessary idling. Additionally, if there had been 2 dumpers taking alternative routes, then the time that the excavator would spend on idle would be much lower for the same amount of material, therefore wasting less fuel. Both the process optimisation and the excess engine idling indicate potential for emissions reductions.

Figure 1. Site map of Case study 1: West Cambridge (not to scale)

Estimation of potential CO2 emission reduction

As witnessed in the case study, machines left on idle is a common problem and presents an opportunity for emissions reductions. As such, this idle problem was analysed. The actual fuel consumption was calculated based on assumptions that, when the excavator and dumper are working, they are operating at their respective medium fuel consumption levels, and when they are not working, they are assumed to be idle and therefore operating at their respective low consumption levels. The ideal fuel consumption is calculated similarly, but when the excavator and dumper are not working, they are assumed to be off and therefore not consuming fuel.

Table 2 The scenarios with varying types of excavators and dumps

| Scenario | # of dumpers | # of excavators | Dumper power (kW) | Excavator power (kW) | Dumper fuel consumption (l/hr) |  |  | Excavator fuel consumption (l/hr) |  |  | Excavator cycle time | # of cycles / min | Excavator bucket capacity (m3) | Dumper bucket capacity (m3) | # cycles to fill dumper | mins per dumper cycle | # of dumper cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Low | Med | high | Low | Med | high |  |  |  |  |  |  |  |
| 1 | 1×3 tonne thwaites | 1×307D | 25 | 40 | 1.9 | 2.5 | 3.1 | 3.7 | 4.9 | 6.2 | 0.22 | 2.8 | 0.23 | 1.68 | 7 | 2.6 | 119 |
| 2 | 1×6 tonne thwaites | 1×307D | 63 | 40 | 4.8 | 6.3 | 7.9 | 3.7 | 4.9 | 6.2 | 0.22 | 2.8 | 0.23 | 3.4 | 15 | 5.2 | 59 |
| 3 | 1×9 tonne thwaites | 1×307D | 70 | 40 | 5.3 | 7.1 | 8.8 | 3.7 | 4.9 | 6.2 | 0.22 | 2.8 | 0.23 | 4.6 | 20 | 7.1 | 43 |
| 4 | 1×3 tonne thwaites | 1×312D | 25 | 70 | 1.9 | 2.5 | 3.1 | 7.5 | 11.5 | 15.2 | 0.21 | 3.0 | 0.52 | 1.68 | 3 | 1.1 | 119 |
| 5 | 1×6 tonne thwaites | 1×312D | 63 | 70 | 4.8 | 6.3 | 7.9 | 7.5 | 11.5 | 15.2 | 0.21 | 3.0 | 0.52 | 3.4 | 7 | 2.2 | 59 |
| 6 | 1×9 tonne thwaites | 1×312D | 70 | 70 | 5.3 | 7.1 | 8.8 | 7.5 | 11.5 | 15.2 | 0.21 | 3.0 | 0.52 | 4.6 | 9 | 3.0 | 43 |
| 7 | 1×3 tonne thwaites | 1×319D | 25 | 100 | 1.9 | 2.5 | 3.1 | 10.5 | 15.5 | 21 | 0.28 | 2.2 | 0.76 | 1.68 | 2 | 1.0 | 119 |
| 8 | 1×6 tonne thwaites | 1×319D | 63 | 100 | 4.8 | 6.3 | 7.9 | 10.5 | 15.5 | 21 | 0.28 | 2.2 | 0.76 | 3.4 | 4 | 2.0 | 59 |
| 9 | 1×9 tonne thwaites | 1×319D | 70 | 100 | 5.3 | 7.1 | 8.8 | 10.5 | 15.5 | 21 | 0.28 | 2.2 | 0.76 | 4.6 | 6 | 2.7 | 43 |
| 10 | 1×3 tonne thwaites | 1×329D | 25 | 140 | 1.9 | 2.5 | 3.1 | 16 | 24 | 32 | 0.25 | 2.5 | 1.35 | 1.68 | 1 | 0.5 | 119 |
| 11 | 1×6 tonne thwaites | 1×329D | 63 | 140 | 4.8 | 6.3 | 7.9 | 16 | 24 | 32 | 0.25 | 2.5 | 1.35 | 3.4 | 3 | 1.0 | 59 |
| 12 | 1×9 tonne thwaites | 1×329D | 70 | 140 | 5.3 | 7.1 | 8.8 | 16 | 24 | 32 | 0.25 | 2.5 | 1.35 | 4.6 | 3 | 1.4 | 43 |

Table 3 West Cambridge site observations of working times of excavator and dumper.

|  | Level of engine usage |  |  |  | Observations |
| --- | --- | --- | --- | --- | --- |
|  | Low |  | Medium |  |  |
|  | Start | Finish | Start | Finish |  |
| 1 | 00:00 | 00:06 | 00:00 | 00:06 | Moving fill around to make room for next load |
|  | (dumper) |  | (excavator) |  |  |
| 2 | 00:06 | 00:25 |  |  | Both idle, trucks blocking pathway 1 and 2 |
|  | (excavator) |  |  |  |  |
| 3 | 00:25 | 00:29 | 00:25 | 00:29 | Excavator filling dumper truck |
|  | (dumper) |  | (excavator) |  |  |
| 4 | 00:29 | 00:35 | 00:29 | 00:35 | Excavator idle, dumper traveling |
|  | (excavator) |  | (dumper) |  |  |
| 5 | 00:35 | 00:37 | 00:35 | 00:37 | Excavator filling dumper truck |
|  | (dumper) |  | (excavator) |  |  |
| 6 | 00:37 | 00:42 | 00:37 | 00:42 | Excavator idle, dumper traveling |
|  | (excavator) |  | (dumper) |  |  |
| 7 | 00:42 | 00:44 | 00:42 | 00:44 | Excavator filling dumper truck |
|  | (dumper) |  | (excavator) |  |  |
| 8 | 00:44 | 00:49 | 00:44 | 00:49 | Excavator idle, dumper traveling |
|  | (excavator) |  | (dumper) |  |  |
| 9 | 00:49 | 00:51 | 00:49 | 00:51 | Excavator filling dumper truck |
|  | (dumper) |  | (excavator) |  |  |
| 10 | 00:51 | 01:00 | 00:51 | 01:00 | Excavator idle, dumper traveling |
|  | (excavator) |  | (dumper) |  |  |

Table 4 gives a summary of the results and shows that the percentage reduction from eliminating idle in each operation varies from 43 - 77% with a mean of 53%. It can be found that the lowest ideal emissions of 131 kgCO2 are achieved in scenarios 4 and 12 while the lowest actual emissions of 254 kgCO2 is achieved in scenario 1. The actual emissions are much higher when the excavator and dumper powers are not well matched e.g. scenario 10 which gives the highest actual emissions. This combination of dumper and excavator also give the highest reduction in emissions by eliminating idle (76%), whereas, the lowest reduction in emissions (43%) occur in scenarios 2 and 6, where the dumper and excavator powers are very well matched. This demonstrates to some extent a potential for emissions reduction by equipment selection, which is discussed in detail in a later section. It is also interesting to note that the lowest operation costs for both ideal and actual conditions occur in scenarios 5, 6, 8 and 9 while only scenario 6 has low emissions. This illustrates that there might be little incentive for the construction companies to select the equipment to lower CO2 emissions as this does not necessarily directly correlate with lowering operation costs.

Table 4. Summary of potential emissions savings by eliminating engine idle.

| Scenario | Ideal cost of operation (€) | Actual cost of operation (€) | Ideal fuel used (l) | Actual fuel used (l) | Ideal CO2 emissions (kg) | Actual CO2 emissions (kg) | Emissions savings (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 291 | 355 | 49.9 | 96.3 | 132 | 254 | 48 |
| 2 | 311 | 370 | 56.0 | 98.7 | 148 | 261 | 43 |
| 3 | 200 | 256 | 50.8 | 91.4 | 134 | 241 | 44 |
| 4 | 341 | 449 | 49.7 | 128.2 | 131 | 338 | 61 |
| 5 | 219 | 284 | 55.8 | 102.9 | 147 | 272 | 45 |
| 6 | 225 | 278 | 50.6 | 89.2 | 134 | 236 | 43 |
| 7 | 408 | 557 | 55.4 | 163.3 | 146 | 431 | 66 |
| 8 | 257 | 341 | 61.5 | 122.4 | 162 | 323 | 49 |
| 9 | 263 | 330 | 56.3 | 104.8 | 149 | 277 | 46 |
| 10 | 499 | 721 | 48.6 | 209.2 | 128 | 552 | 76 |
| 11 | 297 | 412 | 54.7 | 137.9 | 144 | 364 | 60 |
| 12 | 303 | 391 | 49.5 | 112.8 | 131 | 298 | 56 |

Operation planning in the construction industry normally refer to the operation planning of crew. In this case, the crew is the dumper and excavator seen in case study 1. In this study, analysis on the potential emissions reduction from using 2 dumpers instead of 1 was performed based on the assumption that all machines remain on idle when not working. Table 5 show the potential for emissions savings by adding a dumper to the operation of a dumper and excavator as seen in Case study 1. There is a large range of values for the result; 6-42% (with an average of 29%) which is due to the reduction in idle time. If the operation has little reduction in idling time for the excavator or more idling for the dumpers as a result of adding 1 dumper, then the reduction in emissions will be small. However, the difference can be planned such that there is little or no idling time for the excavator or dumper, which will yield the lowest emissions.

Table 5. Summary of potential emissions savings by operation planning.

| Scenario | 1 Dumper |  |  |  | 2 Dumpers |  |  |  | Emissions savings (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Whole cycle time (min) | Total time (hrs) | Cost of operation (€) | Actual CO2 emissions (kg) | Whole cycle time (min) | Total time (hrs) | Cost of operation (€) | Actual CO2 emissions (kg) |  |
| 1 | 7.6 | 15.0 | 355 | 254 | 3.8 | 7.5 | 232 | 181 | 29 |
| 2 | 10.2 | 10.0 | 370 | 261 | 5.2 | 5.2 | 262 | 215 | 17 |
| 3 | 12.1 | 8.7 | 256 | 241 | 7.1 | 5.1 | 294 | 227 | 6 |
| 4 | 6.1 | 12.1 | 449 | 338 | 3.0 | 6.0 | 276 | 219 | 35 |
| 5 | 7.2 | 7.1 | 284 | 272 | 3.6 | 3.5 | 279 | 202 | 26 |
| 6 | 8.0 | 5.8 | 278 | 236 | 4.0 | 2.9 | 293 | 178 | 24 |
| 7 | 6.0 | 11.9 | 557 | 431 | 3.0 | 5.9 | 331 | 266 | 38 |
| 8 | 7.0 | 6.9 | 341 | 323 | 3.5 | 3.4 | 323 | 228 | 29 |
| 9 | 7.7 | 5.6 | 330 | 277 | 3.9 | 2.8 | 334 | 199 | 28 |
| 10 | 5.5 | 10.9 | 721 | 552 | 2.7 | 5.5 | 410 | 322 | 42 |
| 11 | 6.0 | 5.9 | 412 | 364 | 3.0 | 2.9 | 379 | 240 | 34 |
| 12 | 6.4 | 4.6 | 391 | 298 | 3.2 | 2.3 | 385 | 200 | 33 |

As a discussion, if the dumper cycle time (defined in appendix I) is higher than the dumper travel time (5 minutes in this example), then the dumpers will be on idle for a certain amount of time per cycle. Conversely, if the dumper cycle time is lower than the dumper travel time, then the excavator will be on idle for a period of time per cycle. This is seen in scenario 2 where it can be seen that the whole cycle time (5.2 minutes) for 2 dumpers is the closest to the dumper travel time (5.0 minutes) which yields relatively low emissions. However, the next closest cycle time in scenario 6 (4.0 minutes) has the smallest emissions because the cycle time is lower than the dumper travel time, which means that the dumpers are on idle for a short period in every cycle. This shortens the total cycle time as the dumpers are working at maximum capacity thereby reducing emissions. In general, the operations costs are reduced by adding the extra dumper which gives an incentive for the construction companies to use activity planning to their financial advantage. However, the selection of appropriate equipment gives less of a financial advantage as scenario 6, which has the lowest emissions (178 kgCO2) but does not have the lowest cost although the lowest cost is in scenario 1 with €232 which has emissions only marginally higher than scenario 6.

Case Study 2: Elephant and Castle, Laing O’Rourke

This site was situated in Elephant and Castle, London. It was approximately 10,000m2(100m × 100m) and consisted of 2 piling rigs, 3 excavators and 2 dumpers as shown in Figure 2.

Onsite observation on operations of construction equipment

Each piling rig had a designated smaller excavator and 9 tonne dumper which were moving the spoil from the piling rig (after piling was completed) to the spoil heap. This was well executed with the excavator and dumper positioned close to the spoil heap to reduce traveling distance. However, an excavator was positioned in between the spoil from the piling rig and the spoil heap which resulted, unnecessarily, in the excavator having a turning circle of 180 degrees as opposed to 90 degrees as shown in Figure 3. This lengthens the cycle time of the excavator and will therefore result in a higher fuel consumption. Furthermore, in a similar manner to Case Study 1, all machines were left on idle whilst not in use, unless they were to be out of use for more than 20-30 minutes. This reiterates the problem of machines remaining on idle in construction processes unnecessarily and indicates that this may be a common problem on construction sites.

Estimation of potential CO2 emission reduction

Task planning is the planning of a single unit. This section looks at the potential capacity for emission reductions due to adjusting the swing angle of the excavator, which was witnessed in this case study, where an excavator which had a swing angle of 180 degrees instead of a feasible 90 degrees. In this case, the amount of material to be moved was approximately 20m3 and the travel time for the dumper was found to be about 4 minutes. As such, these are the constants that were used in calculating the emission reductions in Table 6. Note that according to [Nunnally 2010], the swing-depth factor was adjusted for each case of 180 degrees and 90 degrees, such that it is:

• 0.95 for 180 degree swing angle, assuming its depth of cut is 30% of it’s maximum.

• 1.15 for 90 degree swing angle, with the same assumptions as above.

Figure 2. Elephant and Castle, London case study site map

Figure 3. Turning circle of excavator observed on site of Case study 2

Table 6 gives the summary of results for changing the swing angle from 180 degrees to 90 degrees. It gives a range of emissions reductions of 3-11% with a mean of 7%. It shows that the lowest actual emissions in both swing angle cases are with the well matched combination of dumpers and excavators such as scenario 6 (which has emissions 21.3 and 19.5 kgCO2) or the scenario 3 (which has emissions of 22.8 and 20.3 kgCO2). The combinations with large power differences between dumper and excavator have relatively high emissions such as the scenarios 10 which has emissions of 44.7 kgCO2. Thus, this illustrates a potential for more emissions savings by selection of equipment.

Table 6. Summary for potential emissions savings by task planning.

| Scenario | Swing angle 180° |  |  | Swing angle 90° |  |  | Emissions savings (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total time (hrs) | Actual fuel used (l) | Actual CO2 emissions (kg) | Total time (hrs) | Actual fuel used (l) | Actual CO2 emissions (kg) |  |
| 1 | 1.33 | 8.6 | 22.7 | 1.24 | 7.9 | 21.0 | 7 |
| 2 | 0.93 | 9.2 | 24.2 | 0.84 | 8.2 | 21.8 | 10 |
| 3 | 0.83 | 8.6 | 22.8 | 0.74 | 7.7 | 20.3 | 11 |
| 4 | 1.02 | 11.0 | 29.0 | 0.98 | 10.5 | 27.6 | 5 |
| 5 | 0.62 | 9.1 | 24.1 | 0.58 | 8.5 | 22.4 | 7 |
| 6 | 0.52 | 8.1 | 21.3 | 0.48 | 7.4 | 19.5 | 8 |
| 7 | 1.00 | 13.9 | 36.8 | 0.97 | 13.3 | 35.1 | 5 |
| 8 | 0.60 | 10.8 | 28.5 | 0.56 | 10.1 | 26.6 | 7 |
| 9 | 0.50 | 9.4 | 24.9 | 0.46 | 8.7 | 22.9 | 8 |
| 10 | 0.90 | 17.4 | 45.9 | 0.88 | 16.9 | 44.7 | 3 |
| 11 | 0.50 | 11.8 | 31.0 | 0.44 | 11.2 | 29.6 | 4 |
| 12 | 0.39 | 9.8 | 25.8 | 0.38 | 9.2 | 24.3 | 5 |

Case Study 3: Trumpington, Coinford

This site was situated in Trumpington, Cambridge. It was approximately 22,500m2(150m × 150m) and had several independent crews of excavator and dumper. Figure 4 shows the schematic site map. All 3 dumpers were 9 tonnes whilst the 2 excavators nearest Storage area 1 were 6.5 tonne excavators and those nearest Storage area 2 were 5 tonne excavators and the final one positioned apart from the storage areas was a 20 tonne excavator.

Onsite observation on operations of construction equipment

This site presented the same problems as the previous 2 case studies where machines were left on idle unnecessarily. In addition, there was little communication between the crews which resulted in the storage areas being badly placed. During the study, the excavator below storage area 1 required material that was only available in storage area 2. As a result, it and the dumper travelled up to storage area 2, having to take a longer route around the partially built up areas, where they proceeded to move material from storage area 2 to below storage area 1. This resulted in the time taken for the dumper to dump the material and return to the excavator being 6 minutes instead of 2 minutes which it could have achieved if the planning had resulted in the material being placed in storage area 1. Therefore, for the 50m3 of material that had to be moved, a significant amount of fuel would have been wasted in both the additional distance that the dumper had to travel and the additional time that the excavator was on idle.

Figure 4. Site map of Case Study 3

Estimation of potential CO2 emission reduction

Reduction potential for the problem found in Case study 3, where the material required in one area of the site was stored too far away, was quantified. The dumper travel time was found to be 6 minutes, when it could have been as little as 2 minutes, and the material to be moved was approximately 50m3. Table 7 gives the potential emissions savings due to activity planning. It shows a range of 32-60% with a mean of 47%. In a similar manner to the task planning result, the lowest emissions are from the scenarios where the combination of dumper and excavator are well matched with power e.g. scenarios 5 and 6 give the lowest emissions for the 2 minutes travel time whilst scenarios 3 and 6 give the lowest emissions in the 6 minute travel time.

Table 7. Summary of potential emissions savings by activity planning.

| Scenario | Excavator power (kW) | Dumper power (kW) | Actual CO2 emissions (kg) |  | Emissions savings (%) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Dumper travel time (6 mins) | Dumper travel time (2 mins) |  |
| 1 | 40 | 25 | 72 | 39 | 45 |
| 2 | 40 | 63 | 72 | 46 | 36 |
| 3 | 40 | 70 | 65 | 45 | 32 |
| 4 | 70 | 25 | 98 | 45 | 54 |
| 5 | 70 | 63 | 77 | 41 | 46 |
| 6 | 70 | 70 | 66 | 38 | 42 |
| 7 | 100 | 25 | 125 | 57 | 55 |
| 8 | 100 | 63 | 92 | 48 | 47 |
| 9 | 100 | 70 | 78 | 44 | 43 |
| 10 | 140 | 25 | 162 | 65 | 60 |
| 11 | 140 | 63 | 105 | 48 | 55 |
| 12 | 140 | 70 | 85 | 41 | 52 |

Discussion

Equipment Selection

In previous sections of case study, the selection of equipment has been noted as a potential method of emissions savings. In this section, the results from each emissions reduction method are analysed to quantify the capacity for emissions reduction by appropriate equipment selection. Table 7 provides a summary of the CO2 emissions of each of the 4 reduction methods given in the previous sections after they have been implemented. For the ‘eliminating engine idle’ column, the emissions come from having no engine idle under the ‘Ideal CO2 emissions’ column in Table 4. For each column in Table 8, the maximum reduction potential can be calculated for the selection of equipment by the equation (2):

| Emission savings by equipment selection= maxCO2-minCO2maxCO2×100% | (2) |
| --- | --- |

The table also shows that the ideal emissions are much lower when the excavator and dumper powers are well matched. For example, Scenario 1 and 6. This combination of dumper and excavator also gives the highest reduction in emissions among the 12 scenarios, whereas, the lowest reduction in emissions occur in scenarios 7 and 10 which have a greater difference between the equipment powers of dumper and excavator. It indicates that matching hauling equipment with loading equipment is essential for the savings of CO2 emissions. In summary, the mean of the emissions savings which can be achieved by appropriate equipment selection is 40%.

Table 8. Summary of potential emissions savings by equipment selection.

| Scenario | Dumper power (kW) | Excavator power (kW) | CO2 emissions (kg) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Eliminating Engine Idle | Task Planning | Operation Planning | Activity Planning | Sum of the four methods (rank) |
| 1 | 25 | 40 | 132 | 21 | 181 | 39 | 373 (2) |
| 2 | 63 | 40 | 148 | 22 | 215 | 46 | 431 (8) |
| 3 | 70 | 40 | 134 | 20 | 227 | 45 | 426 (7) |
| 4 | 25 | 70 | 131 | 28 | 219 | 45 | 423 (6) |
| 5 | 63 | 70 | 147 | 22 | 202 | 41 | 412 (4) |
| 6 | 70 | 70 | 134 | 20 | 178 | 38 | 369 (1) |
| 7 | 25 | 100 | 146 | 35 | 266 | 57 | 504 (11) |
| 8 | 63 | 100 | 162 | 27 | 228 | 48 | 465 (10) |
| 9 | 70 | 100 | 149 | 23 | 199 | 44 | 415 (5) |
| 10 | 25 | 140 | 128 | 45 | 322 | 65 | 560 (12) |
| 11 | 63 | 140 | 144 | 30 | 240 | 48 | 461 (9) |
| 12 | 70 | 140 | 131 | 24 | 200 | 41 | 397 (3) |
|  |  | Emissions savings (%) | 19 | 56 | 44 | 42 | 40 |

Future CO2 Reduction Regulation on engine idle

Although some sites have a policy of turning engines off when they are not doing useful work, it appears from the case studies that this is not always carried through in practice. This could be mitigated by the use of engine idle shutdown, which is a controller currently used in some cars to shutdown the engine after a predetermined amount of time spent on idling.  There are already examples of engine idle shutdown in construction equipment such as in Volvo Construction Equipment, which shuts down the engine if it has been idle for over 4 minutes [Volvo 2018]. However this option is available in very limited equipment types. Thus, developing regulations on engine idling of construction equipment would be an effective and feasible means of reducing CO2 emissions. In the US, some local governments have developed policies to limit engine idling in construction such as North Central Texas [NCTCOG 2018] and California [California Environmental Protection Agency 2008] and the US Green Building Council is currently developing a policy that will apply in all states to limit idling of equipment to 5 minutes [USGBC 2014].

The CO2 regulations imposed on light-duty vehicles has resulted in an increase in hybrid and electric cars. However, according to a study [MacKay 2009], the average electric car produces approximately 105 gCO2/km if the grid is powered mainly by fossil fuels. This is currently lower than the emissions from fuel cars, but the regulations imposed mean that the emissions from fuel cars must be as low as 95 gCO2/km by 2021. Unless the electricity is provided by substantial green or low carbon sources, the switch to electric cars and equipment may actually produce more emissions. Furthermore, it was found from a study [Ji et al. 2012] that the electric vehicles generally had higher particulate matter emissions than the conventional vehicles per km due to the electricity produced from power stations having higher emissions. Another study conducted in the US found that while use of hybrid equipment could reduce the CO2 emissions by up to 28%, the NOX emissions would increase by up to 21% [Johnson 2013] which would likely not meet the previous emissions regulations for equipment. Therefore, if regulations were to be imposed on construction equipment, they would have to cover the resulting emissions from electric equipment as well as ensuring that the electric equipment still meets the previous regulations.

Conclusion

A field study approach for potential emissions savings in construction processes is presented in this paper. The saving approach includes eliminating engine idle, task planning, operation planning, activity planning. Three case studies consisting of site visits to a Morgan Sindall, Laing O’Rourke and Coinford sites were conducted. Every site had emissions wasted by a large amount of time being spent on idle and were estimated to be able to save 53% of their emissions by eliminating idling. The Laing O’Rourke site presented an inefficiency with the swing angle of the excavator being unnecessarily large and was estimated to be able to save 7% of emissions by proper task planning. On the Morgan Sindall site, a potential for emissions savings was identified by the use of 2 dumpers working together per excavator instead of 1, known as operation planning. This planning was estimated to be able to save 29% of emissions. Another potential for more efficient practice was identified on the Coinford site in which a storage area was badly placed and resulted in a 6 minute journey for the dumper as opposed to what could have been 2 minutes. This particular case was estimated to have the capacity to save 47% of emissions.

From the field studies, it was found that there are opportunities for emissions reductions if we focus on behavior rather than regulation. While regulation does have a large capacity for emissions reduction, this is not particularly useful if machines are left on idle unnecessarily and construction sites are subject to poor planning. In addition, findings of this study indicate that the integration of careful planning on construction equipment activities with appropriate equipment selection could contribute to the reduction of carbon gas emissions as well as savings of construction cost. The outcomes of this study also implies that the emission reduction opportunities in real-world operations can be captured with slight adjustments of existing operation plans and the impact of such adjustments can be easily quantified with the simplistic methods. However, as mentioned earlier, CO2 gas emissions from onsite equipment have often been neglected due to the fact that onsite equipment seems to account for a relatively low percentage of the total gas emissions compared to other categories such as the embodied emissions and operational emissions in the project life cycle. As a result, a practical method for reducing CO2 gas emissions related to adjustments of construction operations has not been fully suggested.  Therefore, considering environmental aspects in planning helps construction managers identify the best option that will increase the project’s integrated value. From the implications, it is envisaged that this study contributes to the body of knowledge by 1) providing new findings in common inefficiency of construction operation in construction sites through field studies, and 2) quantifying the potential gas emission reduction with the simplistic method.

However, there are some limitations in this study. First, as an exploratory case study, the scope of this study is limited to the identification and quantification of inefficiency in construction activities. Further study on how to implement the adjustments of inefficient construction equipment activities is necessary. Second, due to the fact that the estimates of gas emissions reductions were computed based on limited field data, generalization of the result is out of the scope of this study. Thus, future studies might look more into diverse case studies with ample observational data of construction activities to find whether potential emissions savings for each inefficient construction activity can be generalized. Third, this study calculated fuel consumption of construction equipment based on the standard document [Caterpillar Performance Handbook 2010], which may not reflect actual fuel consumption of construction equipment. For an accurate gas emission estimation and inefficiency analysis, real emissions based on direct measurement methods at construction sites need to be investigated.

Data Availability Statement

Data generated or analysed during the study are available from the corresponding author by request. Information about the Journal’s data sharing policy can be found here: http://ascelibrary.org/doi/10.1061/%28ASCE%29CO.1943-7862.0001263.

Acknowledgements

Reference

Ahn, C.R., and Lee S. (2013a). “Importance of operational efficiency to achieve energy efficiency and exhaust emissions reduction of construction operations.” J. Constr. Eng. Manage., 139(4), 404-413.

Ahn, C., Lee, S., Peña-Mora, F., and Abourizk, S. (2010). "Toward Environmentally Sustainable Construction Processes: The U.S. and Canada’s Perspective on Energy Consumption and GHG/CAP Emissions.” Sustainability, Molecular Diversity Preservation International (MDPI), 2(1), 354-370.

Ahn, C.R., Lewis, P., Golparvar-Fard, M., and Lee S. (2013b). “Integrated framework for estimating, benchmarking, and monitoring pollutant emissions of construction operations.” J. Constr. Eng. Manage., 139(12), A4013003.

California Environmental Protection Agency, Air Resources Board. (2008). New idling limits for owners, operators, renters or lessees of in-use off-road diesel vehicles. < https://www.arb.ca.gov/msprog/ordiesel/guidance/idling.pdf> (May1, 2018)

Caterpillar performance handbook. (2010). 40th Ed., Caterpillar, Peoria, IL.

CECE (2014). “The voice of the European construction equipment industry.” < http://www.europeandemolition.org/newsstand/files/the-voice-of-the-european-construction-equipment-industry/attachment/cece_barometer_2013_12_summary> (May 1, 2018)

CEMA (European Agricultural Machinery) and CECE (Committee for European Construction Equipment). (2011). “Reducing CO2 Emissions from Mobile Machinery.” <http://www.cema-agri.org/publication/cece-cema-position-co2-emissions-mobile-machinery> (May 1, 2018)

Ecoscore. (2018). How to Calculate the CO2 Emission Level from the Fuel Consumption?, http://ecoscore.be/en/info/ecoscore/co2 (May 1, 2018)

EPA. (2008a). “Quantifying greenhouse gas emissions in key industrial sectors.” EPA 100-R-08-002, Sector Strategies Div., Washington, DC. 〈http://www.epa.gov/sectors/pdf/2008/2008-sector-report-508-full.pdf〉 (May 1, 2018)

European Union (2018a). “Road Transport: Reducing CO2 Emissions from Vehicles.” < https://ec.europa.eu/clima/policies/transport/vehicles_en#tab-0-0> (May 1, 2018)

European Union (2018b). “What is the EU doing about climate change.” < https://ec.europa.eu/clima/citizens/eu_en> (May 1, 2018)

Facanha, C., and Horvath, A. (2005). “Environmental assessment of logistics outsourcing.” J. Manage. Eng., 21:1(27), 27-37.

Gallivan, F. (2010). “Greenhouse gas mitigation measures for transportation construction, maintenance, and operations activities.” NCHRP Project 25-25, Task 58, AASHTO, San Francisco.

Hong, T., Ji C., Jang, M., and Park H. (2014). “Assessment model for energy consumption and greenhouse gas emissions during building construction.” J. Manage. Eng., 30(2), 226-235.

Ji, S., Cherry, C.R., Bechle, M.J., Wu, Y. and Marshall, J.D. (2011). Electric Vehicles in China: Emissions and Health Impacts. United Stages: Environ. Sci. Technol., 46(4), 2018-2024.

Johnson, K.C. (2013). Hybrid Off-Road Equipment In-Use Emissions Evaluation. California: California Air Resources Board. < https://www.arb.ca.gov/msprog/aqip/offroad_hybrid.htm> (May1, 2018)

Kim, B., Lee, H., Park H., and Kin H. (2012). “Greenhous gas emissions from onsite equipment usage in road construction.” J. Constr. Eng. Manage., 138(8), 982-990.

Kim, B., Jang, W., and Lee D. (2015). “Analysis of the CO2 emission characteristics of earthwork equipment.” KSCE J. Civil Eng., 19(1), 1-9.

Lee, K., Tae, S., and Shin, S. (2009). “Development of a life cycle assessment program for building (SUSB-LCA) in South Korea.” Renew. Sustain. Energy Rev., 13(8), 1994-2002.

Lewis, P., Leming M., Rasdorf W. (2012). “Impact of engine idling on fuel use and CO2 emissions of nonroad diesel construction equipment.” J. Manage. Eng., 28(1), 31-38.

Lippiatt, B., and Boyles, A. (2001) “Using BEES to select cost-effective green products.” Int. J. Life Cycle Ass., 6(2), 76-80.

MacKay, D.J.C. (2009). Sustainable Energy – without the hot air. Cambridge, England: UIT Cambridge Ltd.

North Central Texas Council of Governments (NCTCOG). (2018) Locally enforced idling restrictions from NCTCOG. <https://www.nctcog.org/trans/quality/air/for-everyone/engine-off-north-texas> (May 1, 2018)

Nunnally S.W. (2010). Construction Methods and Management, The 8 Edition, New Jersey, Pearson Prentice Hall.

Ortiz, O., Pasqualino, J. C., Diez, G., and Castells, F. (2010). “The environmental impact of the construction phase: An application to composite walls from a life cycle perspective.” Resour. Conserv. Recycl., 54(11), 832-840.

Sandanayake, M., Zhang G., and Setunge S. (2016). “Environmental emissions at foundation construction stage of buildings – Two case studies.” Building Environ., 95, 189-198.

Suh, S., and Lippiatt, B. (2012). “Framework for hybrid life cycle inventory databases: A case study on the building for environmental and economic sustainability (BEES) database.” Int. J. Life Cycle Ass., 17(5), 604-612.

Trani, M.L., Bossi, B., Gangolells, M., and Casals, M. (2016). “Predicting fuel energy consumption during earthworks.” J. Clean. Prod., 112, 3798-3809.

United Nations Environment Programme Sustainable Buildings & Construction Initiative (UNEP SBCI). (2018). “Sustainable buildings & construction initiative: Information note.” 〈 https://business.un.org/en/documents/10372〉 (May 1, 2018).

US Green Building Council (USGBC). (2014). Clean Construction. <http://www.usgbc.org/credits/sspc75> (May1, 2018)

Volvo Construction Equipment. (2018). Cut down fuel consumption with Engine Auto Shut Down, < https://www.volvoce.com/trinidad-tobago/en-us/paramount/services/volvo-services/fuel-efficiency-services/engine-auto-shut-down/> (May 1, 2018)

Appendix I

Definitions of column titles of table used to calculate CO2 emissions

- Scenario – A specific combination of dumper and excavator

- # of dumpers – The number and type of dumpers used in the analysis

- # of excavators – The number and type of excavators used in the analysis, all Caterpillar excavators

- Excavator power (kW) – Rated power of the excavator in kW as given in the Caterpillar Handbook

- Dumper power (kW) – Rated power of the dumper in kW as given in Thwaites catalogue

- Excavator fuel consumption (l/hr) – Average fuel consumption in litres per hour of excavator for low, medium and high usage as given in the Caterpillar Handbook

- Dumper fuel consumption (l/hr) - Average fuel consumption in litres per hour of dumper for low, medium and high usage, calculated from the rated power of the dumper and the average fuel consumption for trucks as given in Table 1

- Excavator cycle time – The time taken to complete 1 cycle for the excavator in minutes given in the Caterpillar Handbook

- # of cycles/min – The number of excavator cycles per minute = 1excavator cycle time × operator skill × job efficiency × swing depth factor

- Excavator bucket capacity (m3) – The bucket capacity of the excavator found as the middle value of the range of values given in the Caterpillar Handbook

- Dumper bucket capacity (m3) – The heaped dumper bucket capacity as given in the Thwaites catalogue

- # of cycles to fill dumper – The number of excavator cycles required to fill the dumper to its capacity assuming that each excavator cycle has its bucket filled to capacity = Dumper bucket capacityExcavator bucket capacity

- mins per dumper cycle – The number of minutes required to fill the dumper = # of cycles to fill dumper# of cycles per min

- mins per whole cycle – The number of minutes required to fill the dumper and for the dumper to travel to the dump site and back. For 1 dumper = mins per dumper cycle + dumper travel time (mins). For 2 dumpers, if mins per cumper cycle < dumper travel time, then = mins per dumper cycle+dumper travel time2, otherwise = mins per dumper cycle

- # of dumper cycles – The number of dumpers needed to complete the task of moving required material = material moveddumper bucket capacity

- Total time (hrs) – Total time in hours to complete the task of moving the required material = mins per whole cycle ×# of dumper cycles60

- Rental days – Total time in days for rental purposes assuming 9 hours in a working day = Total time hrs9

- Ideal – Assuming the task is completed and the machines are switched off when not doing useful work.

- Actual – Assuming the task is completed and the machines are left on idle when not doing useful work.

- Fuel used (l) – The total fuel consumed in litres to complete the task.

For 1 and 2 dumpers ideal =

(mins per dumper cycle ×# of dumper cycles60 ×med excavator fuel consumption) + (dumper travel time ×# of dumper cycles60 ×med dumper fuel consumption)

For 1 dumper actual = ideal fuel consumption + (mins per dumper cycle ×# of dumper cycles60 ×low excavator fuel consumption) + (dumper travel time ×# of dumper cycles60 ×low dumper fuel consumption)

For 2 dumpers actual, if mins per dumper cycle < dumper travel time, then =

(dumper travel time ×# of dumper cycles60 ×med dumper fuel consumption) + (mins per dumper cycle ×# of dumper cycles60 ×(med excavator+low dumper fuel consumption))+

((dumper travel time-mins per dumper cycle) ×# of dumper cycles60 ×low dumper fuel consumption),

Otherwise  (dumper travel time ×# of dumper cycles60 ×med dumper fuel consumption) + (mins per dumper cycle ×# of dumper cycles60 ×(med excavator+low dumper fuel consumption))+

((mins per dumper cycle- dumper travel time) ×# of dumper cycles60 ×low dumper fuel consumption)

- Cost of fuel (€) – The cost of fuel used in Euros given the average price of fuel in the EU = fuel used (l) × price of fuel (€/l)

- CO2 emissions (kg) – Carbon dioxide emissions given the fuel used to complete the task = fuel used (l) × 2.64 kgCO2/l

- Rental per day (€) – Rental cost per day in the UK of dumpers and excavators in Euros obtained from SELECT HIRE UK.

- Cost of operation (€) – The total cost of completing the task including rental of the machines and fuel costs = (rental days × (excavator + dumper rental per day)) + cost of fuel