The potential of Green Ammonia for Agricultural and Economic Development in Sierra Leone

Collin Smith1 and Laura Torrente-Murciano1,2,3

1Department of Chemical Engineering and Biotechnology, University of Cambridge, Philippa Fawcett Drive, CB3 0AS, Cambridge, UK

2Correspondence: lt416@cam.ac.uk, telephone: +44 1223 768664

3Lead Contact

Summary

Sierra Leone is one of the least developed countries in the World, with an economy strangled by the necessity of importing rice to feed the population. In part, this deficit results from domestic farmers rarely using inorganic fertilizer, which is synthesized from fossil fuels internationally. Here we evaluate the economic benefits of producing green ammonia from renewable local hydropower for low-carbon cost-effective fertilizer production. Its use as fertilizer estimates a 30-year net present value (NPV) of ~ $230M (~165% return of investment) compared to simply importing fertilizers, which would already save at least $50M a year compared to the current situation of importing rice, but hinges on additional external factors related to implementing modern agriculture. In addition, green ammonia can buffer seasonal fluctuations of hydroelectricity from 50-900 MW and produce a consistently available 370 MW of power. While this study presents an initial analysis of Sierra Leone as case study, it exemplifies the possible economic and social benefits of green ammonia in developing countries.

Keywords: green ammonia, fertilizer, Haber-Bosch, Sub-Saharan Africa

Introduction

Sub-Saharan Africa (SSA) is at the crossroads of systemic deficiencies in both electrical energy and agricultural output. The implementation of renewable energy in Africa has gained considerable attention in recent years because of the plentiful renewable sources – particularly solar – and the strong correlation between energy consumption and gross domestic product (GDP) around the world.1 At the same time, SSA is struggling to feed its burgeoning population while facing a crisis due to the extraction of nutrients from the soil which leads to the unnecessary destruction of habitat in search of more virgin land.2 Both electricity accessibility and sustainable agriculture are crucial to the development of Africa. This paper presents the unique opportunities provided by renewable energy produced ammonia, also called green ammonia, to  fuel agricultural growth and  buffer seasonal electricity fluctuations to underpin economic development in the region.

While the role of electricity in accessing modern amenities is ubiquitous, its role in lifting people out of poverty through economic growth is less certain. There is still significant debate as to whether energy consumption drives economic growth or vice versa.3 Indeed, it likely depends on the limiting factor of a country’s economy, as energy consumption can certainly limit growth if energy is scarce,4 but so can unproductive use of human labor. At the level of individuals, it is clear that renewable energy will have a positive effect on many of the Sustainable Development Goals laid out by the UN, such as increasing income, promoting gender equality, and improving education.5-7 Access to energy (i.e. electricity) is especially important for industrial growth.8 While solar and wind energy are the most popular forms of renewable energy, SSA also has plentiful hydroelectric resources – 92% of which undeveloped – that are generally acknowledged the most cost effective option.9 However, despite its potential, hydroelectricity production can experience significant seasonal fluctuations even if large areas are flooded for a reservoir.10

The intensification of agriculture in Asia and Latin American – known as the “Green Revolution” – is largely responsible for providing food to growing populations in the second half of the 20th century.11 This transformation was rooted in improved seed varieties, but it was crucially supplemented by fertilizers and irrigation to the point where increased yields shifted farmers from subsistence to market orientation through rising incomes.12 SSA has largely not enjoyed these benefits, and has instead tried (unsuccessfully) to meet food demands through extensive rather intensive agriculture. Originally, the improved seed varieties of the 1950s-1970s were not suitable for the climates of SSA,13 but more recently the problem has shifted to limited financing, lack of infrastructure, underdeveloped markets and poor soil productivity. Even if seed varieties capable of growing in infertile soil can be developed,14 the problem remains the balance of key nutrients in the soil when extraction through crops outweighs natural regeneration and a systemic deficiency is established. Indeed, average agricultural production has dropped during the past 50 years, and it is largely attributed to the mining of soil nutrients which extracts on average 22 kg nitrogen, 2.5 kg phosphorous, and 15 kg potassium  per hectare each year.2, 15

The ammonia molecule sits at the intersection of increasing agricultural production through the use of fertilizers and adopting intermittent renewable energy in SSA. The cornerstone of increased fertilizer use (and agricultural output) in the 20th century is the Haber-Bosch (HB) process for the synthesis of the ammonia, which is then converted into nitrogen based fertilizers (N-fertilizers). The conventional HB process uses methane as an energy source and feedstock of hydrogen to combine with nitrogen from the air to synthesize ammonia (brown ammonia). However, the lack of established natural gas supplies in many SSA countries (and strained natural gas supplies in those with established wells) makes local production of nitrogen fertilizers infeasible. Recently, an electrically driven HB process has gained attention for the possible coupling with renewable electricity (green ammonia).16 In this process, hydrogen is derived from water through electrolysis and nitrogen is separated from air through pressure swing adsorption (PSA), allowing renewable energy, plentiful in SSA, to produce N-fertilizer. In addition, ammonia can act as a hydrogen carrier to store energy in a dense liquid form and buffer the supply of renewable energy for the electricity grid. Solar and wind energy fluctuates on the time scale of hours and days, while hydroelectric energy fluctuates seasonally – particularly in SSA – making it ideal for the electric HB process.

Sierra Leone ranks as the country with the 8th lowest human development index out of 189 countries and is well below UN thresholds of least developed countries.17, 18 Both electricity availability and agricultural output deficiencies are systemic as only 9% of the population has access to electricity19 and >300,000 mt of rice – a staple for Sierra Leonans - must be imported even though rice cultivation constitutes greater than 50% of household farmland.20 The import of rice accounts for 15% of total imports, higher than every other country in West Africa aside from Guinea-Bissau (Figure S.1).21 As a result, 50% of households are food insecure20 and only 7% of children under the age of two have a minimally acceptable diet.22 Fertilizer use at 4 kg/ha is low even for SSA standards at 9 kg/ha and negligible compared to the world at >100 kg/ha.15, 23 Further, Sierra Leone experiences some of the highest annual rainfall in Africa,24 leading to significant hydroelectric potential that fluctuates drastically during seasons.25 Indeed, the majority of grid electricity is supplied by a 50 MW hydroelectric plant at Bumbuna.19

This work utilizes Sierra Leone as a case study to investigate the possible effect of green ammonia on the  agricultural and electrical development of a country when using hydroelectric energy - which is often the first option for implementing modern energy in SSA. Several studies have previously examined the possibility of green ammonia displacing existing supplies of brown ammonia,26, 27 including the building of a pilot-plant for producing green ammonia from a wind turbine in Morris, Minnesota.28 Conversely, this work analyses the economic benefit of green ammonia in an underdeveloped market without previous fertilizer use, considering its effect on  the national economy rather than the economics for individual farmers.29-31 As such, the economic benefit to rice cultivation through increased yields when importing fertilizer or synthesizing fertilizer from local hydroelectric energy is examined with respect to the cost of importing rice as a baseline. Coffee cultivation and rice irrigation are added for supplemental economic benefit. The comparison of domestic green ammonia to domestic brown ammonia made from imported methane is not included as it is expected that investment in new infrastructure should be renewably-based, or otherwise use pre-existing non-renewable infrastructure. While green ammonia for N-fertilizer is the primary focus of this work because agricultural development is typically the foundation for economic growth in impoverished countries, the capacity of ammonia to buffer seasonal hydroelectricity is also analyzed. A number of other studies have examined the cost of energy when ammonia is used as a buffer for renewable energy.32-36 In this work, the analysis is limited to the buffering capacity of ammonia because a complete economic cost-benefit analysis relative to N-fertilizer would require a determination of the complex economic benefit of implementing electricity where it was previously non-existent. Using estimates of parameters pertaining to ammonia synthesis and fertilizer application, we demonstrate the economic benefits associated to the production of domestic green ammonia derived N-fertilizer compared to simply importing N-fertilizer and the current situation of importing rice. In addition, green ammonia can buffer seasonal fluctuations of hydroelectricity to produce a consistently available 370 MW of power. While this study of green ammonia in Sierra Leone is a preliminary analysis using estimates of uncertain parameters, it exemplifies the positive potential of renewably derived green ammonia on the development of impoverished countries.

Results

The use of green ammonia as fertilizer

The population of over 7 million in Sierra Leone is largely dispersed, as shown in Figure 1A, with only 1 million people living in the capital city of Freetown in the East of the country. Using the rice production (Table S3) and the population of each center, a map of rice balance indicates that there is a widespread deficit of rice (illustrated by red circles in Figure 1B). While a few centers produce more rice than they consume over the course of a year (green circles in Figure 1B), the majority of centers require importation of rice – the largest of which is Freetown because of its urban population.

Importing fertilizers as a baseline

As a baseline, the cost reduction when importing fertilizer for rice cultivation relative to importing rice is calculated representing the conventional route to increasing yields, alleviating hunger, and spurring growth. If no fertilizer is utilized, the yearly cost for imported rice is estimated as $114M and $8M for rice transport. When fertilizer is utilized, the import and transport costs for fertilizer are $56M and $6M, respectively, while the cost of rice transport to redistribute excess within the country is now $10M. Therefore, the estimated yearly cost savings are $50M – a substantial value considering the national GDP is approximately $4B. These results are contrary to previous general notions that infrastructure and transport costs are limiting in the price of goods in SSA.31, 37, 38 This is likely the result of Sierra Leone being a very small country with a sea port, whereas landlocked or expansive countries could have different cost characteristics. Further, the cost of maintaining a supply chain for both rice and fertilizer over the given area is not included in the present cost estimates and could increase the transport cost relative to wholesale import. However, as fertilizer contains more value per mass it is likely to require a less expensive supply chain if additional safety considerations are minimal, as would be the case when using urea rather than ammonia.

Even though transport cost only constitutes ~10% of fertilizer costs, it has a noticeable effect on the locations of optimal fertilizer use, illustrated in Figure 2.  In the baseline scenario, the Kambia district in the Northwest of the country shows a strong positive correlation of fertilizer consumption with overall cost savings because this district currently has the highest starting rice yield (0.87 t/ha milled equivalent) due to a high proportion of lowland rice cultivation, and doubling that yield maximizes the usefulness of fertilizer. Meanwhile, the Koinadugu district in the Northeast of the country has a large negative correlation of fertilizer use with cost savings because the district is far from major population centers and requires substantial travel on dirt roads for high cost. Nevertheless, transport costs are expected to be lower for fertilizer than rice due to the considerably lower volume and weight of the former.

The effect of increasing or decreasing fertilizer and rice transport cost is illustrative of general principles useful in fertilizer distribution. When transport costs increase 50% across all roads (Figure 2a), there is a preference to use fertilizer closer to major population centers so rice does not need to be transported as far. The Port Loko and Bombali districts (region 1) do not have a high starting rice yield (0.52 & 0.53 t/ha) but are a short distance on paved roads from Freetown – the largest rice consumption location. Indeed, the center of Bo (region 2) switches from negative to positive correlation in the case of higher transport costs because Bo is another major population center even though it has a low starting yield. On the other hand, if transport costs are reduced 50%, those districts close to Freetown but with low starting yields switch to negative correlation (region 1), while the highest yield region in the Northwest becomes more favored. These general trends indicate that when transport costs are high, it is optimal to produce rice as close to population centers as possible, and when transport costs are low it is optimal to maximize the effectiveness of the fertilizer in yield increase. It is important to note that this analysis assumes fertilizer has a proportional effect on yields (i.e. the most initially productive regions have the best potential for fertilizer use), which is true if variation in yields are primarily due to farming practices and types of rice cultivation rather than fluctuations in soil depletion.

The sensitivity of the fertilizer profitability at the national scale is assessed in Figure 3 for a number of relevant parameters highlighting several crucial points. First, transport costs have a small impact on overall cost savings of providing rice to the country, and while fertilizer price has a much stronger effect, profitability is maintained up to an 85% price increase. Thus, extra costs associated with domestic marketing can be absorbed without losing profitability. Second, the increase in rice yields with fertilizer – initially estimated to be 100% (i.e. double current yields) - is the parameter most likely to disrupt the profitability of using fertilizer. If yields increase by only 60% rather than 100%, the use of fertilizer does not have any effect on the general cost of providing rice as the total cost of the fertilizer is higher than the benefits from it. This emphasizes that fertilizer must be managed properly and supplemented with additional measures such as improved seeds and better agricultural practices. In the best case, a thorough adoption of the best modern farming practices could establish parity with worldwide rice yields (a yield response of >400%) and significantly enhance the economic benefit of fertilizer. Lastly, if the import price of rice increases or more rice is consumed per person to approach the FAO target of 150kg per year, the cost savings with fertilizer implementation increase dramatically.

Production of Domestic Green Ammonia for N-fertilizer

The 27 sites for potential hydroelectric power in Sierra Leone vary both spatially and according to yearly power output per initial capital investment. All undeveloped sites39 are analyzed for green ammonia production applied to rice cultivation as N-fertilizer in conjunction with imported monoammonium phosphate (MAP) and potassium chloride. The payback period of each site is shown in Figure 4 with respect to importing rice (primary y-axis) and importing fertilizers (secondary y-axis). Payback period is utilized for comparison rather than NPV to include those sites outside a typical 30 year project lifetime. The payback period with respect to importing fertilizer represents the benefit of domestic N-fertilizer production relative to its import. In this case, the payback period is considerably longer than that relative to imported rice. Nevertheless, both analyses indicate that the amount of energy generated per capital cost is the most important parameter as both sets of data follows a similar trend. The small amount of scatter in the data indicates that the location of the hydroelectric plant relative to the optimal areas for rice cultivation is much less significant than the energy pay-out of the hydroelectric dam.

Figure 5b presents the 30 year NPVs with a 2% inflation discount rate for the eight most promising hydroelectric plant sites with green ammonia production for N-fertilizer. While the cost benefit ranges from $600 million to $1.2 billion when including the general savings of rice no longer imported, the benefit relative to no longer importing N-fertilizer ranges from only $50 million to $225 million. Imported N-fertilizer for rice cultivation can be entirely displaced by domestic green ammonia with a combination of the Benkagor 3 and Yiben 1 sites (17% of total available hydroelectric energy). The capital cost break-downs for three hydroelectric green ammonia plants are illustrated in Figure 5a. In all cases, the hydroelectric dam constitutes the majority of capital costs and the remaining ammonia synthesis components scale with the rated MW capacity, of which the synthesis loop and electrolyzers constitute the largest costs.

The dependence of 30-year cost savings in terms of displaced import of N-fertilizer when produced domestically through green ammonia is shown in Figure 6 for the Benkagor 3 site. As expected, the model is particularly sensitive to the cost of fertilizer. While it is possible that imported fertilizer could become less expensive in the future, the present understanding suggests that fertilizer costs may be largely underestimated by the wholesale import price, as previously discussed, and therefore the benefit of domestic N-fertilizer could be higher. Similarly, the energy efficiency (i.e. energy loss) of the electric HB process has a large impact on NPV, while the capital cost of the synthesis process is less important because it is overshadowed by the capital cost of the hydroelectric plant (Figure 5a). The reduction in the capital costs of ammonia synthesis has been the focus of innovation40, 41 because it limits the economics of a small-scale renewable energy installation (MWs) with unfavorable economies-of-scale. Medium and large-scale instillations (>10s MW) necessarily exhibit a high dependence on the efficiency to convert electricity to ammonia. The capital of renewable energy often constitutes a significant fraction of projected costs for green ammonia32, 34, 36. In addition, the efficiency of energy conversion is crucial (though often overlooked) because it determines the ability to utilize capital investment (assuming the capital has a high level of utilization). This principle is particularly applicable when using hydroelectric energy rather than wind or solar energy because it involves higher capital costs relative to operating costs.

The current model for domestic N-fertilizer in Sierra Leone using hydroelectric power indicates only a minor effect of transportation costs relative to the whole cost (Figure 6). N-fertilizer must be distributed all across the country regardless of whether it is imported through Freetown or produced domestically at a hydroelectric dam from green ammonia, and therefore an increased transport cost will be equally negative for both scenarios. However, if the transport cost is applied as an additional distance to the point of final use (i.e. a landlocked country far from a port), the economics are more strongly favorable for domestically sourced N-fertilizer. As the distance over land to the primary point of distribution increases to 1,000km, the benefit of domestic fertilizer nearly doubles from $230mill to $410mill. In practice, this indicates the much larger economic benefit of domestic fertilizer to landlocked countries like Mali or Burkina Faso, where the distance from the nearest port to the capital city is approximately 900 km and 1,000 km, respectively. The same economic benefit would apply to large seaside countries where the agricultural land is far from the port.

The hydroelectric site Benkagor 3 is able to provide 60% of the green ammonia needed for N-fertilizer in rice cultivation throughout Sierra Leone (Figure S.2). While adding Yiben 1 as a second hydroelectric site meets the balance of N-fertilizer, the return on investment decreases from 165% to 110% because the hydroelectric energy requires more capital per energy generated. When coffee cultivation is also considered as a potential application for green ammonia, it is economically optimal to divert some green ammonia from rice to coffee cultivation because it nearly doubles the return on investment, but a third hydroelectric site (Betmai 1) is then required to meet the entire N-fertilizer market. Alternatively, implementing lowland rice irrigation with fertilizer results in a more efficient use of green ammonia to saturate the N-fertilizer market, but only a small economic benefit (Figure S.2) due to the costs of irrigation. Nevertheless, the greatest return on investment in every case is with a single hydroelectric plant (Benkagor 3) because it is the most cost effective energy production. The mass flows of fertilizer, rice, and coffee for the case of the Benkagor 3 site (Figure 7) indicate that imported N-fertilizer is concentrated near the port of Freetown, while green ammonia derived N-fertilizer is used across the interior to produce surpluses of rice that are funneled to Freetown. Additionally, even with a single hydroelectric site, some green ammonia is optimally diverted to coffee cultivation because even though it is not a food staple, cash crops like coffee are beneficial to the overall economy.

Green Ammonia as seasonal energy Buffer of Hydroenergy

Rivers in Sierra Leone – and SSA generally – often experience drastic fluctuations in flow between wet and dry seasons.25 This can be partially alleviated by constructing large reservoirs, however the geography or social demographics of the region frequently prohibit flooding large areas. Run-of-the-river hydroelectric dams (as opposed to reservoir dams) harness the energy in water with minimal obstruction to flow, and therefore have a much smaller impact on the surrounding area, but also have less buffer against seasonal fluctuations. In this context, ammonia could be used for seasonal energy storage, being synthesized in the wet season and decomposed into hydrogen for electricity use in the dry season. In such a scenario, additional facilities would be required to convert ammonia back into electricity by either using ammonia directly in turbines or solid oxide fuel cells, or decomposing it to produce hydrogen for a PEM fuel cell.42, 43 However, an economic analysis including this equipment and other infrastructure to generate a cost-benefit scheme for buffered electricity storage is considered outside the current scope.

As shown in Figure 8 while over 900 MW are produced in the wet season, this drops to less than 100 MW for several months during the dry season. Using green ammonia as an energy buffer allows consistent energy supply at approximately 370MW. With a population of 7.4 million, this would raise the electricity energy consumption to 435 kWh per capita, which is more than twice the average value for West Africa (188 kWh/capita) but far below North Africa (1574 kWh/capita).9 Even if hydropower is first used for fertilizer production, it is crucial that electricity provision also soon develops to grow the industrial sector and absorb excess labor from the agricultural sector. Otherwise, excess labor will shift to mining – Sierra Leone’s largest current export market – which is a notoriously unproductive sector in terms of labor.44

Additional considerations

The study presented herein is focused on the agricultural and economic opportunities offered by the development of green ammonia technologies in developing countries, taking Sierra Leone as a case study. However, a holistic approach needs to be taken when developing related policies to acknowledge the individual social, cultural, environmental and political aspects.

The implementation of inorganic fertilizer around the world has garnered criticism due to negative side-effects for the environment. It is projected that N and P induced eutrophication – the depletion of oxygen in a body of water through algal blooms – will increase 250% by 205045 due to runoff of excess fertilizer. When deploying the use of fertilizers for the first time where it previously lacked (i.e. SSA), environmental impacts and market development aspects should be considered with a  focus on balance between synthetic and natural methods. Lessons should be learnt from previous experiences in India, China and Thailand with low efficiency of N-fertilizer usage when first intensified (30-40%) and how it can be increased with better farming practices (60-70% in USA and France).46 The strategies for the use of fertilizers should be tailored to fit the soil characteristics of any particular region, and natural means of improving soil health should be implemented in conjunction whenever possible.

The development of a market for fertilizer is limited by factors involving both supply and demand which necessitate a multifaceted approach to market interventions. Preliminary attempts such as the Millennium Villages Project started in 2004 through the UN, invest not only in soil nutrients but also in communities through training in farming practices and market access.47. These initiatives closely match those proposed by The World Bank as part of a multifaceted approach to increasing agricultural outputs.48

When viewed from the perspective of sustainable market development, the current analysis of domestic green ammonia production for N-fertilizer is properly positioned to achieve a multi-faceted approach. On the supply side, a large addition of N-fertilizer to the market through an associated green ammonia plant would overcome many of the barriers for market entry through economies-of-scale. On the demand side, the commissioning of green ammonia plants will open up opportunities for job diversification beyond agricultural ones, setting the foundations of a market-oriented agricultural sector and an industry-based economy.

Conclusions

This study uses Sierra Leone as a case study representing the sub-Saharan Africa shows that the production of green ammonia using renewable energy for N-fertilizer is economically beneficial compared to the already beneficial baseline of importing N-fertilizer with the potential of modernizing agriculture as a based on an industrial-based economy transformation. In addition, green ammonia can buffer seasonal hydroelectricity to provide consistent per capita power above West Africa standards. The capital of a hydroelectric plant relative to its power output drives the economics of domestic N-fertilizer usage and causes high sensitivity to the energy efficiency of the electric HB process – which is a focus of technological innovation. In the future, the energy consumption for the electric Haber Bosch (HB) process may be as low as 35 MJ/kg NH3 as opposed to the current estimate of 40 MJ/kg NH3 (26% efficiency gain), particularly when using a low-pressure HB process with absorptive separation. An increase in energy efficiency will also strongly impact the available yearly energy buffer as the current efficiencies result in 80% of wet season energy being lost through ammonia storage. Additionally, while the capital cost of the ammonia synthesis loop is insignificant relative to a hydroelectric plant, it may be impactful when using other renewable energies. Hydroelectric power is already centralized and fluctuates slowly– congruent with the current HB process – but solar and wind energy are dispersed and fluctuate quickly, likely requiring a modified HB process that is low-capital and able to respond to a changing energy supply.

Experimental Procedures

Resource Availability

Lead Contact

Further information and requests for resources and reagents should be directed to and will be fulfilled by the Lead Contact, Laura Torrente-Murciano (lt416@cam.ac.uk)

Materials Availability

This study did not generate new unique materials

Data and Code Availability

The datasets generated during this study underpinning the described model are available in the SI and at the Cambridge research repository Apollo https://doi.org/10.17863/CAM.62731

Model details

A model for the spatial distribution of rice and fertilizer was developed by dividing Sierra Leone into 48 centers (Table S2, S3, S4) characterized by population, rice consumption (based on population), rice cultivation area (upland and lowland),49 rice yield50 and distance to every other center (distinguishing paved and dirt roads). Each center has an existing positive/negative annual rice balance, and can utilize fertilizer to fill rice deficits and/or generate a surplus for transport to neighboring centers, rather than importing rice at approximately $366/tonne.51 Fertilizer can also be utilized for coffee cultivation as a cash crop in 3 centers (Kenema, Kailahun, and Kono) in the eastern region of the country, and can be sold at port for approximately $1300/tonne.52 Further details of the model can be found in the Supplemental Experimental Procedures in the SI.

The model used for analysis solves for the distribution of fertilizer to 51 possible centers (48 rice, 3 coffee) to maximize overall economic benefit by randomly assigning fertilizer in many iterations (>10,000) and calculating the cost savings (i.e. profit) in each case. Taken together, the iterations form a normal distribution that is representative of the simulation and determines the approximate maximum. This approach does not systematically find the absolute optimum in the distribution of fertilizer to 51 centers as in mixed integer non-linear optimization however, the computational complexity is reduced leading to an approximate optimum. Considering the highly uncertain constraints of the model, an exact optimum is unnecessary. The net present value (NPV) is utilized to measure economic benefit as it evaluates the balance between large up-front capital costs and substantial yearly reduction in expenditures over 30 years with a 2% rate of inflation. In cases where the NPV is negative, payback period (i.e. years to payback initial capital investment) is used as the metric for analysis. Details of the numerical model can be found in the supplementary information.

Fertilizer is mainly composed of nitrogen, phosphate, and potassium, which are assumed to be applied as urea, monoammonium phosphate, and potassium chloride. When imported, these components cost approximately $310/tonne, $580/tonne, and $310/tonne, respectively (Table S5).53 If a farm plot utilizes fertilizer (in the model) it is assumed to apply the optimum quantities of each component for both rice and coffee cultivation, which are approximately 99 kg N, 15 kg P, 30 kg K and 163 kg N, 11 kg P, 128 kg K per hectare, respectively.54, 55 Important to note, these fertilizer prescriptions are estimates suitable for economic comparison, but should be refined through the testing of soils in specific localities.

When imported fertilizer is replaced by domestic supplies, only the N component (N-fertilizer) is considered through green ammonia, with the remaining P and K components are imported to maintain the optimum balance. Assuming that direct ammonia N-fertilizer should not be used in developing countries due to safety considerations, ammonia needs to be converted to either urea,56 ammonium bicarbonate,57 or ammonium sulfate, in which case a feed of CO2 will be required. Since the envisioned process is powered by renewable energy, the CO2 will need to be derived from biomass gasification/pyrolysis,19, 58, 59 which will increase capital and operating costs while also supplying additional energy. Capital costs and additional benefits of this process were not considered in the model.

Despite the majority of the rural population in Sierra Leone growing rice, the yields are quite low, ranging from 0.47-0.88 t/ha (Table S6). The effect of fertilizer is estimated to double yields due to their current low yield values.60-64 This value is highly variable and likely depends on other factors such as improved seed varieties65, 66 and farmer education for proper application. Additionally, irrigation can be incorporated in conjunction with fertilizer – particularly with lowland rice – increasing yields to an estimated triple the original value, which it still below world standards.12, 30, 67, 68 For coffee cultivation, current yields in the 3 centers are 190, 165, and 61 kg/ha (Table S7).49 With fertilizer, they are estimated to be 400 kg/ha, which is at the lower end of the yields potential for a refurbished farm (400-600 kg/ha52) and well below yields in the East Africa (1000 kg/ha).69

Green ammonia – whether for N-fertilizer or energy storage in the electrical grid – is produced using the electric HB process. Electricity is derived from one of the potential hydroelectric sites in Sierra Leone, and therefore the feedstock cost is the capital and operating costs of the hydroelectric plant. Additionally, the rated capacity of the hydroelectric plant determines the capacity of the electric HB process, assuming an initial overestimate of 40MJ/kg NH3 required for conversion,40 and the capital cost of the electrolyzer, PSA and synthesis loop are estimated using this capacity.

The buffering capability of green ammonia for the electrical grid is analyzed by examining  the rated capacity and estimated yearly energy output (Table S1) of Sierra Leone’s 26 potential hydroelectric sites.39 By approximating a binary energy output of both wet and dry seasons, and assuming each site on average experiences the same ratio season fluctuation as Benkagor 3 (from 85MW to 5MW39), the length of the wet season for each site in weeks is estimated using the yearly power output and rated capacity. The round-trip efficiency of using ammonia as energy storage is estimated as 20% (50% for ammonia synthesis, 40% for ammonia decomposition).70 With this model, the buffered yearly energy supply is calculated using all potential hydroelectric sites.

Acknowledgements

LTM would like to acknowledge the UK Engineering and Physical Science Research Council for her Fellowship award (grant number EP/L020432/2). CS is thankful to The Cambridge Trust for partially funding his scholarship.

Author Contribution

DECLARATION OF INTERESTS

The authors declare that they have no competing interests.

References

Uncategorized References

1.	Correlation of energy cosumption and GDP per person: European Environment Agency (2015). [updated 28-2-1527-4-20]. Available from: https://www.eea.europa.eu/data-and-maps/figures/correlation-of-per-capita-energy.

2.	Sanchez, PA, Swaminathan MS. (2005) Hunger in Africa: the link between unhealthy people and unhealthy soils. Lancet 365(9457), 442-4.

3.	Odhiambo, NM. (2010) Energy consumption, prices and economic growth in three SSA countries: A comparative study. Energy Policy 38(5), 2463-9.

4.	Stern, DI. (2010). The Role of Energy in Economic Growth. USAEE, IAEE.

5.	Schwerhoff, G, Sy, M. (2017). Financing renewable energy in Africa - Key challenge of the sustainable development goals. Renewable & Sustainable Energy Reviews 75, 393-401.

6.	Lipscomb, M, Mobarak, AM, Barham, T. (2013). Development Effects of Electrification: Evidence from the Topographic Placement of Hydropower Plants in Brazil. American Economic Journal-Applied Economics 5(2), 200-31.

7.	Grogan, L, Sadanand, A. Rural Electrification and Employment in Poor Countries: Evidence from Nicaragua. World Development 43, 252-65.

8.	Barrett, CB, Christiaensen, L, Sheahan, M, Shimeles, A. (2017). On the Structural Transformation of Rural Africa. World Bank Group Jobs Cross Cutting Solution Area.

9.	(2015) I. Africa 2030: Roadmap for a Renewable Energy Future. Abu Dhabi.

10.	Gyamfi, S, Modjinou, M, Djordjevic, S. (2015). Improving electricity supply security in Ghana-The potential of renewable energy. Renewable & Sustainable Energy Reviews 43, 1035-45.

11.	Khush, GS (2001). Green revolution: the way forward. Nature Reviews Genetics 2(10), 815-22.

12.	Pingali PL. (1997) From Subsistence to Commercial Production Systems: The Transformation of Asian Agriculture. American Journal of Agricultural Economics 79, 628.

13.	Evenson RE, Gollin D. (2003). Assessing the impact of the Green Revolution, 1960 to 2000. Science 300(5620),758-62.

14.	Lynch, JP. (2007). Roots of the second green revolution. Australian Journal of Botany, 55(5) 493-512.

15.	Bumb. BL, Johnson. ME, Fuentes. PA. (2011). Policy Options for Improving Regional Fertilizer Markets in West Africa. International Food Policy Research Institute.

16.	Ammonia: zero-carbon fertilizer, fuel and energy store. 2020.

17.	2019 Human Development Index Ranking. In: Programme UND, editor. 2019.

18.	Least Developed Countries (LDCs): United Nations Department of Economic and Social Affairs; 2020 [Available from: https://www.un.org/development/desa/dpad/least-developed-country-category.html.

19.	National Energy Profile of Sierra Leone. (2012). United Nations Development Program.

20.	State of Food Security in Sierra Leone 2015. (2015). MAFFS, FAO, WFP.

21.	AJG, Simoes CH. (2011) The Economic Complexity Observatory: An Analytical Tool for Understanding the Dynamics of Economic Development. Workshops at the Twenty-Fifth AAAI Conference on Artificial Intelligence.

22.	Monica M P, Thilsted; SH, Phillips; MJ, Koroma AS. (2016). Food and Nutrition security in Sierra Leone with a focus on fish in Tonkolili District. Penang, Malaysia: WorldFish.

23.	National Sustainable Agriculture Development Plant 2010-2030. (2009). In: Agriculture F, and Food Security, editor. Freetown, Sierra Leone: The Republic of Sierra Leone.

24.	Average precipitation in depth: The World Bank; 2019 [Available from: https://data.worldbank.org/indicator/AG.LND.PRCP.MM?view=map.

25.	Kling H. (2017). GIS Hydropower Resource Mapping and Climate Change Scenarios for the ECOWAS Region, Country Report for Sierra Leone. Praia, Cabo Verde.

26.	Allman, A., Daoutidis, P., Tiffany, D., Kelley, S. (2017). A framework for ammonia supply chain optimization incorporating conventional and renewable generation. Aiche Journal 63(10), 4390-402.

27.	Palys, MJ., Allman, A., Daoutidis, P. (2019). Exploring the Benefits of Modular Renewable-Powered Ammonia Production: A Supply Chain Optimization Study. Industrial & Engineering Chemistry Research 58(15), 5898-908.

28.	Reese M, Marquart C, Malmali M, Wagner K, Buchanan E, McCormick A, Cussler E. (2016). Performance of a Small-Scale Haber Process. Industrial & Engineering Chemistry Research 55(13), 3742-50.

29.	Louhichi K, Paloma SGY. (2014). A farm household model for agri-food policy analysis in developing countries: Application to smallholder farmers in Sierra Leone. Food Policy 45, 1-13.

30.	Chenoune R, Allen T, Komarek AM, Paloma SGY, Flichman G, Capillon A, Belhouchette, H. (2017). Assessing consumption-production-resources nexus decisions for rice-focused agricultural households in Sierra Leone. Land Use Policy 67, 597-607.

31.	Liverpool-Tasie LSO, Omomona BT, Sanou A, Ogunleye W. (2015). Is Increasing Inorganic Fertilizer Use in Sub-Saharan Africa a Profitable Proposition ? Evidence from Nigeria. World Bank Group Africa Region.

32.	Palys MJ, Daoutidis P. (2020). Using hydrogen and ammonia for renewable energy storage: A geographically comprehensive techno-economic study. Computers & Chemical Engineering, 136.

33.	Ikaheimo J, Kiviluoma J, Weiss R, Holttinen H. (2018). Power-to-ammonia in future North European 100 % renewable power and heat system. International Journal of Hydrogen Energy 43(36), 17295-308.

34.	Armijo J, Philibert C. (2020) Flexible production of green hydrogen and ammonia from variable solar and wind energy: Case study of Chile and Argentina. International Journal of Hydrogen Energy 45(3), 1541-58.

35.	Demirhan CD, Tso WW, Powell JB, Heuberger CF, Pistikopoulos EN. (2020) A Multiscale Energy Systems Engineering Approach for Renewable Power Generation and Storage Optimization. Industrial & Engineering Chemistry Research 59(16), 7706-21.

36.	Cesaro Z, Ives M, Nayak-Luke R, Mason M, Bañares-Alcántara R. (2021). Ammonia to power: Forecasting the levelized cost of electricity from green ammonia in large-scale power plants. Applied Energy 282, 116009.

37.	Casaburi L, Glennerster R, Suri T. (2013) Rural Roads and Intermediated Trade: Regression Discontinuity Evidence from Sierra Leone. London, U.K..

38.	Winter-Nelson A, Temu A. (2005). Impacts of prices and transactions costs on input usage in a liberalizing economy: evidence from Tanzanian coffee growers. Agricultural Economics 33(3), 243-53.

39.	Fujii K. (2009). Final Report on the master plan study on power supply in western area in The Republic of Sierra Leone. Japan International Cooperation Agency.

40.	Smith C, Hill AK, Torrente-Murciano L. (2020). Current and future role of Haber-Bosch ammonia in a carbon-free energy landscape. Energy & Environmental Science 13(2), 331-44.

41.	Palys MJ, McCormick A, Cussler EL, Daoutidis P. (2018). Modeling and Optimal Design of Absorbent Enhanced Ammonia Synthesis. Processes 6(7).

42.	Yun Zhao BPS, Junhua Wang, Jared Nash, Teng Wang, Bingjun Xu, Yushan Yan. (2019). An Efficient Direct Ammonia Fuel Cell for Affordable Carbon-Neutral Transportation. Joule.

43.	Valera-Medina A, Gutesa M, Xiao H, Pugh D, Giles A, Goktepe B, Marsh, R., Bowen, P. (2019) Premixed ammonia/hydrogen swirl combustion under rich fuel conditions for gas turbines operation. International Journal of Hydrogen Energy 44(16), 8615-26.

44.	McMillan M, Rodrik D, Verduzco-Gallo I. (2014). Globalization, Structural Change, and Productivity Growth, with an Update on Africa. World Development 63, 11-32.

45.	Tilman D, Fargione J, Wolff B, D'Antonio C, Dobson A, Howarth R, Schindler, D., Schlesinger, W., Simberloff, D., Swackhamer, D. (2001). Forecasting agriculturally driven global environmental change. Science, 292(5515):281-4.

46.	Zhang X, Davidson EA, Mauzerall DL, Searchinger TD, Dumas P, Shen Y. (2015). Managing nitrogen for sustainable development. Nature 528(7580), 51-9.

47.	Sanchez PA, Denning GL, Nziguheba G. (2009). The African Green Revolution moves forward. Food Security 1(1):37-44.

48.	Morris M, Kelly VA, Kopicki RJ, Byerlee D. (2007). Fertilizer Use in African Agriculture. Washington, D.C..

49.	L.S.Gboku M, Davowa SK, Gassama A. (2017). Sierra Leone 2015 Population and Housing Census: Thematic Report on Agriculture.

50.	Spencer DSC, Deen S, Wilson C. (2009). Economics of Rice Production in Sierra Leone: Report of a Survey in Three Northern Districts. Freetown, Sierra Leone.

51.	Trimm DL, Stanislaus A. (1986). THE CONTROL OF PORE-SIZE IN ALUMINA CATALYST SUPPORTS - A REVIEW. Applied Catalysis 21(2), 215-38.

52.	Spencer DSC.(2009). Farmer Perceptions, Child Labour, and Economics of  Tree Crops Production and Marketing in Kailahun,  Kenema and Kono Districts of Sierra Leone. Freetown, Sierra Leone: Promoting Agriculture, Governance and the Environemnt Project.

53.	Lee HC, Kim HJ, Chung SH, Lee KH, Lee HC, Lee JS. (2003). Synthesis of unidirectional alumina nanostructures without added organic solvents. J Am Chem Soc. 125(10), 2882-3.

54.	Bijay-Singh, Singh VK. (2017). Fertilizer Management in Rice.  Rice Production Worldwide: Springer, 217-53.

55.	Ohring M. (1992) The materials Science of thin films. San Diego, California: Academic Press.

56.	Meessen J. Urea synthesis. (2014). Chemie Ingenieur Technik 86(12), 2180-9.

57.	Ching-Kwei L, Rong-Yen C. (1980) Ammonium bicarbonate used as a nitrogen fertilizer in China. Fertilizer research 1, 125-36.

58.	Oreggioni GD, Brandani S, Luberti M, Baykan Y, Friedrich D, Ahn H. (2015) CO2 capture from syngas by an adsorption process at a biomass gasification CHP plant: Its comparison with amine-based CO2 capture. International Journal of Greenhouse Gas Control 35, 71-81.

59.	Xie QL, Borges FC, Cheng YL, Wan YQ, Li Y, Lin XY, Liu, YH., Hussain, F., Chen, P., Ruan, R. (2014). Fast microwave-assisted catalytic gasification of biomass for syngas production and tar removal. Bioresource Technology 156, 291-6.

60.	Katic PG, Namara RE, Hope L, Owusu E, Fujii H. (2013). Rice and irrigation in West Africa: Achieving food security with agricultural water managment strategies. Water Resources and Economics 1, 75-92.

61.	Andrews R, Jacques D, Qian D, Dickey EC. (2001) Purification and structural annealing of multiwalled carbon nanotubes at graphitization temperatures. Carbon 39(11), 1681-7.

62.	Ye Q, Zhang H, Wei H, Zhang Y, Wang B, Xia K, Huo, Z., Dai, Q., Xu, K. (2007). Effects of nitrogen fertilizer on nitrogen use efficiency and yield of rice under different soil conditions. Frontiers of Agriculture in China 1, 30-6.

63.	Haefele SM, Naklang K, Harnpichitvitaya D, Jearakongman S, Skulkhu E, Romyen P, Phasopa, S., Tabtim, S., Suriya-arunroj, D., Khunthasuvon, S., et al. (2006). Factors affecting rice yield and fertilizer response in rainfed lowlands of northeast Thailand. Field Crops Research, 98(1):39-51.

64.	Islam SMM, Gaihre YK, Shah AL, Singh U, Sarkar MIU, Satter MA, Sanabria, J.,  (2016). Rice yields and nitrogen use efficiency with different fertilizers and water management under intensive lowland rice cropping systems in Bangladesh. Nutrient Cycling in Agroecosystems 106(2), 143-56.

65.	Saito K, Sokei Y, Wopereis MCS. (2012). Enhancing Rice Productivity in West Africa through Genetic Improvement. Crop Science 52(2), 484-93.

66.	NERICA: the New Rice for Africa - a Compendium. (2008). Cotonou, Benin.

67.	Hussain I, Hanjra MA. (2004). Irrigation and poverty alleviation: Review of the empirical evidence. Irrigation and Drainage 53(1), 1-15.

68.	Jacobsen CJH, Dahl S, Hansen PL, Tornqvist E, Jensen L, Topsoe H, Prip, D., Moenshaug, P., Chorkendorff, I. (2000). Structure sensitivity of supported ruthenium catalysts for ammonia synthesis. J Mol Catal A-Chem 163(1-2), 19-26.

69.	Bucagu C, Vanlauwe B, Giller KE. (2013). Managing Tephrosia mulch and fertilizer to enhance coffee productivity on smallholder farms in the Eastern African Highlands. European Journal of Agronomy 48, 19-29.

70.	Giddey S, Badwal SPS, Munnings C, Dolan M. (2017). Ammonia as a Renewable Energy Transportation Media. Acs Sustainable Chemistry & Engineering 5(11), 10231-9.

CAPTIONS OF FIGURES

Figure 1  Population and Rice Distribution of Sierra Leone. A. approximate population centers. B. balance of rice consumption and production. The relative value for each center is proportional to the diameter of the circle. Negative rice balances are red and positive are green.

Figure 2  Spatial Distribution of Fertilizer with Variable Transport Costs. Positive and negative correlations of fertilizer application with cost savings are presented yellow and red, respectively, for the control scenario (B), transportation costs increased 50% (A), and transport cost decreased 50% (C). The area of the circles is proportional to the magnitude of the correlation. Regions 1 & 2 are emphasized for the stark changes with increased/decreased transport costs.

Figure 3  Cost Savings for providing rice to Sierra Leone in the case of importing fertilizers as a function of percent change of a number of relevant parameters. The percent yield increase was initially estimated to be 100% (i.e. double the yield without fertilizer). The percent yield increase is the parameter most likely to inhibit the profitability of fertilizer use because improper fertilizer usage and/or lack of other inputs (i.e. improved seeds) could easily cause yield increase to be only 60% (or less). Alternatively, the adoption of modern farming practices are expected to cause a much higher yield increase.

Figure 4 Payback Period of Individual hydroelectric Sites According to Energy per Capital. Payback period with respect to imported rice includes the national economic benefit of using fertilizer for rice cultivation, while payback period with respect to imported fertilizer only assesses the benefit of domestically sourcing N-fertilizer from green ammonia rather than importing, and therefore requires a long time to return capital investment. The tight pattern of data (low scatter) indicates that the yearly energy generated by the hydroelectric dam is of primary importance compared to both rated capacity of the hydroelectric plant or location relative to rice cultivation.

Figure 5  Economics of Eight Promising Hydroelectric Sites in Sierra Leone. A. capital costs (in $ Millions) of three potential sites with green ammonia plants. The hydroelectric dam constitutes the largest capital in all cases, and the remaining components depend on the rated capacity of the dam. B. 30-year net present value (NPV) of eight most promising sites with respect to both imported rice and imported fertilizer. C locations of the eight sites. Circle size is proportional to energy generated per capital cost.

Figure 6  Sensitivity analysis of net present value (NPV) for Benkagor 3 with respect to imported N-fertilizer. N-Fertilizer cost may be much higher than estimated because of domestic marketing costs, and this would significantly influence the value of domestically sourced N-fertilizer. Technological innovations should focus on reducing the energy loss of the electric ammonia synthesis process to make the biggest gain.

Figure 7 Spatial Distribution for the Hydroelectric Site Benkagor 3. A. Potential Distribution of N-fertilizer from hydroelectric site Benkagor 3 and imported from Freetown. P-K-fertilizer is only imported. Area of circle is proportional to amount of fertilizer.  B. Distribution of excess/deficit for rice and coffee. Red/orange represents deficit and excess of rice, respectively. Area of circle is proportional to mass of rice/coffee.

Figure 8  Estimated Electric Energy Supply over a Year. The round-trip efficiency for storing energy in ammonia is approximated as 20%. All potential hydroelectric sites in Sierra Leone are included for analysis.