Optimising Prostate mpMRI: Prepare for success

Highlights

- Routine use of antiperistaltic agents prior to mpMRI is recommended

- Rectal distension significantly distorts DWI and should be minimized prior to mpMRI

- 3 days refraining from ejaculation may improve the evaluation of seminal vesicles

- PROPELLER DWI significantly improves DW image quality in patients with hip metalwork

- Acquisitions in the plane axial to the patient are recommended

Introduction

Prostate cancer (PCa) is the commonest malignancy in Western men and is the second leading cause of cancer-related mortality [1], with the incidence forecasted to double by 2030 [2]. Diagnosis of PCa has traditionally relied on prostate specific antigen (PSA) levels, digital rectal examination (DRE) and systematic trans-rectal ultrasound (TRUS) guided biopsy [3]. However, there is increasing evidence that the pre-biopsy multiparametric MRI (mpMRI) outperforms systematic TRUS-guided biopsy and can lead to an increased detection of clinically significant prostate cancer (csPCa) whilst at the same time reducing over-diagnosis of clinically insignificant cancer [4, 5]. MpMRI in combination with clinical risk assessment can potentially obviate the need for biopsy in 25-30% of men [4, 5] due to its high negative predictive value in diagnosing csPCa [6]. As a result, there has been a steady increase in the use of mpMRI and particularly pre-biopsy mpMRI which is now being performed in up to 75% of men with suspicion of prostate cancer in UK [7].

Given the central role of mpMRI in prostate cancer management pathway, imaging of the highest quality is essential. In order to achieve this, in 2015 European Society of Urogenital Radiology (ESUR) and American College of Radiology (ACR) published a second, extended version of prostate imaging and reporting data system (PI-RADS) recommendation guidelines which are aimed at standardising MRI acquisition and interpretation [8]. Detailed guidance is given on technical considerations and diagnostic evaluation, however, no definite recommendation is given regarding patient preparation, mainly due to the lack of sufficient evidence available at the time. MpMRI consists of three key sequences: T2-weighted imaging (T2WI), diffusion-weighted imaging (DWI), and dynamic contrast-enhanced (DCE) imaging [8]. The quality of these sequences depends on the hardware and software utilized and scanning parameters selected, but also on several other factors including bowel peristalsis, rectal distension, the presence of total hip replacement (THR), post-biopsy haemorrhage, and abstinence from ejaculation. Since the publication of PI-RADS v2, several new studies exploring these areas have been performed giving us additional insight on how to better overcome these challenges.

The aim of this review is therefore to briefly summarize technical considerations and highlight the key issues of patient preparation in order to further optimize imaging quality of mpMRI.

Technical considerations

Prostate mpMRI at both 1.5T and 3T can provide satisfactory and reliable diagnostic examinations when appropriate contemporary technology is employed and acquisition parameters are optimized [8]. 3T is generally considered superior [8, 9] due to increased signal-to-noise ratio (SNR), superior spatial resolution and decreased acquisition times [8-10]. Performing mpMRI at magnetic field strengths below 1.5T is not recommended.

The use of an endorectal coil provides an increase in SNR which can lead to improved diagnostic accuracy [9, 11-13]. However, the limited number of head-to-head studies comparing 1.5T-ERC to 3T- pelvic phased-array coil (PPAC) have demonstrated no significant difference in either cancer detection rate or local staging accuracy [14-16]. Additionally, there are several drawbacks to ERC use, including distortion of the prostate contour, near field coil flare, increased time and cost of examination and an important increase in patient discomfort during the examination when compared to the pelvic phased-array coil PPAC; Figures 1, 2; [8, 9]. Routine use is not currently recommended, however, an ERC should be employed to improve SNR on some older 1.5T scanners or in larger patients where SNR in the gland may be reduced with the use of PPAC alone [8]. In case of ERC use, perfluorocarbon fluid or barium sulfate suspensions are advised to fill the balloon in order to minimize the susceptibility artifacts which can affect DWI [8].

Sequences

PI-RADS v2 recommends that T2WI, T1WI, DWI, and DCE pulse sequences are included for all prostate MR exams [8].  A full review of the PI-RADS guidelines for technical parameters is beyond the scope of this article, however, a summary of key recommendations is outlined in Table 1.

Orientation of the axial plane

It is mandatory for DWI and DCE sequences to match the location and slice thickness of axial T2W, thereby allowing synchronous scrolling and comparison between the sequences. All sequences are acquired without breath holding. PI-RADS v1 recommended obtaining an oblique axial plane i.e. orthogonal to the rectum [17], however, the update version of the guidelines make no recommendation on the orientation of the axial plane. Some authors obtain an axial plane perpendicular to the long axis of prostate [18] which could minimize partial volume effects at the posterior border; Figure 3. There is no evidence of potential superiority of one approach over another, but a recent UK consensus paper recommends obtaining a true axial plane to the patient, which is easier to set for radiographers and is more reproducible, especially for active surveillance and measuring lesion size [19].

A potential downside of the true axial approach is compromising correlation with prostatectomy specimens, but this issue will typically only affect research studies [20]. Theoretical mismatches in apex, mid and base may occur when performing cognitive fusion with true axial MR images being compared TRUS probe images in the plane axial to the prostate [21], however, any errors may be less significant that inter-reader variability between radiologists when defining sector location within the gland [22]. Similar mismatches may occur if rigid fusion software is used for biopsy procedures, which use a transrectal probe regardless of a transrectal or transperineal approach. However, any quantification of mismatch and/or error rates are yet to be defined; Figure 4.

Time interval after biopsy

In addition to lesion detection prior to biopsy, indications for mpMRI include local staging of transrectal ultrasound-guided (TRUS) biopsy proven carcinoma or previous negative systematic biopsy [23, 24]. Standard systematic TRUS-guided biopsy acquires 10-12 biopsy cores which induce haemorrhage and inflammation of the gland [25, 26], which may remain for many weeks and even months [27]. Recent biopsy can induce capsular irregularity and be mistaken for extracapsular extension (ECE). Persistence of haemorrhage is not easily predicted and may relate to variation in citrate levels in the PZ epithelial cells [28]; citrate primarily functions as a semen preservative [28] but additionally exhibits anticoagulant properties [29]. The peripheral zone is thus generally the more affected, with areas of haemorrhage exhibiting T1 hyperintensity and intermediate to low T2 signal [30]. The packed cells within tumours act as a barrier to spread of haemorrhage and concentration of citrate is additionally reduced in prostate cancer, thus blood products within cancerous tissue resolves more rapidly than normal PZ [30, 31]. This can be exploited in cases of post-biopsy haemorrhage for detection, with lesions demonstrating the “haemorrhage exclusion sign” (i.e. no T1 hyperintensity) in combination with homogenously low T2, which is highly predictive for malignancy with PPV at 96% [32]; Figure 5. However, presence of haemorrhage more typically causes a diagnostic challenge as it can either mask the tumour, or potentially lead to a false positive call; Figure 6. The T1W images therefore warrant assessment prior to assessment of the other sequences. It should be noted that haemorrhage-induced T2 changes can persist even after resolution of T1 hyperintensity, and therefore knowledge of any previous biopsy also has to be considered, with DWI and ADC are key to differentiate tumour [33, 34]. Haemorrhage may also be present in the seminal vesicles due to either direct trauma from biopsy or by retrograde spread via ejaculatory ducts [35]. The same evaluation approach as for lesion detection is required, in order to avoid overcalling T3b disease [27, 36, 37]; Figure 7.

Overall, post-biopsy haemorrhage can compromise imaging interpretation and there is no clear optimal time window prior to MRI, with persistence in an individual patient hard to predict. Studies have reported conflicting results with suggested time intervals ranging from 0 to 6 weeks, likely due to significant differences in methodology [27, 38, 39]. PI-RADS v2 recommends a minimum delay of 6 weeks for the purpose of local staging [8], however, even at this time-point around half of the patients still exhibit some haemorrhage [37], however, this needs to be balanced against any delay in cancer treatment pathways.

Abstinence from ejaculation

PI-RADS v2 [8] makes no recommendation on abstinence from ejaculation prior to prostate MRI, given the limited available evidence at the time. However, some institutions instruct the patients to refrain from ejaculation for three days prior to MRI examination in order to achieve maximal distension [40, 41]. Several recent studies support this practice, demonstrating a significant decrease in SV volume after ejaculation which could theoretically hinder interpretation of SV invasion (T3b disease) [42-45]; Figure 8. The average reported decreases in SV volume are 41% immediately after one ejaculation [42], and 57% immediately after two consecutive ejaculations [43]. The average volume reduction of 25.6% after 24h and recovery to baseline at day 3 is likely to be of more clinical relevance [44]; Figure 9. It should be noted that this latter study involved relatively young volunteers (mean 35.9 years) and that the process of seminal fluid replenishment may take longer in older individuals [45], hence an abstinence of 3 days was therefore proposed as a minimum [44, 45].

Beside SV volume, ejaculation has also been shown to have effect on ADC and T2 values by dehydrating the gland, i.e. decreasing the bulk water content. Medved et al and Shin et al reported significantly decreased ADC values of the peripheral zone by 14.5% and 18.6%, respectively, whilst the difference in the central gland (combined transition and central zone) was non-significant [42, 43]. Barrett et al. also demonstrated significantly lower ADC values post ejaculation, but with a lower difference at 4% [44], possibly reflecting the whole-gland ADC measurement employed. T2 values have also been shown to be significantly lower post ejaculation with the reported differences in the peripheral zone between 13-18% [42, 43]. The effect of post-ejaculatory status ADC and T2WI is potentially more clinically significant as this may affect lesion conspicuity and detection, whilst SV invasion can be assessed by other secondary signs such as asymmetry or bulging and focal areas of enhancement and/or restricted diffusion.

The current evidence suggests that 3 days of ejaculatory abstinence prior to prostate MRI may be beneficial for SV and PZ evaluation but further longitudinal studies including patients with prostate cancer would help to establish the effect on lesion detection.

Antispasmodic agents

Bowel peristalsis is known to cause motion artefact at MRI, however, it has been suggested that the low pelvic location of the gland, remote from small bowel may limit the benefit of antisperistaltic agents (e.g. hyoscine butylbromide, glucagon) in prostate MRI [46, 47]. HBB administration does carry a risk of side-effects, mainly related to its anti-cholinergic activity [48, 49], and its use will add to the time and the cost of the MRI appointment. Nevertheless, recent studies which are the largest to date [50, 51] are in agreement with earlier work [52, 53] and highlight a significant improvement in image quality of T2WI after intravenous (i.v.) administration of HBB. Caglic et al reported significantly better T2W image quality with less blurring and motion in 173 patients administered i.v. 20 mg HBB [50], and Ullrich et al. showed significantly improved visualisation of anatomical details and decreased motion artefacts on T2W in 70% of 103 patients assessed using 40 mg i.v. HBB, thereby reducing non-diagnostic MRI to <1% [51]. Of the 2 negative studies, it should be noted that Roethke et al reported reduced artefact in all the assessed categories but not reaching significance, and recommended the use of HBB in patients with hyper-motile intestine and flatulence [46]. The second study used an endorectal coil (ECR), which is not reflective of current UK practice, and may have served as a mechanical barrier to movement of the rectum and prostate, thus minimising the effects of HBB [47]. Only one of these studies assessed the mpMRI functional sequences in addition to T2WI [50]. Although the results showed a trend towards improved quality of DWI and DCE, this did not reach significance. This may reflect reduced effect over time or inherent differences in acquisition of these functional sequences, making them less sensitive to motion artefact. HBB anti-peristaltic effect lasts 20-40 minutes [54, 55] suggesting that either a second bolus of HBB prior to DCE [56] or a combination of HBB with longer-acting intramuscular glucagon [54] may be beneficial in the setting of longer imaging protocols, however, this requires further evaluation.

In conclusion, the current evidence suggests routine use of antiperistaltic agents (i.v. 20 mg HBB) prior to prostate mpMRI is necessary to optimise T2W image quality, a key sequence of mpMRI, in particular for TZ assessment and local staging.

Bowel preparation

Increased rectal loading has been shown to significantly correlate with increased DWI distortion and reduced DW image quality by inducing susceptibility artefacts [57]. Figure 10. These especially affect Echo Planar Imaging (EPI) sequences which are currently the most commonly used DWI sequence in abdominal and pelvic imaging, and these effects are further magnified at higher 3T field strengths [58-60]. Susceptibility artefacts occur in areas around the air-tissue interfaces [58-60], such as the rectum and posterior part of the gland, which is where the majority of prostate carcinomas arise [61]. Although no study to date has prospectively evaluated the potential effect of compromised DWI on lesion detection and staging, there is a tendency towards biopsying a higher number of patients as the rectal distension increases and with a higher percentage of these being false positive, suggesting decreased reader confidence [57]. In addition to compromising DWI, increased rectal distension has also been shown to significantly increase motion artefact on T2WI [57] by inducing rectal contractions [62].

A variety of approaches have therefore been proposed in order to overcome these issues , including simple advice for patients to empty their bowel prior to the examination. Another solution may be switching to a prone position to allow air to move to a non-dependent location away from the prostate. More invasive steps involve either actively removing the air from the rectum with a small catheter [8], or administration of a microenema [63, 64]. Griethuysen at al. evaluated 335 1.5T MRI studies, including patients who self-administered a micro-enema shortly before acquisition and reported significantly decreased incidence and severity of gas-induced artefacts [63]. A smaller study at 3T showed no improvement in DWI or T2W image quality with the use of a pre-MRI enema [64], however, only a minority of patients in the control, non-enema group (16%) had a moderate or severely distended rectum and use of the enema itself may introduced rectal air. Another factor to consider is the potential for an enema to irritate the bowel and propagate peristalsis.

Overall, there is currently no technique that has been shown to consistently reduce rectal loading, and thus correlation with DCE which is less prone to susceptibility artefacts [57] is recommended or recalling of patients in specific non-diagnostic patients may be considered. This mirrors the difficulties faced in radiotherapy planning where several strategies have been investigated but with no clear evidence to suggest use of one over another [65, 66]. Thus, non-patient related approaches including post-processing techniques or changing acquisition parameters warrant attention. Among these techniques, reduced field-of-view (FOV) DWI [67, 68], read-out segmented EPI sequences [69], parallel imaging [68], and acquisition of opposite phase encoding polarities to correct magnetic field inhomogeneity [70], or post-processing techniques including correcting for B0 distortion effects [71], all of which have shown promise for improving image quality.

Implanted devices and total hip replacement

Whilst often safe to image at 3T, total hip replacement patients (THR) should be imaged a 1.5T due to significant distortion artefact which compromise image quality and likely render the study non-diagnostic [58, 59, 72]; Figure 11. THR is an established standard treatment for end-stage hip disease [73], the typical patient age-group overlaps with that of prostate cancer and the number of procedures is expected to double by 2030 [74], making this more of an issue. DWI is the dominant sequence for PZ assessment [8], however, it is the most prone to THR artefact, with the echo planar readouts being susceptible to the local B0 magnetic field inhomogeneities induced, with resultant signal dropout and geometric distortions [58-60].

Current evidence for reducing THR distortion is limited. Techniques used to reduce rectal gas distortion [67-71] could theoretically also limit THR induced susceptibility. Rosenkrantz et al. reported improved image quality at 3T MRI by using reduced-FOV DWI with parallel imaging in 2 patients with THR [75]. In a larger 1.5T study by Czarniecki et al. the use of periodically rotated overlapping parallel lines with enhanced reconstruction (PROPELLER) DWI significantly reduced distortion and improved image quality compared to the standard EPI-DWI [76]; Figure 12. Although the latter work showed promising results, additional studies are clearly needed in order to establish a standardized imaging protocol which would provide adequate diagnostic performance for the increasing cohort of patients with THR and suspicion of prostate cancer.

Conclusion

MpMRI has become established for the detection, local staging and treatment planning of prostate cancer, emphasizing the need for high quality studies. Image quality depends on the acquisition parameters employed, but also other factors including bowel peristalsis, rectal distension, post-biopsyhaemorrhage, post-ejaculatory status and presence of metalwork. Increasing evidence exploring aspects of patient preparation has been accrued since the release of PI-RADS v2 guidelines in 2015, however, further prospective studies that include prostate cancer patients are needed in order to inform decision-making and standardization of these aspects for mpMRI protocols.

Acknowledgements

Author T. Barrett acknowledges support from Cancer Research UK, National Institute of Health Research Cambridge Biomedical Research Centre, Cancer Research UK and the Engineering and Physical Sciences Research Council Imaging Centre in Cambridge and Manchester and the Cambridge Experimental Cancer Medicine Centre.

References

- Torre LA, Bray F, Siegel RL, Ferlay J, Lortet-Tieulent J, Jemal A. Global cancer statistics, 2012. CA Cancer J Clin 2015;65:87-108. doi:10.3322/caac.21262.

- Maddams J, Utley M, Møller H. Projections of cancer prevalence in the United Kingdom, 2010-2040. Br J Cancer. 2012;107:1195-202. doi:10.1038/bjc.2012.366.

- Litwin MS, Tan HJ. The Diagnosis and Treatment of Prostate Cancer: A Review. JAMA 2017;317:2532-2542. doi:10.1001/jama.2017.7248.

- Ahmed HU, El-Shater Bosaily A, Brown LC, et al. PROMIS study group. Diagnostic accuracy of multi-parametric MRI and TRUS biopsy in prostate cancer (PROMIS): a paired validating confirmatory study. Lancet 2017;389:815-822. doi:10.1016/S0140-6736(16)32401-1.

- Kasivisvanathan V, Rannikko AS, Borghi M, et al. PRECISION Study Group Collaborators. MRI-Targeted or Standard Biopsy for Prostate-Cancer Diagnosis. N Engl J Med 2018;378:1767-1777. doi:10.1056/NEJMoa1801993.

- Moldovan PC, Van den Broeck T, Sylvester R, et al. What Is the Negative Predictive Value of Multiparametric Magnetic Resonance Imaging in Excluding Prostate Cancer at Biopsy? A Systematic Review and Meta-analysis from the European Association of Urology Prostate Cancer Guidelines Panel. Eur Urol 2017;72:250-266. doi:10.1016/j.eururo.2017.02.026.

- https://www.npca.org.uk/content/uploads/2018/02/NPCA-2017-Annual-Report_final_211117.pdf; Last accessed: September 18, 2018

- Weinreb JC, Barentsz JO, Choyke PL, et al. PI-RADS prostate imaging–reporting and data system: 2015, version 2. Eur Urol 2016;69:16–40. doi:10.1016/j.eururo.2015.08.052.

- B. Turkbey, M.J. Merino, E.C. Gallardo, V. Shah, O. Aras, M. Bernardo, et al. Comparison of endorectal coil and non-endorectal coil T2W and DW MRI at 3T for localizing prostate cancer: correlation with whole-mount histopathology. J Magn Reson Imaging 2014;39:1443-8. doi:10.1002/jmri.24317.

- Rouviere O, Hartman RP, Lyonnet D. Prostate MR imaging at high-field strength: evolution or revolution? Eur Radiol 2006;16:276–84.

- Haider MA, Krieger A, Elliott C, Da Rosa MR, Milot L. Prostate imaging: evaluation of a reusable two-channel endorectal receiver coil for MR imaging at 1.5 T. Radiology 2014;270:556–565.

- Heijmink SW, Futterer JJ, Hambrock T, et al. Prostate cancer: body- array versus endorectal coil MR imaging at 3 T—comparison of image quality, localization, and staging performance. Radiology 2007;244:184–195.

- Futterer JJ, Engelbrecht MR, Jager GJ, et al. Prostate cancer: comparison of local staging accuracy of pelvic phased-array coil alone versus integrated endorectal-pelvic phased-array coils. Local staging accuracy of prostate cancer using endorectal coil MR imaging. Eur Radiol 2007;17:1055–1065.

- Shah ZK, Elias SN, Abaza R, et al. Performance comparison of 1.5-T endorectal coil MRI with 3.0-T nonendorectal coil MRI in patients with prostate cancer. Acad Radiol 2015;22:467–474.

- Park BK, Kim B, Kim CK, Lee HM, Kwon GY. Comparison of phased- array 3.0-T and endorectal 1.5-T magnetic resonance imaging in the evaluation of local staging accuracy for prostate cancer. J Comput Assist Tomogr 2007;31:534–538.

- Beyersdorff D, Taymoorian K, Knosel T, et al. MRI of prostate cancer at 1.5 and 3.0 T: comparison of image quality in tumor detection and staging. AJR Am J Roentgenol 2005;185:1214–1220.

- Barentsz JO, Richenberg J, Clements R, et al. ESUR prostate MR guidelines 2012. European Radiology 2012;22(4):746-757. doi: 10.1007/s00330-011-2377-y.

- Purysko AS, Rosenkrantz AB. Technique of Multiparametric MR Imaging of the Prostate. Radiol Clin North Am 2018;56:211-222. doi: 10.1016/j.rcl.2017.10.004.

- Brizmohun Appayya M, Adshead J, Ahmed HU, et al. National implementation of multi-parametric magnetic resonance imaging for prostate cancer detection - recommendations from a UK consensus meeting. BJU Int 2018;122:13-25. doi: 10.1111/bju.14361.

- Chen LH, Ho H, Lazaro R, et al. Optimum slicing of radical prostatectomy specimens for correlation between histopathology and medical images. Int J Comput Assist Radiol Surg 2010;5:471-87. doi: 10.1007/s11548-010-0405-z.

- Puech P, Ouzzane A, Gaillard V, et al. Multiparametric MRI-Targeted TRUS Prostate Biopsies Using Visual Registration. BioMed Research International 2014;2014:819360. doi: 10.1155/2014/819360.

- Greer MD, Shih JH, Barrett T, et al. All over the map: An interobserver agreement study of tumor location based on the PI-RADSv2 sector map. J Magn Reson Imaging 2018;48:482-490. doi: 10.1002/jmri.25948.

- Rosenkrantz AB, Verma S, Choyke P, et al. Prostate magnetic resonance imaging and magnetic resonance imaging targeted biopsy in patients with a prior negative biopsy: a consensus statement by AUA and SAR. J Urol 2016;196:1613–8.

- Hoeks CM, Barentsz JO, Hambrock T, et al. Prostate cancer: multiparametric MR imaging for detection, localization, and staging. Radiology 2011;261:46–66.

- Bjurlin MA, Taneja SS. Standards for prostate biopsy. Current opinion in urology. 2014;24:155-161. doi:10.1097/MOU.0000000000000031.

- Loeb S, Vellekoop A, Ahmed HU, et al. Systematic review of complications of prostate biopsy. Eur Urol 2013;64:876-92. doi: 10.1016/j.eururo.2013.05.049.

- White S, Hricak H, Forstner R, et al. Prostate cancer: effect of postbiopsy hemorrhage on interpretation of MR images. Radiology 1995;195:385e90.

- Costello LC, Franklin RB. The Intermediary Metabolism of the Prostate: A Key to Understanding the Pathogenesis and Progression of Prostate Malignancy. Oncology 2000;59:269-282.

- Janssen MJ, Huijgens PC, Bouman AA, Oe PL, Donker AJ, van der Meulen J. Citrate versus heparin anticoagulation in chronic haemodialysis patients. Nephrol Dial Trans- plant 1993;8:1228–1233.

- Tamada T, Sone T, Jo Y, et al. Prostate cancer: relationships between postbiopsy hemorrhage and tumor detectability at MR diagnosis. Radiology 2008;248:531–9.

- Zakian KL, Shukla-Dave A, Ackerstaff E, Hricak H, Koutcher JA. 1H magnetic resonance spectroscopy of prostate cancer: biomarkers for tumor characterization. Cancer Biomark 2008;4:263–276.

- Barrett T, Vargas HA, Akin O, Goldman DA, Hricak H. Value of the Hemorrhage Exclusion Sign on T1-weighted Prostate MR Images for the Detection of Prostate Cancer. Radiology 2012;263:751-757. doi:10.1148/radiol.12112100.

- Rosenkrantz AB, Kopec M, Kong X, Melamed J, Dakwar G, Babb JS Taouli B. Prostate cancer vs. post-biopsy hemorrhage: diagnosis with T2- and diffusion-weighted imaging. J Magn Reson Imaging 2010;31:1387-94. doi: 10.1002/jmri.22172.

- Latifoltojar A, Dikaios N, Ridout A, et al. Evolution of multi-parametric MRI quantitative para- meters following transrectal ultrasound-guided biopsy of the prostate. Prostate Cancer Prostatic Dis 2015;18:343–51.

- Mirowitz SA. Seminal vesicles: biopsy-related hemorrhage simulating tumor invasion at endorectal MR imaging. Radiology 1992;185:373–376.

- Kim CK, Choi D, Park BK, Kwon GY, Lim HK. Diffusion-weighted MR imaging for the evaluation of seminal vesicle invasion in prostate cancer: initial results. J Magn Reson Imaging 2008;28:963-9. doi: 10.1002/jmri.21531.

- Rosenkrantz AB, Mussi TC, Hindman N, et al. Impact of delay after biopsy and post-biopsy haemorrhage on prostate cancer tumour detection using multi-parametric MRI: a multi-reader study. Clin Radiol 2012;67:e83-90. doi: 10.1016/j.crad.2012.08.014.

- Park KK, Lee SH, Lim BJ, Kim JH, Chung BH. The effects of the period between biopsy and diffusion-weighted magnetic resonance imaging on cancer staging in localized prostate cancer. BJU Int 2010;106:1148-51. doi: 10.1111/j.1464-410X.2010.09287.x.

- Sharif-Afshar AR, Feng T, Koopman S, et al. Impact of post prostate biopsy hemorrhage on multiparametric magnetic resonance imaging. Can J Urol 2015;22:7698–702.

- Bloch BN, Lenkinski RE, Rofsky NM. The role of magnetic resonance imaging (MRI) in prostate cancer imaging and staging at 1.5 and 3 Tesla: the Beth Israel Deaconess Medical Center (BIDMC) approach. Cancer Biomark 2008;4:251–262

- Sankineni S, Osman M, Choyke PL. Functional MRI in prostate cancer detection. Biomed Res Int 2014:590638

- Medved M, Sammet S, Yousuf A, Oto A. MR imaging of the prostate and adjacent anatomic structures before, during, and after ejaculation: qualitative and quantitative evaluation. Radiology 2014;271: 452–460

- Shin T, Kaji Y, Shukuya T, Nozaki M, Soh S, Okada H. Significant changes of T2 value in the peripheral zone and seminal vesicles after ejaculation. Eur Radiol 2018;28:1009-1015. doi: 10.1007/s00330-017-5077-4.

- Barrett T, Tanner J, Gill AB, Slough RA, Wason J, Gallagher FA. The longitudinal effect of ejaculation on seminal vesicle fluid volume and whole-prostate ADC as measured on prostate MRI. Eur Radiol 2017;27:5236-5243. doi: 10.1007/s00330-017-4905-x.

- Kabakus IM, Borofsky S, Mertan FV et al. Does abstinence from ejaculation before prostate MRI improve evaluation of the seminal vesicles? AJR Am J Roentgenol 2016;207:1–5

- Roethke MC, Kuru TH, Radbruch A, Hadaschik B, Schlemmer HP. Prostate magnetic resonance imaging at 3 Tesla: Is administration of hyoscine-N-butyl-bromide mandatory? World J Radiol 2013;5:259-63

- Wagner M, Rief M, Busch J, et al. Effect of butylscopolamine on image quality in MRI of the prostate. Clin Radiol 2010;65:460-4

- Dyde R, Chapman AH, Gale R, Mackintosh A, Tolan DJ. Precautions to be taken by radiologists and radiographers when prescribing hyoscine-N-butylbromide. Clin Radiol 2008;63:739-43

- Buscopan. Prescribing information and patient medication information. Data on file, Boehringer Ingelheim GmbH 2016.

- Slough RA, Caglic I, Hansen NL, Patterson AJ, Barrett T. Effect of hyoscine butylbromide on prostate multiparametric MRI anatomical and functional image quality. Clin Radiol 2018;73:216.e9-216.e14. doi: 10.1016/j.crad.2017.07.013.

- Ullrich T, Quentin M, Schmaltz AK, et al. Hyoscine butylbromide significantly decreases motion artefacts and allows better delineation of anatomic structures in mp-MRI of the prostate. Eur Radiol 2018;28:17-23. doi:10.1007/s00330-017-4940-7.

- Johnson W, Taylor MB, Carrington BM, Bonington SC, Swindell R. The value of hyoscine butylbromide in pelvic MRI. Clin Radiol 2007;62:1087-93

- Kier R, Wain S, Troiano R. Fast Spin-Echo MR Images of the Pelvis Obtained with a Phased- Array Coil: Value in Localizing and Staging Prostatic Carcinoma. AJR Am J Roentgenol 1993;161:601-6

- Gutzeit A, Binkert CA, Koh DM, et al. Evaluation of the anti-peristaltic effect of glucagon and hyoscine on the small bowel: comparison of intravenous and intramuscular drug administration. Eur Radiol 2012;22:1186-94

- Tygat GN. Hyoscine butylbromide - a review on its parenteral use in acute abdominal spasm and as an aid in abdominal diagnostic and therapeutic procedures. Curr Med Res Opin 2007;24:3159-73

- Tolan DJ, Greenhalgh R, Zealley IA, Halligan S, Taylor SA. MR enterographic manifestations of small bowel Crohn disease. Radiographics 2010;30:367-84

- Caglic I, Hansen NL, Slough RA, et al. Evaluating the effect of rectal distension on prostate multiparametric MRI image quality. Eur J Radiol 2017;90: 174-180.

- Soher BJ, Dale BM, Merkle EM. A review of MR physics: 3T versus 1.5T, Magn Reson Imaging Clin N Am 2007;15:277–290. http://dx.doi.org/10. 1016/j.mric.2007.06.002.

- Dietrich O, Reiser MF, Schoenberg SO, Artifacts in 3-T MRI: Physical background and reduction strategies, Eur J Radiol 2008;65:29–35. 10.1016/j.ejrad.2007.11.005.

- Huang SY, Seethamraju RT, Patel P, Hahn PF, Kirsch JE, Guimaraes AR. Body MR imaging: artifacts, k-Space, and solutions, Radiographics 2015;35:1439–1460. doi: 10.1148/rg.2015140289.

- McNeal JE, Redwine EA, Freiha FS, Stamey TA. Zonal distribution of prostatic adenocarcinoma. Correlation with histologic pattern and direction of spread. Am J Surg Pathol 1988;12:897–906

- Padhani AR, Khoo VS, Suckling J, Husband JE, Leach MO, Dearnaley DP. Evaluating the effect of rectal distension and rectal movement on prostate gland position using cine MRI. Int J Radiat Oncol Biol Phys 1999;44:525–533. doi: 10.1016/S0360-3016(99)00040-1.

- van Griethuysen JJM, Bus EM, Hauptmann M, et al. Gas-induced susceptibility artefacts on diffusion-weighted MRI of the rectum at 1.5 T - Effect of applying a micro-enema to improve image quality. Eur J Radiol 2018;99:131-137. doi: 10.1016/j.ejrad.2017.12.020.

- Lim C, Quon J, McInnes M, Shabana WM, El-Khodary M, Schieda N. Does a cleansing enema improve image quality of 3T surface coil multiparametric prostate MRI? J Magn Reson Imaging 2015;42:689–697. doi: 0.1002/jmri.24833.

- McNair HA, Wedlake L, Lips IM, Andreyev J, Van Vulpen M, Dearnaley D. A systematic review: effectiveness of rectal emptying preparation in prostate cancer patients. Pract Radiat Oncol 2014;4:437–447. doi: 10. 1016/j.prro.2014.06.005.

- Bayles H, Collins M, Clarkson M. How can the aetiological factors of rectal distension be managed to reduce interfraction prostate motion during a course of radiotherapy treatment. J Radiother Pract 2016;15:76–84. doi: 10.1017/S1460396915000436.

- Korn N, Kurhanewicz J, Banerjee S, Starobinets O, Saritas E, Noworolski S, Reduced-FOV excitation decreases susceptibility artifact in diffusion-weighted MRI with endorectal coil for prostate cancer detection. Magn Reson Imaging 2015;33:56–62. doi: 10.1016/j.mri.2014. 08.040.

- Rosenkrantz AB, Chandarana H, Pfeuffer J, et al. Zoomed echo-planar imaging using parallel transmission: impact on image quality of diffusion-weighted imaging of the prostate at 3T. Abdom Imaging 2015;40:120–126. doi: 10.1007/s00261-014- 0181-2.

- Thian YL, Xie W, Porter DA, Weileng Ang B. Readout-segmented echo-planar imaging for diffusion-weighted imaging in the pelvis at 3T-A feasibility study. Acad Radiol 2014;21:531–537.

- Holland D, Kuperman JM, Dale AM, Efficient correction of inhomogeneous static magnetic field-induced distortion in Echo Planar Imaging. Neuroimage 2010,50:175–183. doi: 10.1016/j.neuroimage.2009.11.044.

- Rakow-Penner RA, White NS, Margolis DJA, et al. Prostate diffusion imaging with distortion correction Magn Reson Imaging 2015;33:1178–1181 doi: 10.1016/j.mri. 2015.07.006.

- Olsen RV, Munk PL, Lee MJ, et al. Metal artifact reduction sequence: early clinical applications. RadioGraphics 2000;20:699–712.

- Shan L, Shan B, Graham D, Saxena A. Total hip replacement: a systematic review and meta-analysis on mid-term quality of life. Osteoarthritis Cartilage. 2014 Mar;22(3):389-406. doi: 10.1016/j.joca.2013.12.006.

- Kurtz S, Ong K, Lau E, Mowat F, Halpern M. Projections of primary and revision hip and knee arthroplasty in the United States from 2005 to 2030. J Bone Joint Surg Am 2007;89(4):780–785.

- Rosenkrantz AB, Taneja SS. Use of Reduced Field-of-View Acquisition to Improve Prostate Cancer Visualization on Diffusion-Weighted Magnetic Resonance Imaging in the Presence of Hip Implants: Report of 2 Cases. Curr Probl Diagn Radiol 2018;47:125-127. doi: 10.1067/j.cpradiol.2017.03.004.

- Czarniecki M, Caglic I, Grist JT, et al. Role of PROPELLER-DWI of the prostate in reducing distortion and artefact from total hip replacement metalwork. Eur J Radiol 2018;102:213-219. doi: 10.1016/j.ejrad.2018.03.021.

Table 1. PI-RADS v.2 recommended MR Imaging protocols

| Imaging Sequence | Technical Parameters |
| --- | --- |
| T2W Imaging | Fast-spin-echo (FSE) or Turbo-spin-echo (TSE) imaging Multiplanar imaging (axial, coronal and sagittal) |
|  | FOV: 12-20 cm to encompass the entire prostate gland and seminal vesicles |
|  | Section thickens, gap: 3 mm, 0 mm |
|  | In-plane resolution: ≤0.7 mm (phase) x ≤0.4 mm (frequency) |
| DW Imaging | Axial plane (same locations as for T2WI) |
|  | Free-breathing spin echo EPI sequence combined with spectral fat saturation is recommended |
|  | Section thickness: 3 mm, 0 mm |
|  | TE: ≤90 ms; TR: >3000 ms |
|  | FOV: 16-22 cm |
|  | In plane dimension: ≤2.5 mm phase and frequency |
|  | At least two b-values should be acquired in three orthogonal directions |
|  | ADC map calculation: low b-value to be set at 50-100 s/mm2, high b-value should be >800 s/mm2, up to a maximum of 1000 s/mm2. Additional b-values may provide more accurate calculations. |
|  | “High b-value”: if adequate SNR permits, b-values of 1400 – 2000 sec/mm2 or higher are preferred; it can be acquired by scanning or calculated |
| DCE | Axial plane (same locations as for T2WI) |
|  | Fat suppression and/or subtraction is recommended |
|  | 2D or preferably 3D T1 gradient echo (GRE) sequence |
|  | Section thickness, gap: 3 mm, 0 mm |
|  | Dose: 0.1 mmol/kg Injection rate: 2-3 ml/s |
|  | TR/TE: <100 ms/ <5 ms |
|  | In-plane dimension: ≤2mm X ≤2mm |
|  | Temporal resolution: <10 s (<7 s is preferred) |
|  | Total observation: >2min |

FOV, field of view; EPI, echo planar imaging; ADC, apparent diffusion coefficient; 2D, two- dimensional; 3D, three-dimensional; TR, repetition time, TE, echo time; T2WI, T2-weighted imaging; GRE, gradient echo.

FIGURES

Figure 1. ERC incorrectly sited
A: T2 axial image with ER coil sites; dotted line highlights coil plane. B: Flare from the coil elements in the right mid PZ on T1-weighted imaging pre-contrast may mask enhancement or be mistaken for early enhancement if not appreciated.

Figure 2. Endorectal coil distortion effect
Top Row: 1.5T endorectal coil imaging. A: T2WI shows a focal area of low T2 signal in the right mid peripheral zone. B, C: No restricted diffusion on b-value imaging (B) or ADC maps (C), the dominant PZ sequences. Bottom row: same patient with repeat imaging at 3T with a pelvic phased-array coil. Right mid gland peripheral zone lesion is well demonstrated on T2WI (A) and with marked restricted diffusion on b-value DWI (B) and additional early enhancement at DCE (C). Lesion confirmed as Gleason grade 4+3 on targeted biopsy

Figure 3. Figure 3. Representation of potential axial imaging planes
Lines represent the orthogonal plane from which images will be acquired. Solid line indicates the true axial plane to the patient as recommended in current UK consensus documentation [19]. Dashed line is oriented orthogonal to the rectum and posterior aspect of the prostate. Dotted line indicates a plane axial to the long axis of the prostate.

Figure 4. Potential mismatch between true axial and axial-to-prostate acquisitions.
Prostate base, mid, and apex sectors as defined by true axial plane (solid lines) and the plane orthogonal to the rectum and posterior prostate (dashed lines). The difference in angle will determine the degree of mismatch (highlighted pink) compared to matching areas (orange).

Figure 5. Haemorrhage exclusion sign 
TRUS biopsy demonstrates a Gleason 4+4 lesion in left mid PZ. MRI performed post biopsy shows haemorrhage in the right PZ on T1W (A) with ill-defined matching low T2 signal (B). Tumour in the left mid PZ demonstrates more focal low T2 signal (A) and “excludes” haemorrhage, which surrounds and outlines the lesion on T1WI (A).

Figure 6. Haemorrhage artefact and resolution 
Left column baseline staging MRI for TRUB biopsy proven low volume Gleason 3+3 disease. Haemorrhage in the right PZ appears as high signal on T1 (top) and mimics tumour on T2WI (bottom), appearing as a “circumscribed homogeneous hypointense focal mass”. There was no corresponding restriction diffusion on DWI (not shown). Right column: Follow-up AS MRI at 2 years shows resolution of the haemorrhage and disappearance of the T2 change at the same level.

Figure 7. Haemorrhage in the seminal vesicles 
Haemorrhage in the left seminal vesicle mimicking T3b disease with low T2 signal (A) but high signal on T1W with fat suppression (B).

Figure 8. Effect of abstinence from ejaculation

A: patient with collapsed SV (volume 3.8 ml) imaged within 1 day of ejaculation. B: Patient with over-distended SV (volume = 34.9 ml) imaged with a minimum of 3 days abstinence from ejaculation.

Figure 9. Effect of abstinence from ejaculation

Images from a 37-year old normal volunteer taken at baseline, following 4 days abstinence (A), day 1 post ejaculation (B) and day 3 post ejaculation (C). Volume reduces from 10.4 ml to 8.6 ml at day 1, then increase to 9.8 ml at Day 3.

Figure 10. Effect of increased rectal loading

64-year old man with a of PSA 4.11 ng/ml, undergoing MRI pre-biopsy. A: T2 axial image shows marked rectal loading. B/C: Diffusion weighted imaging shows warping on b-1400 images (B) and ADC map (C), particularly effecting the posterior gland.

Figure 11. THR artefact on T2WI 
Same patient scanned in the same session at 3T (A) then moved to 1.5T (B). No clinical detail stating the patient had bilateral THRs prior to scheduling.

Figure 12. THR artefact on DWI improved with PROPELLER 
71-year old patient with left THR. 1.5T scanner. A: T2WI, B: ADC map from EPI-DWI shows warping and distortion. C: ADC map from PROPELLER-DWI has little artefact or distortion (note streak artefact from THR in the top right-hand side of the image).