Integrated approach for optimizing quality control in international manufacturing networks

Tobias Arndta,*, Mukesh Kumarb, Gisela Lanzaa, Manoj Kumar Tiwaric

a wbk Institute of Production Science, Karlsruhe Institute of Technology (KIT), Karlsruhe, Germany; b Department of Engineering, Institute for Manufacturing, University of Cambridge, Cambridge, UK; c Department of Industrial and Systems Engineering, Indian Institute of Technology Kharagpur, Kharagpur, India

* Corresponding author: tobias.arndt@kit.edu

Integrated approach for optimizing quality control in international manufacturing networks

This paper aims at providing an integrated approach for optimizing quality control in International Manufacturing Networks (IMN), which can be characterized by consisting of numerous plants acting autonomously according to an individual target system. A key challenge is to ensure the overall process quality despite distributed value creation processes and differing target systems in dynamic environments. Hence, the developed approach allows for identifying potentials in the quality control strategy by using a value stream-based method to visualize quality characteristics and procedures in the production process chain. Furthermore, the approach contains a framework for identifying possible improvement measures and a simulation-based evaluation concept to evaluate the effects of different measures with respect to individual target systems. The simulation combines elements of a discrete-event simulation in order to depict the value stream with agent-based modelling for realizing different target systems by considering distinctive plant roles. The paper concludes with a case study of a globally operating automotive supplier to apply the approach. A forward research agenda is proposed that evaluates the approach in multiple cases, deriving patters across companies or industries.

Keywords: quality control, international manufacturing network, plant roles, simulation-based evaluation

# 1 Introduction

Over the last decades, globalization trends have drastically transformed the industrial landscape. Companies have changed from providing home markets, via exporting products to international markets, to providing products worldwide by a global network of manufacturing sites. This was account for by growing global competition and rapid globalization of sales and procurement markets. The drastic increase in international trade and foreign direct investment (FDI) within the past decades (Cheng, Farooq and Johansen 2015) also underlines the significance of an organized network of dispersed plants, an international manufacturing network (IMN).

The overall challenge for companies is to configure and coordinate the manufacturing network in a dynamic environment, because its efficiency and effectiveness determines the success of the company. Therefore, individual plants increasingly focus on their core competencies and specialize in manufactured products, the supplying market or the manufacturing processes carried out (Friedli, Mundt and Thomas 2014). That way, they are able to better perform in the interest of the company irrespective of low-cost labour or tariff reduction manufacturing models (Ferdows 1997; Feldmann and Olhager 2013). But along with this, a space of action or target system is defined, according to which each plant autonomously defines individual optimization measures. This leads to mutual interdependencies involving goal conflicts and asymmetric information distribution among them (Tapiero and Kogan 2007).

Especially the assurance of quality in an IMN implicates a special challenge. Despite different plants being involved in the manufacturing process, the required quality of the final product must be ensured by high process quality control strategies at minimal costs (Jochem and Menrath 2015). This requires an integrated examination of quality, production and logistic processes across on all levels of the production network (Colledani et al. 2014). Currently, these strategies focus on quality control on plant level, without adequately regarding the specific plant roles, their interdependencies and their impact on the manufacturing network (Lanza and Arndt 2016).

Therefore, improvement potentials remain undetected. Asymmetric information distribution may lead to redundant tests at different locations or long time intervals between the cause of failure in the production process and the identification of this failure (Belleflamme and Peitz 2014). This means there is value added to defective intermediate products and an increasing transport of scrap (Wankhade and Dabade 2006). Furthermore, an improved inspection planning by higher utilization of expensive measuring equipment at other plants is often not realized. Both result in significant inefficiencies in the manufacturing network (Erasmus and Wulfsberg 2007).

On the other hand, the efficiency of quality related improvement measures cannot be sufficiently evaluated. The main reason is that the quality of interlinked and interdependent cross-site value creation processes can only be partially described and evaluated based on specific key figures. This is especially important in the automotive industry (Yüzgülec, Ullerich and Siemes 2015). In addition, an evaluation of quality related improvement measures always requires a situation-specific identification of the characteristics of the respective manufacturing network (Zu and Kaynak 2012), which are exposed to constantly changing influencing factors. Otherwise, specific measures may not be carried out after a process-related intra-site cost / benefit analysis, even though they would have a positive effect on all other partners in the manufacturing network and therefore improve the holistic quality. This imposes a significant challenge under the described asymmetric information distribution and divergent target systems.

For analysing processes and identifying improvement potentials Value Stream Mapping (VSM) is a widely accepted approach (Nyhuis, Nickel and Tullius 2008; Spalt, Braun and Schöllhammer 2013). Nevertheless, the analysis of quality strategies across multiple locations has not been taken into consideration, despite several extensions of the original concept. Therefore, this paper addresses the following research questions:

How can weaknesses and improvement potentials in the quality control strategy of an IMN be systematically identified on the basis of the VSM concept?

How can quality related improvement measures be dynamically evaluated with respect to divergent target systems?

The main objective of the paper is to present an integrated approach, considering production, inspection and transportation processes in the context of divergent target systems in IMN to optimize the overall process quality. The paper is structured as follows: In the next section, the literature related to quality control in IMN is examined and the scope and methodologies of the approach are derived. Afterwards, the developed approach is described. In section four, the approach is evaluated in a case study of a globally operating automotive supplier and implications for research and industry are presented. The paper concludes with a summary and a research outlook.

# 2 Theoretical foundations

Based on the defined targets of the paper, in the following chapter theoretical foundations related to IMN, its characteristics regarding quality control as well as concepts for dynamically evaluating and improving process and network configurations are discussed. Furthermore, requirements regarding a methodology for evaluating and increasing process quality in IMN are derived, against which existing approaches are evaluated.

## 2.1 International Manufacturing Networks

An IMN is understood to be a network of interdependent plants of one organization with an array of connections, where each plant cannot be solitary managed, because it affects other factories (Shi and Gregory 1998). According to Rudberg and Olhager (2003) this structure has to be differentiated from the supply chain concept, which consists of multiple organizations with only one plant and the inter-firm network that contains multiple plants of multiple organizations. Feldmann, Olhager and Persson (2009) consider factories of only one organization in their definition, but additionally include their respective external suppliers. In this paper, the understanding of IMN according to Shi and Gregory (1998) will be followed.

As a result of the development of IMN, their management gained more importance (Shi and Gregory 1998). However, the tools for managing a single plant or production system cannot be applied for managing IMN. Instead, individual methods for analysis, (re-)design and the optimization of the whole network, embedded in a holistic management paradigm, are necessary (Friedli and Schuh 2012). For managing IMN, all important decisions can be divided in to the two organizational elements of configuration and coordination (Cheng, Farooq and Johansen 2015). By bringing these elements in accordance with each other, organizations could gain competitive advantages from manufacturing (Rudberg and Olhager 2003). This was also shown by Colotla, Shi and Gregory (2003), who analysed location and network capabilities in combination with the attainment of strategic goals.

The configuration layer describes the physical structure and design of a manufacturing network. It contains the geographic dispersion of the factories, associated with product, resource, technology, and capacity allocation (Meijboom and Vos 1997) as well as risks from the factory specific supply chain (Kumar, Srai and Gregory 2016; Kumar and Gregory 2013) and market access. Furthermore, it includes four types of network strategies: product-oriented (Product Plant Strategy), market-oriented (Market Area Plant Strategy), process-oriented (Process Plant Strategy) and flexibility-oriented (General Purpose Plant Strategy) (Schmenner 1982). In the presented approach it is assumed, that in the network a process-oriented strategy is followed. The reason for that is, that quality improvement measures focus on processes in the individual plant and are usually not variant or product specific or differ for individual markets.

In contrast to the network strategy studies, plant roles are used to define different strategic targets for each plant within the network with regard to specific criteria and can be regarded as the interface between the configuration and coordination layer. These targets define desired states that should occur as outcomes of implementing measures. They are often combined in a target system on plant level, which is hierarchical in nature and aligns sub-goals regarding defined performance targets e.g. in the dimensions of costs, time, quality, sustainability and flexibility with the overall plant goals. The sub-goals may be neutral, complementary or adversarial and can be operationalized using Key Performance Indicators (KPI) as compressed metrics. For differentiating individual target systems, different plant role concepts are described in literature (Weber 1995, Ferdows 1997, Vereecke and Van Dierdonck 2002, Vokurka and Davis 2004, Wiendahl, Reichardt and Nyhuis 2014).

According to Cheng, Farooq and Johansen (2015) the coordination layer of IMN is related to the coordination of practices, the transfer of production technologies and knowledge, and the optimization of physical distribution. The purpose of the coordination is to achieve common goals that cannot be achieved by individual actors (Kaipia 2007). The use of coordination instruments leads to reconciliation costs, whereas a waiver can result in cost autonomy.

The evaluation and improvement of process quality in the IMN is part of the configuration and coordination level. From the perspective of network configuration, the underlying network strategy is defined as process-oriented and the geographic distribution and capacity allocation is accepted as given and unchangeable in this context. The other dimensions are considered to be design dimensions of the process quality. From a coordination perspective quality control is essentially related to the coordination of practices as well as the transfer of technologies and knowledge. This includes relationships between the company's internal sites as well as with suppliers and customers.

Therefore, from this perspective two requirements regarding a methodology for evaluating and increasing process quality in IMN can be derived. First, to evaluate and increase the process quality in the defined scope, the individual target systems of all internal sites as well as all relationships with each other have to be taken into account. Second, the strategic dimensions of network configuration as well as tactical operational concepts of coordinating production networks are to be considered as design dimensions of the process quality.

## 2.2 Quality control in international manufacturing networks

In order to control the quality within the hereby defined manufacturing network and to ensure that the quality required by the customer can be maintained or even improved, current works mostly aim at providing methods for process-based problem solving at plant level.

Tague (2005) develops a pragmatic tool-matrix in order to select particularly suitable management methods for specific quality problems. The matrix contains 72 different tools categorized in three dimensions: target of using the tool, the position in the quality improvement process, and the question if a creative-extending or an analytic-focusing process is regarded. Johannsen and Leist (2009) evaluated 17 quality methods regarding usability from the user's perspective, functional suitability and functional flexibility. Thereby, some plausible and pragmatic criteria for the selection and use of quality methods as well as typical application scenarios are presented. Rau (2015) presents a standardized and systematic selection procedure for problem-solving methods and techniques. This includes a seven-step approach to choosing the most appropriate problem-solving method. The first six steps include a detailed description of the problem, while in the seventh step the solution approach is planned.

All the above mentioned approaches focus on providing specific methods for solving a specific quality related problem at plant level. Therefore, there is limited research investigating weaknesses in a holistic quality control strategy. Furthermore, it is not evident, how these single methods affect other plants and associated processes in the manufacturing network. Although all single methods also apply for quality control in IMN, a concept for selecting appropriate quality measures with respect to the specific characteristics of IMN is overlooked in research so far.

However, mechanisms on how to evaluate quality management strategies involving dispersed plants with multiple ownerships are provided from the research area of Supply Chain Quality Management. Robinson and Malhotra (2005) argue in their literature analysis that the work so far distinguishes between intra-organizational quality management and inter-organizational coordination in supply chains and illustrate the necessity to integrate these parts. Initial works regarding the definition of Supply Chain Quality Management come e.g. from Forster (2008) or Kaynak and Hartley (2008).

Current approaches in this research area focus on selecting the optimal quality control strategy, especially regarding the involvement of multiple suppliers. Erasmus (2008) focuses on the importance of knowledge exchange for coordinating supply chains from the quality point of view. It is stated that especially the improvement of communication structures between individual organisations in a supply chain can contribute to an economic and reliable fulfilment of the customer’s quality requirements (Erasmus 2008). However, there is limited empirical evidence, on how to realize this improvement in industry. Das and Sengupta (2010) set up a mathematical network model consisting of multiple suppliers, a central enterprise and several retailers. The goal of the model is to find a cost-optimal supply chain, while taking the capabilities of suppliers, e.g. quality rates, process skills and the quality management system, into account. It was shown that involving various competencies in coordination mechanisms can lead to an optimum. Liu and Hipel (2012) present a decision framework based on the concept of the Quality Function Deployment (QFD) methodology for selecting optimal quality control strategies for supply chains of complex products. However, it remains on an abstract level and is highly dependent on subjective assessments. Findings of Zu and Kaynak (2012) suggest, that rather than imposing one generic quality control strategy on all suppliers, companies need to choose different mechanisms for different suppliers based on the attributes of individual suppliers. Fuhrmann et al. (2016) developed a Java-based game to qualify today’s quality managers to cope with the arising challenges of Supply Chain Quality Management.

From current approaches of Supply Chain Quality Management frameworks for the evaluation of quality strategies involving different organizations can be derived. However, special characteristics of IMN including different target systems resulting from individual roles of individual plants are currently not discussed. Furthermore, these approaches disregard the process level, but only analyse whole plants as one object.

Overall it can be concluded that many approaches contribute to the understanding of process quality both at plant as well as supply chain level, but without providing uniform description dimensions or performance indicators. Furthermore, there is no systematic approach, which allows for analysing the process quality in a manufacturing network as well as deriving measures for improvement. Therefore, the developed integrated approach must contain defined performance indicators to describe the process quality in manufacturing networks, a procedure to analyse these indicators as well as systematized measures to improve the across-site quality control. Since quality control approaches do not allow for dynamically evaluating the effects of individual improvement measures for specific network configurations, related approaches are discussed in the following chapter.

## 2.3 Dynamic evaluation and optimization of process and network configurations

For evaluating production processes and deriving weaknesses in processes chains, which can also include several plants, VSM is a widely accepted procedure. Furthermore, it allows for establishing improvement processes in a structured approach (Rother and Shook 2003). While VSM was developed for production environments, it is increasingly applied in many non-production areas such as product development procedures (McManus 2005) or logistics processes and passenger handlings at airports (O’Connor 2014). Still, there are only very few approaches that consider quality aspects in VSM, even though an outstanding process quality is one of the most important goals of lean management. Haefner et al. (2014) developed an approach, which visualizes quality related quantities within the production line by integrating different symbols and performance indicators for quality related processes into the VSM. Moreover, weaknesses in the quality control strategy can be visualized. However, in spite of numerous extensions and transfers of the VSM concept to non-production areas, the analysis of quality strategies across multiple locations has not been taken into consideration (Arndt et al. 2016).

Despite all advantages of VSM, its biggest weakness is the absent of dynamic characteristics, considering only a selected point in time (Wolff 2013). Several approaches try to address these challenges by integrating dynamics with the aid of simulation concepts. McDonald, Van Aken and Rentes (2002) propose a simulative approach to display several parallel processes. The approach of Agyapong-Kodua, Ajaefobi and Weston (2009) provides the opportunity of evaluating fluctuating demands as well as varying the product mix. It can be concluded, that simulation-based VSM is a suitable method to dynamically evaluate weaknesses and define improvement measures in the process-oriented value chain. However, current approaches only analyse the processes on plant level and do not take quality related processes and measures into account.

According to Kaczmarek (2006), simulation is also of high importance for evaluating manufacturing networks before beginning with a cost-intensive implementation, with regard to different configurations or potential optimization measures. Especially agent-based simulation models have been widely used for the operational planning of manufacturing networks or supply chains, since they are particularly suitable for describing and evaluating systems, which consist of autonomous entities (Leitao 2009). In these models independent agents act, based on predefined rules, both with each other and with their environment. Typical characteristics for these models are an unbalanced information distribution as well as the absence of a central evaluative control instance. That means, that each agent has only limited problem-solving possibilities, with which it pursues its own goals and all actions are performed locally and asynchronously.

Several works in literature use agent-based simulation in order to dynamically evaluate different coordination and configuration mechanisms in manufacturing networks. Related to coordination mechanisms, Long (2014) proposes an integration of agent-based simulation and supply chain operations reference (SCOR) model, which allows for a rapid configuration of a manufacturing network in different hierarchical levels. Since target systems of individual units are not regarded, the advantage of agent-based simulation remains partially unused. On coordination mechanisms, Giannakis and Louis (2011) present a theoretical framework for an agent-based risk management approach in demand-oriented supply chains. In this approach information of suppliers such as their production progress or quality level is combined in KPI and compared with reference values to detect deviations from expected values. In case of deviations, the consequences are calculated by means of an integrated simulation environment and suitable action proposals are determined. However, it is not further specified, which particular measures can be used.

Overall, the presented approaches show, that simulation-based VSM is a widely accepted approach to identify weaknesses in the value chain on a process level under volatile influencing factors, whereas agent-based simulation is well suited to depict the characteristics of manufacturing networks. However, there are currently no approaches that use the advantages of simulation-based VSM involving an agent-based approach in order to analyse weaknesses in the quality control strategy and evaluate the effects of improvement measures with respect to individual target systems in IMN. Therefore, the developed integrated approach has to address and combine these to concepts in order to be able to evaluate different quality measures in accordance to individual target systems in an IMN.

## 2.4 Research deficit

Concluding the literature review, it has to be stated, that . All previously described approaches were evaluated according to six criteria. It was analysed, if individual target systems as well as the relationship between internal sites, suppliers and customers are sufficiently regarded. Furthermore, the consideration of manufacturing network coordination and configuration layer was analysed. Lastly, it was examined, if by the approaches the quality level in manufacturing networks can be described, analysed and improved considering dynamic influencing factors. The result of this evaluation is shown in Figure 1.In total it can be stated that quality control is not sufficiently regarded in coordination and configuration of IMN. Current approaches either focus on providing specific methods for solving a specific quality related problem at plant level or provide mechanisms for supply chains, disregarding the process level and specific characteristics of IMN like individual target systems. Simulation-based VSM involving agent-based modelling appears to be one of the concepts for the evaluation and optimization of process and network configurations, but are currently not used for quality control.

Figure 1. Research deficit

# 3 Development of a quality control approach in IMN

The identified research deficit serves as the basis for the developed approach, which is depicted in Figure 1. It consists of four steps. Based on the defined requirements in the field of IMN a hierarchical target system model is developed, which allows for evaluating the current level of process quality in the manufacturing network based on defined performance indicators, considering individual sites with their individual roles and resulting targets. By the extended quality value stream mapping, which is explained in the second step, it is possible to systematically record these performance indicators, evaluate the individual processes on this basis and thus gain an understanding of the influencing factors of the process quality both at the configuration and coordination level of the manufacturing network. Furthermore, the weaknesses in the company-specific quality control strategy can be visualized and improvement potentials derived.

Since the analysis of literature shows, that there is no systemic method to identify improvement measures regarding process quality in an IMN, in a third step a categorization method is presented, with which possible quality measures can be characterized with regard to their respective effects on the target system as well as the affected objects in the manufacturing network. In the last step, all concepts are integrated into a multi-method simulation model, combing the explained benefits of a discrete-event simulation and agent-based modelling. It enables individual production networks to be configured modularly at station and site level, taking into account site-specific target systems. With this, quality control in the manufacturing network can be optimized by dynamically evaluating the benefit of different discrete combinations of quality measures according to defined performance indicators depending on the individual target systems of different plants.

Figure 2. Framework for Optimization of Quality Control in IMN

## 3.1 Hierarchical Target System Model

As described, individual sites in IMN pursue different targets according to their defined role. In the developed approach, targets of an individual plant are distinguished in five target dimensions: (1) Costs, (2) Time, (3) Quality, (4) Sustainability and (5) Flexibility. These dimensions vary in the significance for a plant. In order to systematize individual target systems, existing approaches in literature regarding plant roles and typologies were analysed with regard to the differentiation of the defined target dimensions in order to derive a consistent model. Table 1 shows the final plant role model, which differentiates four basic dimensions, and integrates the classifications of Weber (1995), Vokurka and Davis (2004) as well as Wiendahl, Reichardt and Nyhuis (2014). Furthermore, the importance of the dimension of quality is described. Other approaches do not allow a differentiation according to the defined target dimensions.

The overall categorization is taken from the internationalization motives of globally operating companies according to Weber (1995), which differentiates four types of plants: resource-oriented, market-oriented and innovation-oriented as well as lead plants. In this work, lead plants are not considered an additional factory type, since resource-oriented, market-oriented and innovation-oriented locations can serve as lead plants if they additionally take over a coordination functions. However, this does not affect any of the target dimensions distinguished. Furthermore, process oriented plants according to the definition of Vokurka and Davis (2004) are regarded as fourth factory type. These plants can be characterized by a strong focus on their individual process knowledge and describe different targets regarding the dimensions costs and flexibility. Resource-oriented plants produce high lot sizes and a low number of variants, focusing on low production costs as well as production efficiency. Market-oriented plants aim at fulfilling regionally specific customer requirements, while innovation-oriented plants develop innovative products, manufactured as prototypes or in small series.

Table 1. Plant role concept

| Factory types | Plant role | Importance of Quality | Characteristics |
| --- | --- | --- | --- |
| Process-oriented factories | Standardizing Factory | Very high | High volumes with low product, material and customer diversity, requesting high process quality |
|  | Automating Factory | Very high | High volumes with high product diversity, possible because of high technological capacities and high process quality |
|  | Customizing Factory | High | Low volumes but wide of variants for many different types of customers, which requires high process flexibility |
| Resource-oriented factories | Resource-efficient Factory | Medium | Resource-efficiency demands reasonable process quality and low scrap rates. Sustainability is of very high importance. |
|  | Low-cost Factory | Very low | High-scrap rates are tolerated in favour of cost benefits, continuous cost reduction is the ultimate goal, so mature products are manufactured in high lot sizes and few variants |
| Market-oriented factories | Responsive Factory | Very low | Minimizing lead-times for example by high-performance logistics are of high importance, in order to ensure a rapid availability of products |
|  | Volume-flexible factory | Low | High degree of flexibility due to a low level of automation |
|  | Variant-flexible factory | Low | Focus on flexibility in order to supply a defined market with customized products |
| Innovation-oriented factories | High-tech factory | Very high | High Process quality is distinguished characteristic as well as innovative products and technologies |
|  | Innovative factory | Very high | High Process quality is required to manufacture innovative products |

In order to quantify the influence of the systematized factory type on the IMN, the significance of the target dimension for each type is evaluated on a scale from 1 (very low significance) to 5 (very high significance) on the basis of the literature used for deriving the plant role concept. Due to its similariy, the description of location types according to Schenk, Wirth & Müller (2014) is also used to support the assessment. The result is shown in Table 2.

Table 2. Significance of target dimensions for the individual factory types

|  | Low-Cost- Factory | Resource-efficient Factory | Responsive Factory | Volume flexible Factory | Variant-flexible Factory | High-Tech-Factory | Innovative Factory | Standardizing Factory | Customizing-Factory | Automating Factory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Quality | 1 | 2 | 2 | 3 | 2 | 5 | 4 | 4 | 4 | 5 |
| Cost | 5 | 4 | 1 | 3 | 2 | 1 | 1 | 4 | 3 | 4 |
| Time | 2 | 3 | 5 | 4 | 3 | 1 | 1 | 4 | 4 | 4 |
| Flexibility | 1 | 1 | 3 | 5 | 5 | 4 | 4 | 2 | 3 | 2 |
| Sustaina-bility | 1 | 5 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 |

## 3.2 Extended Quality Value Stream Mapping

In a second step, the process quality in the manufacturing network has to be analysed according to the defined target dimensions in order to derive improvement potentials. Therefore, in accordance to the VSM concept, at first all relevant production and transportation processes and their respective performance indicators are recorded for all participating plants and the transportation processes in between. Additionally, quality inspections, rework and scrapping processes are integrated in the analysis using the notation of Haefner et al. (2014). For quality inspections test duration, inspection characteristics identified failures and related costs are recorded. For rework processes, their respective costs and required times are recorded as well as scrapping costs and the value of the scrapped part for scrapping processes.

Subsequently, based on this analysis, quality inspections are linked to the production process, where the identified failures emerge. Each link is then characterized by an extended risk priority number (RPNext). According to the Failure Mode and Effects Analysis (FMEA), the original RPN is calculated by multiplying indicators for the severity of the failure (S), the probability of the occurrence of the failure (O) and the probability of detecting the failure (D). In addition, the RPNext contains indicators for the increased value of the product until the discovery of the failure (V) and the replacement time (R). All indicators are rated from 1 to 10.

RPNext = S * O * D * V* R				       (1)

Weaknesses in the quality control strategy can be visualized by colouring the links according to the value of RPNext. Based on a company individual scale this could e.g. be carried out in green (low value), yellow (medium value) or red (critical value). In addition, the amount of failures, the quality- related costs and the quality-related time are visualized by means of a failure curve, a quality cost curve and a quality-time curve. An example for a visualization of the Extended Quality Value Stream is shown in Figure 3.

Figure 3. Visualisation of Extended Quality Value Stream (Arndt et al. 2016)

## 3.3 Categorization of Quality Measures

Based on this analysis, quality improvement measures are defined as actions, which can improve the defined RPNext. They can be categorized according to five fields of action and three impact dimensions. Depending on the evaluation of the quality control loop, an improvement measure can increase the probability of detections or reduce the probability of failure occurrence, the replacement time, the increase of value or the quality related costs (fields of action). These measures can affect each object of the manufacturing network (impact dimensions): the production process (e.g. poka yoke, autonomation, visual management measures), quality related processes such as the inspection process itself, scrap and rework processes (e.g. implementation of an additional inspection station, increase of the sample size, outsourcing of inspection to a third-party provider) or a logistics process like storage or transportation processes (e.g. consignation storage, new packaging concept). Each possible combination of quality measures has an impact on the targets of each plant. A fully automated inspection device is e.g. more expensive, usually faster, more accurate but less flexible than a manual inspection.

The implementation of a measure results in an alternative configuration of the manufacturing network, keeping the system boundary unchanged. Each configuration is evaluated against the defined target system model. Since in each configuration the process quality is changing over time due to volatile influencing factors, a static evaluation of the process quality in each configuration is not productive. Therefore, in the step all defined concepts are integrated in a multi-method simulation model.

## 3.4 Multi-method Simulation Model

By this model it is on the one hand possible to identify more weaknesses in the quality control strategy than it is allowed by a static snapshot. Furthermore, the advantageousness of quality control measures for each site or the entire production network can be evaluated before the implementation while accounting for individual target systems.

The processes depicted in the Extended Quality Value Stream are modelled as individual process agents using elements of a discrete-event simulation such as queuing, combining, separating or delaying modules. Additionally, for each module individual actions are defined to take place on entry or on exit of a product. The resulting process agents can be used and combined as often as desired. These agents can also be integrated in other agent classes, which allows for a detailed differentiation of individual plants.

These are modelled as a combination of process agents, which are influenced by a defined target system. For each target dimension performance indicators are provided, for which user input is required in order to define a maximum and minimum value as well as to derive the utility of an indicator on a scale from 0 to 1. According to the user’s preference function, these indicators are aggregated to a utility value in the respective target dimensions. According to the defined plant role, the utility of each dimension is weighted by a defined importance of the respective target dimension, e.g., the utility of the cost dimension is more important in a low-cost factory than in a high-tech factory. When implementing a quality measure it is possible to evaluate its effect by comparing the utility values before and after the implementation.

# 4 Application of the approach in a case study

## 4.1 Description

In order to apply the developed approach, an industrial case study was conducted. Therefore, the network of a globally operating automotive supplier was examined consisting of five plants jointly producing tubes for different applications and supplying several OEM was examined. The case was selected for two different reasons: On the one hand, the regarded network contains three plants, which supply intermediate products to others resulting in a lack of transparency in the across-site quality control strategy. On the other hand, plants 3,4 and 5 partially conduct the same processes, yet serve different customer with individual quality requirements, which leads to differing target systems of the plants.

Plant 1 produces steel strips as the required raw material for the final tubes. In Plant 2 the steel strip is rolled and braced to a continuous tube (Process 1), which is coated in Process 2 and winded on a coil. These coils are transported to Plants 3 to 5. In the plants the coils are cut (Process 3), assembled with secondary products (Process 4) and finally bent in the shape required by the individual customer (Process 5). The bending process is only performed at Plants 4 and 5 or can even be done by the customer, so not all products need to go through Process 5 before being sent to the customer. In the following all steps of the presented approach are described along exemplary results of the case study.

Figure 4. Evaluated network

## 4.2 Configuration of the hierarchical target system

In order to configure the hierarchical target system for the production network at each plant a target workshop was conducted with plant managers, process supervisors, quality managers as well as production and logistic planners. By taking into account the different perspectives of the participating functions while defining the individual plant role, a distortion of the evaluation could be minimised. As a result, plant 1 strives to ensure the shortest possible delivery times to its internal customer (plant 2) and therefore takes on the role of a responsive factory. Plant 2 is regarded as a high-tech location, as it can be differentiated from others especially by the innovative process technology used. Plant 3 is based on the typology of a volume-flexible plant, because here especially the fluctuations in demand in customers are balanced. Plant 4 and 5 act as low-cost locations and aim to realize wage cost advantages during assembly and bending processes.

## 4.3 Analysing weaknesses in the current quality control strategy by Extended Quality Value Stream Mapping

In order to analyse the current quality control strategy according to the described methodology, the value stream was recorded and extended by quality related processes. The necessary data was taken from the used production planning software. In case of missing or conflicting data, already conducted process analyses were additionally evaluated and interviews with operator or time measurements in the plants conducted. Failure curve, quality cost curve and a quality-time curve were recorded in order to evaluate the current quality control strategy in these three dimensions.

The analysis in the dimension of time shows that quality related times were relatively low compared to production and transportation times, so here was no need for improvement. The evaluation of quality related costs is shown in Figure 5. It can be seen that for Plant 2, the share of failure costs in relation to the overall quality costs is very low, while at the other plants the ratio of failure costs and inspection costs are nearly balanced. From this perspective, it could be concluded that there is no need for quality improvements at Plant 2, but potential quality measures have to focus on all other plants.

Figure 5. Analysis of quality costs

The evaluation of failures takes place in two dimensions. At first the failure quantities are analysed according to the defects per million opportunities (DPMO) as well as the scrap percentage for each process. The results are shown in Table 3. It can be seen that most failures are determined after process 3. From this analysis the conclusion could be drawn, that improvement measures should be focusing on process 3 or the quality control after the process respectively. It is also noticeable, that the DPMO after process 1 are significant, but not leading to a large amount of scrap.

Table 3. Evaluation of failure quantities

| Plant | Plant 2 |  | Plant 3 |  | Plant 4 |  |  | Plant 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection after process | 1 | 2 | 3 | 4 | 3 | 4 | 5 | 3 | 4 | 5 |
| DPMO | 3789 | 704 | 4351 | 1666 | 3239 | 1280 | 338 | 6347 | 1202 | 176 |
| Percentage Scrap [%] | 0,38 | 0,21 | 3,05 | 1,06 | 2,27 | 0,9 | 0,2 | 2,54 | 0,6 | 0,07 |

Secondly, the individual types of failures are analysed. The evaluation shows, that out of the distinguished 71 failures, most of the recorded scrap results from three failure types:

Failure 8 - brazing failures while processing steal to tubes,

Failure 24 - coating failures

Failure 30 - corrugated polyamide coating

Failure 24 alone leads to 53% of the overall scrap in the manufacturing network. Failure 8 accounts for 17%, Failure 30 for 9%. The analysis suggests that the focus should be on reducing these 3 failures.

However, by connecting the production processes with the inspection by quality control loops, and subsequently evaluating the respective RPNext (see Table 4) it becomes evident, that the low failure costs at Plant 2 result from the low value of the scrapped products and from the fact, that large quantities of defective pipes are identified in other plants leading to higher costs there. Furthermore it is underlined, that Failure 24 should have the most focus on, since it causes the highest value, due to its high severity and occurrence probability. So instead of focusing on failure 8, which admittedly accounts for 17% of the total scrap in the manufacturing network, it is necessary to put attention on Failure 20. While Failure 8 is most probably detected shortly after its incurrence, Failure 20 leads to an increased RPNext at three different quality inspections. This is attributable to a general low detection probability and a high value added between process 2 and the inspection stations at Plants 4 and 5. Therefore, according to this analysis for deriving improvement measures, the focus was put on Failure 24, since it has the highest RPNext as well as Failure 20 because it is detected at various stations downstream.

Table 4. RPNext of evaluated quality control loops

| Failure type |  | Failure detec-tion | During Process 2 (Plant 2) | After Process 2 (Plant 2) | After Process 3 (Plant 3) | After Process 3 (Plant 4) | After Process 3 (Plant 5) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Failure emer-gence |  |  |  |  |  |  |
| Failure 8 | Process 1 |  | 64 | 1 | 1 | 1 | 1 |
| Failure 18 | Process 2 |  | 0 | 10 | 1 | 1 | 1 |
| Failure 19 | Process 2 |  | 0 | 64 | 24 | 24 | 8 |
| Failure 20 | Process 2 |  | 0 | 72 | 56 | 64 | 8 |
| Failure 24 | Process 2 |  | 0 | 96 | 16 | 24 | 8 |
| Failure 30 | Process 2 |  | 0 | 32 | 12 | 12 | 1 |

## 4.4 Selection of quality measures

Two types of measures can be implemented to improve the quality control strategy: On the one hand, the coating process could be improved, resulting on fewer failures. On the other hand, the quality inspection after process 2 could be improved in order to recognize more failures at an early stage. An evaluation of the RPNext clearly suggests that it should be focused on the latter, since the value for the occurrence probability is significantly lower than the one for the detection probability. Therefore, the effects of a potential implementation of an additional camera after Process 2 are analysed. Process experts assumed, that this way the detection probability of all failures could be increased by 50%, or at least amount 10%. On the other hand this implementation leads to higher inspection costs at Plant 2.

Table 5. Effects of implementing quality control measure

| Failure type | Probability of detection |  |
| --- | --- | --- |
|  | Before implementation | After implementation |
| Failure 18 | 14% | 28% |
| Failure 19 | 27% | 54% |
| Failure 20 | 15% | 30% |
| Failure 24 | 2% | 10% |
| Failure 30 | 7% | 14% |

## 4.5 Implementation of multi-method simulation model

To implement the multi-method simulation model, Java-based simulation software AnyLogic® is used as a suitable platform. This simulation model was implemented in AnyLogic ® University 7.3.4 and tested in the Windows 7 Enterprise (64-bit) operating system. To perform the simulation experiments, an Intel Core i7-3770 processor with 3.4 GHz and 16GB of memory was used.

Using the defined process agents, the manufacturing network was modelled and the required input data inserted based on the extended quality value stream analysis. After each simulation run, the performance indicators were recorded, aggregated to a utility value in the respective target dimensions and weighted according to the significance of the target dimension for the respective plant. The quality control measures are directly linked to defined parameters of the model. If a measure is implemented, the corresponding parameters are selected directly and can be changed in accordance to the effects anticipated by the user. An absolute or relative change can be differentiated. That means that for the occurrence probability for example it can be defined, if the target parameter is to be changed by the specified percentage value or adjusted to the stated probability value. This is possible for each failure individually. In addition, one-time implementation expenses of the respective measure are also taken into account.

## 4.6 Optimization of quality control in IMN

The results of the simulation as a dynamic quality value stream showed, that the implementation of an additional camera after process 2 leads to increased detection probabilities for the defined failures and hence to an increase of scrap in Plant 2 by 43.5%. It was calculated by the amount of products, which contain a type of failure, arising in dependence of the defined occurrence probability, which cannot be reworked. In contrast to Plant 2 the scrap decreased by respectively 4.4%, 12.1% and 31.8%, in Plants 3, 4, and 5. The total amount of scrap in the network thus rose by 1.9%, as more defective products are now discovered before delivery to the customer, which is accompanied by a reduction in the external failure costs.

For Plant 2, the higher amount of scrap as well as the additional testing costs also lead to a simultaneous increase in quality related costs by 65.6%. In Plant 3, 4, and 5, the quality related costs decreased by 4.1%, 10.9% and 32.1% respectively. These are defined as a sum of inspection, scrapping and rework costs. Although the absolute amount of scrap has increased and the failure costs in Plant 2 have grown enormously, the implementation of this quality-enhancing measure has had a slight positive effect on the quality related costs of the overall network. These were reduced by 1.1%, since less scrap is transported and further processed, thus resulting is lower reject costs per meter.

Figure 6. Effects of quality measure on quality level (amount of scrap)

Figure 7. Effects of quality measure on quality costs

It remains to be answered, how the implementation of the measure affects the targets of the individual plants and the whole production network. The results are depicted in Figure 8. It can be seen, that the overall quality performance of the network improves, due to fewer defective products delivered to the customer and the overall costs. Although the benefit in the dimension of sustainability decreases, the measure has an overall positive effect on the overall network.

If looking at the individual plants, it can be seen, that due to the improvement measure, the performance of Plant 2 decreases significantly in the dimensions quality cost and sustainability, because as a high-tech factory, these dimensions take a high importance in their target system. The implementation of the quality measure is advantageous for Plants 4 and 5, since these low cost factories especially benefit from the reduction in quality costs due to the importance of this dimension. The benefits regarding the quality level and reduced amount of scrap is not seen as highly important. It can be concluded, that an implementation of the quality measure at Plant 2 is beneficial for the whole manufacturing network. However, the cost benefits at Plant 4 and 5 should be shared with Plant 2 in order to compensate for implementing the quality measure in favor of the whole manufacturing network.

Figure 8. Evaluation of plant and network performances

# 5 Conclusion and research outlook

This paper derived the need for developing an integrated approach for optimizing quality control in IMN from literature. This includes a hierarchical target system model, a value-stream based approach to identify weaknesses and improvement potentials in the quality strategy and a systematization concept for appropriate quality measures. All these concepts are implemented in a multi-method simulation allowing globally operating companies to dynamically evaluate different improvement measures with regard to individual target systems of involved plants as well as the whole network. The developed approach was applied in an industrial case study. The following are the key findings from the case study:

The evaluation of failures during the analysis phase showed, that out of the distinguished 71 failures, most of the recorded scrap in the manufacturing network results from three failure types

It became evident, that from plant perspective focus should be put on other failures and processes in the manufacturing network compared to taking the whole manufacturing network into account

The implemented quality measure has a negative impact on the dimensions quality and costs of one plant, but has positive effects on all other plants, resulting in a better performance of the whole manufacturing network

Because of the negative impact of the quality measure on the plant implementing the measure in favour of the whole manufacturing network, the plant should be compensated for the implementation

Plant 4 and plant 5 profit most from the implementation of the quality measure, because their quality related cost drastically decreased and their main target as low-cost factories is to reduce costs

As implications for industry it can be concluded, that the VSM concept extended by quality processes and indicators is a suitable tool for systematically identifying weaknesses and improvement potentials in the quality control strategy of an IMN. In addition, the multi-method simulation approach allows for dynamically evaluating the effects of quality improvement measures on different plants with divergent target systems and the whole network without having to implement the measure at potentially high costs.

For academics, further research includes the evaluation of the concept in more case studies to derive patterns across companies or whole industries. Since the focus was put on a network with a process-oriented strategy, it needs to be evaluated, if the approach would be also valid for other strategies. Furthermore, the network simulation could be enhanced by an optimization module, which could either allow for a direct definition of the optimal strategy under given restrictions or an evaluation of the simulation results using statistical tools.

# Funding

This work was supported by the Deutsche Forschungsgemeinschaft (German Research Foundation) under Grant LA 2351/28-1.

# References

Abdulmalek, F.A., and Rajgopal J. 2007. “Analyzing the benefits of lean manufacturing and value stream mapping via simulation: A process sector case study.” International Journal of Production Economics 107: 223-236.

Agyapong-Kodua, K., Ajaefobi, J.O., and Weston R.H. 2009. “Modelling dynamic value streams in support of process design and evaluation.” International Journal of Computer Integrated Manufacturing 22: 411-427.

Arndt, T., Buderer, C., Hofmann, M., and Lanza G. 2016. “Simulation-Based Evaluation of Quality Control Strategies in Global Manufacturing Networks.” Advanced Materials Research 1140: 473-480.

Belleflamme, P., and Peitz M. 2014. “Asymmetric information and overinvestment in quality.” European Economic Review 66: 127-143.

Cheng, Y., Farooq, S., and Johansen J. 2015. “International manufacturing network. Past, present, and future.” International Journal of Operations and Production Management 35: 392-429.

Colledani, M., Tolio, T., Fischer, A., Iung, B., Lanza, G., Schmitt, R. and Váncza, J. 2014. “Design and management of manufacturing systems for production quality.“ CIRP Annals - Manufacturing Technology 63: 773–796.

Colotla, I., Shi, Y., and Gregory M. 2003. “Operation and performance of international manufacturing networks.” International Journal of Operations and Production Management 23: 1184-1206.

Das, K., and Sengupta S. 2010. “Modelling supply chain networks: a quality-oriented approach.” International Journal of Quality Reliability Management 27: 506-526.

Erasmus, J. H. 2008. Qualitätsmanagement in global verteilten Wertschöpfungsprozessen. Aachen: Shaker

Erasmus, J. H., and Wulfsberg, J. P. 2007. “Robuste QM-Strukturen für globalisierte Fertigungsorganisationen“, Zeitschrift für wirtschaftlichen Fabrikbetrieb 2007: 273–279.

Feldmann, A., Olhager, J., and Persson F. 2009. “Designing and managing manufacturing networks - a survey of Swedish plants.” Production Planning and Control 33: 101-112.

Feldmann, A., and Olhager J. 2013. “Plant roles: site competence bundles and their relationships with site location factors and performance.” International Journal of Operations & Production Management 75: 722-744.

Ferdows, K. 1997. “Making the most of foreign factories.” Harvard Business Review 15: 73–88.

Friedli, T., Mundt, A., and Thomas S. 2014. Strategic Management of Global Manufacturing Networks. Heidelberg: Springer.

Friedli, T., and Schuh G. 2012. Wettbewerbsfähigkeit der Produktion an Hochlohnstandorten. Heidelberg: Springer.

Forster, S. T. 2008. “Towards an understanding of supply chain quality management.” Journal of Operations Management 26: 461-467.

Fuhrmann, M., Frank, D., Falk, B., and Schmitt R. 2016. “Improving a Game-based Education Tool for Quality Management in Production Networks.” Procedia CIRP 41: 692–696.

Giannakis, M., and Louis, M. 2011. “A Multi-Agent Based Framework for Supply Chain Risk Management.” Journal of Purchasing and Supply Management 17: 23-31.

Haefner, B., Kraemer, A., Stauss, T., and Lanza G. 2014. “Quality Value Stream Mapping.” Procedia CIRP 17: 254-259.

Jochem, R., and Menrath, M. (Ed.) 2015. Globales Qualitätsmanagement. Basis für eine erfolgreiche internationale Unternehmensführung. Duesseldorf: Symposion Publ.

Johannsen, F., and Leist S. 2009. “Evaluation ausgewählter Qualitätstechniken in Six-Sigma-Projekten.“ Chap. in Six Sigma in der Finanzbranche. Frankfurt: Frankfurt School.

Kaczmarek, M. 2006. Modellbasierte Gestaltung von Supply Chains - ein prozess- und simulationsorientierter Ansatz. Hamburg: Dr. Kovac.

Kaipia, R. 2007. Supply chain coordination. Studies on planning and information sharing mechanisms. Espoo: Helsinki University of Technology.

Kaynak, H., and Hartley J. 2008. “A replication and extension of quality management into the supply chain.” Journal of Operations Management 26: 468-489.

Kumar, M., and Gregory M. 2013. “An exploration of Risk Management in Global Industrial Investment.” Risk Management Journal 15: 271-300.

Kumar, M., Srai, J. S., and Gregory M. 2016. “Risk management in plant investment decisions: risk typology, dimensions and process.” Production Planning and Control 27: 761-773.

Lanza, G., and Arndt T. 2016. Leitfaden zur Gestaltung von Qualitätssicherungsstrategien in globalen Wertschöpfungsnetzwerken. Frankfurt: FQS Forschungsgemeinschaft Qualität e.V.

Leitao, P. 2009. “Agent-based Distributed Manufacturing Control: A State-Of-The-Art Survey.” Engineering Application of Artificial Intelligence 22: 979-991.

Liu, Y., and Hipel K. W. 2012. “A Hierarchical Decision Model to Select Quality Control Strategies for a Complex Product.” IEEE Transactions on Systems, Man, and Cybernetics - Part A: Systems and Humans 42: 814-826.

Long, X. 2014. “Distributed supply chain network modelling and simulation: integration of agent-based distributed simulation and improved SCOR model.” International Journal of Production Research 52: 6899-6917.

McDonald, T., Van Aken E. M., and Rentes A. F. 2002. “Utilising simulation to enhance value stream mapping: A manufacturing case application.” International Journal of Logistics Research and Applications 5: 213-232.

McManus, H. L. 2005. Product development value stream mapping (PDVSM) manual. Cambridge: Lean Aerospace Initiative.

Meijboom, B., and Vos B. 1997. “International manufacturing and location decisions. Balancing configuration and co‐ordination aspects.” International Journal of Operations & Production Management 17: 790–805.

Nyhuis, P., Nickel, R., and Tullius, K. (Ed.) 2008. Globales Varianten-Produktionssystem. Globalisierung mit System, Garbsen: PZH-Verl.

O’Connor, B. E. 2014. “Modified value stream mapping of airport check-in procedures.” PhD diss., University of Rio de Janeiro.

Rau, M. 2015. “Leitlinie zur Problemlösung.“ Chap. in Handbuch QM-Methoden. Munich: Hanser.

Robinson, C.J., and Malhotra M.K. 2005. “Defining the concept of supply chain quality management and its relevance to academic and industrial practice.” International Journal of Production Economics 96: 315-337.

Rother M., and Shook J. 2003. Learning to see Value-Stream Mapping to Create Value and Eliminate Muda. Brookline: Lean Enterprise Institute.

Rudberg, M., and Olhager J. 2003. “Manufacturing networks and supply chains: An operations strategy perspective.” Omega 31: 29–39.

Rudberg, M., and West B. M. 2008. “Global operations strategy. Coordinating manufacturing networks.” Omega 36: pp. 91–106.

Schenk, M., Wirth, S. and Müller, E. 2014. “Fabrikplanung und Fabrikbetrieb. Methoden für die wandlungsfähige, vernetzte und ressourceneffiziente Fabrik.” Berlin, Heidelberg: Springer Vieweg.

Schmenner, R. 1982. “Multiplant Manufacturing Strategies Among the Fortune 500.” Journal of Operations Management 2: 77-86.

Shi, Y., and Gregory M. 1998. “International manufacturing networks—to develop global competitive capabilities.” Journal of Operations Management 16: 195–214.

Spalt, P., Braun, A.-T., and Schoellhammer, O. 2013. “Globales Wertstrommanagement. Eine Methode zur Komplexitätsbeherrschung und kontinuierlichen Verbesserung von Wertschöpfungsnetzen.“ Zeitschrift für wirtschaftlichen Fabrikbetrieb 108: 926–930.

Tague, N. R. 2005. The quality toolbox. Milwaukee: ASQ Quality Press.

Tapiero, C. S., and Kogan K. 2007. “Risk and Quality Control in a Supply Chain: Competitive and Collaborative Approaches.” Journal of the Operational Research Society 58 11: 1440–1448

Weber, J. 1995. Modulare Organisationsstrukturen internationaler Unternehmensnetzwerke. Wiesbaden: Gabler.

Vereecke, A., and Van Dierdonck R. 2002. “The strategic role of the plant: testing Ferdow's model.” International Journal of Operations & Production Management 22: 492-514.

Vokurka, R. J., and Davis R. A. 2004.  “Manufacturing strategic facility types.” Industrial Management & Data Systems 104: 490-504

Wankhade, L., and Dabade, B. M. 2006. “Analysis of quality uncertainty due to information asymmetry.“ International Journal of Quality & Reliability Management 23: 230–241.

Wiendahl, H. P., Reichardt, J., and Nyhuis, P. 2014. Handbuch Fabrikplanung - Konzept Gestaltung und Umsetzung wandlungsfähiger Produktionsstätten. Munich: Carl Hanser.

Wolff, M. 2013. Methode zur Wertstromoptimierung mittels simulativer Bewertung von Handlungsalternativen. Aachen: Shaker.

Yüzgülec, G., Ullerich, S. F., and Siemes, F. 2015. “Der Weg zur Premiumqualität. Die Herausforderungen der Automobilhersteller“, productivITy 20: 18–20.

Zu, X., and Kaynak H. 2012. “An agency theory perspective on supply chain quality management.” International Journal of Operations & Production Management 32: 423-446.