Supplementary Information

SiDG Control script (Python 3.0):

# Sideways FDG script

# written by Jheng-Han Tsai, November 2018

##############################################################################

#1. Select mode - calibration or measurement

mode = 'mea' # 'cal'(calibration) or 'mea'(measurement)

#2. Select folder for steps and data

folder = 'steps/' #Steps' folder name

folder_data = 'data/' #data's folder name

###### Calibration mode #################

#3. Select file name for calibration steps

stepsX_fileName = 'X.csv' #File name for X-axis steps

#4. Run

###### Measurement mode #################

#3. Select file name for Z-axis movement

stepsZ_fileName = 'Z_pva.csv' #File name for Z-axis steps

#4. Select file name for Cd reference

cal_fileName = 'cal.csv' #File name for Cd reference

#5. Enter length of X-axis positioner

lenX = 6

#6. Run

##############################################################################

import threading

import time

import pandas as pd

import numpy as np

import matplotlib.pyplot as plt

from scipy import signal, interpolate, stats

import concurrent.futures

import os

from zaberbinary import BinarySerial, BinaryDevice, BinaryCommand, BinaryReply

import nidaqmx

from nidaqmx.constants import TerminalConfiguration, VoltageUnits, Edge, AcquisitionType

import harvard

#################### Zaber T-serial Positioner ###############################

class Zaber():

def __init__(self):

# Open a serial port.

self.portZ = BinarySerial('COM7')

self.portX = BinarySerial('COM3')

# Get a handle for device #1 on the serial chain. This assumes you have a

# device already in ASCII 115,220 baud mode at address 1 on your port.

self.deviceZ = BinaryDevice(self.portZ, 1) # Device number 1

self.deviceX = BinaryDevice(self.portX, 2) # Device number 2

print('Opening Zaber.')

def __enter__(self):

return self

def __exit__(self, exc_type, exc_value, traceback):

#This is the class destructor, ensures that the stage is not left active on exit

try:

self.portZ.close()

self.portX.close()

except:

pass

print('Closing Zaber.')

def moveDown(self, steps):

# Now move the device to a non-home position.

print ('Z-axis zaber moves '+'{:.2f}'.format(steps/2000)+' mm ('+str(int(steps))+' steps). \n')

reply = self.deviceZ.move_rel(steps) # move rel 2000 microsteps

def moveLeft(self, steps):

# Now move the device to a non-home position.

print ('X-axis zaber moves '+'{:.2f}'.format(steps/2000)+' mm ('+str(int(steps))+' steps). \n')

reply = self.deviceX.move_rel(steps) # move rel 2000 microsteps

def upToHome(self):

# Home the device and check the result.

print ('Z-axis zaber goes home.\n')

reply = self.deviceZ.home()

def rightToHome(self):

# Home the device and check the result.

print ('X-axis zaber goes home.\n')

reply = self.deviceX.home()

def calibration(self, stepsX_path):

stepsX_cal = np.array(pd.read_csv(stepsX_path))

print (stepsX_cal)

df = pd.DataFrame({'X' : [],

'Z' : [],

'direction' : [],

'flowrate' : [],

'targettime' : [],

'pressure mean': [],

'pressure std': [],

'Cd mean': [],

'Cd std': [],

'Cd se': [],

'Cd n': []})

loop = stepsX_cal.shape[0]

for i in range(0,loop):

print (i)

###### Z-axis Zaber positioner ######

if i == 0:

stepsZ = 0

else:

movestepsZ = -(stepsX_cal[i,1]-stepsX_cal[i-1,1])

if movestepsZ == 0:

stepsZ = 0

else:

stepsZ = int(movestepsZ*2000)

self.moveDown(stepsZ)

time.sleep(3)

###### X-axis Zaber positioner ######

if i == 0:

steps = 0

else:

if stepsX_cal[i,0] == 'home':

self.rightToHome()

else:

if stepsX_cal[i-1,0] == 'home':

movesteps = -(float(stepsX_cal[i,0])-lenX)

else:

movesteps = -(float(stepsX_cal[i,0])-float(stepsX_cal[i-1,0]))

steps = int(movesteps*2000)

self.moveLeft(steps)

time.sleep(1)

###### Syringe pump ######

direction = stepsX_cal[i,2]

flowrate = stepsX_cal[i,3]

targettime = stepsX_cal[i,4]

###### NiDAQ ######

filenameT = 'X'+str(stepsX_cal[i,0])+'Z'+str(stepsX_cal[i,1])+'_temperature_waveform.txt'

filename = 'X'+str(stepsX_cal[i,0])+str(stepsX_cal[i,2])+str(stepsX_cal[i,3])+'mlmin'\

+str(stepsX_cal[i,4])+'s'+'Z'+str(stepsX_cal[i,1])+'_waveform.txt'

operation = True

while operation:

me = threading.Thread(target=measure(flowrate, direction, targettime, filename, filenameT))

me.start()

me.join()

lp = filter_lowpass(filename,flowrate,targettime)

if abs(lp[0]) >= 100:

operation = False

else:

print ('Suffering an issue of syringe pump, repeat measurement.')

df2 = pd.DataFrame({'X' : stepsX_cal[i,0],

'Z' : stepsX_cal[i,1],

'direction' : stepsX_cal[i,2],

'flowrate' : stepsX_cal[i,3],

'targettime' : stepsX_cal[i,4],

'pressure mean': [lp[0]],

'pressure std': [lp[1]],

'Cd mean': [lp[2]],

'Cd std': [lp[3]],

'Cd se': [lp[4]],

'Cd n': [lp[5]]})

print (df2)

df = pd.concat([df,df2])

os.remove(filename)

os.remove(filenameT)

print (df)

ffname = time.strftime('%Y%m%d')+'_'+time.strftime('%H%M%S')+'_calibration.csv'

df.to_csv(ffname,index=False,sep=',')

print ('All done.')

def sideways(self, stepsZ_path, cal_path):

stepsZ_list = np.array(pd.read_csv(stepsZ_path))

cal_list = np.array(pd.read_csv(cal_path))

print (stepsZ_list)

wholeloop = stepsZ_list.shape[0]

for i in range(0,wholeloop):

if i == 0:

zaber.rightToHome() # X-axis zaber goes home for starting.

recordX = 0

else:

if stepsZ_list[i,0] == stepsZ_list[i-1,0]:

recordX = recordX

else:

zaber.rightToHome()

recordX = 0

###### Z-axis zaber positioner ######

# Read Zaber's current position

zr = open(folder+'Zzaber_position.txt', "r")

lines = zr.readlines()

zr.close()

z_record = lines[0]

print (z_record)

movestepsZ = (float(stepsZ_list[i,0])-float(z_record))

stepsZ = -int(movestepsZ*2000)

self.moveDown(stepsZ)

# Start to record time, just immerse

if i == 0:

initialTime = time.clock()

df = pd.DataFrame({'loop' : [],

'X' : [],

'Z' : [],

'flow rate' : [],

'target time' : [],

'E_pressure mean': [],

'E_pressure std': [],

'E_Cd mean': [],

'E_Cd std': [],

'E_estimated h': [],

'E_estimated h std': [],

'E_thickness': [],

'S_pressure mean': [],

'S_pressure std': [],

'S_Cd mean': [],

'S_Cd std': [],

'S_estimated h': [],

'S_estimated h std': [],

'S_thickness': [],

'E_operation time': [],

'S_operation time': [],

'sample number': [],

'Temperature mean': [],

'Temperature std': []})

else:

if stepsZ_list[i,0] == stepsZ_list[i-1,0]:

initialTime = initialTime

df = df

else:

initialTime = time.clock()

df = pd.DataFrame({'loop' : [],

'X' : [],

'Z' : [],

'flow rate' : [],

'target time' : [],

'E_pressure mean': [],

'E_pressure std': [],

'E_Cd mean': [],

'E_Cd std': [],

'E_estimated h': [],

'E_estimated h std': [],

'E_thickness': [],

'S_pressure mean': [],

'S_pressure std': [],

'S_Cd mean': [],

'S_Cd std': [],

'S_estimated h': [],

'S_estimated h std': [],

'S_thickness': [],

'E_operation time': [],

'S_operation time': [],

'sample number': [],

'Temperature mean': [],

'Temperature std': []})

time.sleep(2)

# Write Zaber's current position

new_position = stepsZ_list[i,0]

zw = open(folder+'Zzaber_position.txt', "w")

zw.write(str(new_position))

zw.close()

loop = int(stepsZ_list[i,10])

flowrate = stepsZ_list[i,3]

targettime = stepsZ_list[i,4]

pre_approach = stepsZ_list[i,5]

move_approach = stepsZ_list[i,6]

pre_away = stepsZ_list[i,7]

move_away = stepsZ_list[i,8]

time.sleep(stepsZ_list[i,1]) # Wait for starting.

movestepsX = 0

hE = 0

hS = 0

for j in range(0,loop):

loopnumber = j+1

print ('Start '+str(loopnumber)+' of '+str(loop)+' loops.')

###### X-axis zaber positioner ######

if j == 0:

if i > 0 and stepsZ_list[i,0] == stepsZ_list[i-1,0]:

movestepsX = 0

else:

movestepsX = stepsZ_list[i,2]

else:

movestepsX = movestepsX

stepsX = -int(movestepsX*2000)

self.moveLeft(stepsX)

recordX = recordX+movestepsX

time.sleep(1)

###### filename for temperature measurement ######

filenameT = folder_data+'Z'+str(stepsZ_list[i,0])+'X'+str(recordX)+'loop'+str(j+1)+'_temperature_waveform.txt'

###### Measurement for ejection pressure (Temperature) ######

ejection = True

while ejection:

filenameE = folder_data+'Z'+str(stepsZ_list[i,0])+'X'+str(recordX)+'E'+str(flowrate)+'mlmin'\

+str(targettime)+'sloop'+str(j+1)+'_waveform.txt'

meE = threading.Thread(target=measure(flowrate,'E', targettime, filenameE, filenameT))

meE.start()

meE.join()

lpE = filter_lowpass(filenameE,flowrate,targettime)

if abs(lpE[0]) >= 100:

ejection = False

else:

print ('Suffering an issue of syringe pump, repeat ejection measurement.')

endTimeE = time.clock()-initialTime

###### Measurement for suction pressure (Temperature) ######

suction = True

while suction:

filenameS = folder_data+'Z'+str(stepsZ_list[i,0])+'X'+str(recordX)+'S'+str(flowrate)+'mlmin'\

+str(targettime)+'sloop'+str(j+1)+'_waveform.txt'

meS = threading.Thread(target=measure(flowrate,'S', targettime, filenameS, filenameT))

meS.start()

meS.join()

lpS = filter_lowpass(filenameS,flowrate,targettime)

if abs(lpS[0]) >= 100:

suction = False

else:

print ('Suffering an issue of syringe pump, repeat suction measurement.')

endTimeS = time.clock()-initialTime

###### Analysis of temperature data ######

lpT = temperature(filenameT)

###### Interpolation ######

fcE = interpolate.interp1d(cal_list[:,1], cal_list[:,0], kind='linear', bounds_error=False, fill_value='extrapolate')

fcS = interpolate.interp1d(cal_list[:,2], cal_list[:,0], kind='linear', bounds_error=False, fill_value='extrapolate')

hE = fcE(lpE[2])

hS = fcS(lpS[2])

thicknessE = lenX+recordX-hE

thicknessS = lenX+recordX-hS

###### Adjust clearance ######

print ('Measured suction pressure: '+str(abs(lpS[0]))+' Pa.')

if abs(lpS[0]) < pre_approach:

movestepsX = -move_approach

elif abs(lpS[0]) > pre_away:

movestepsX = move_away

else:

movestepsX = 0

df2 = pd.DataFrame({'loop' : loopnumber,

'X' : recordX,

'Z' : stepsZ_list[i,0],

'flow rate' : flowrate,

'target time' : targettime,

'E_pressure mean': [lpE[0]],

'E_pressure std': [lpE[1]],

'E_Cd mean': [lpE[2]],

'E_Cd std': [lpE[3]],

'E_estimated h': hE,

'E_estimated h std': interpolateSDE,

'E_thickness': thicknessE,

'E_operation time': endTimeE,

'S_pressure mean': [lpS[0]],

'S_pressure std': [lpS[1]],

'S_Cd mean': [lpS[2]],

'S_Cd std': [lpS[3]],

'S_estimated h': hS,

'S_estimated h std': interpolateSDS,

'S_thickness': thicknessS,

'S_operation time': endTimeS,

'sample number': [lpS[5]],

'Temperature mean': [lpT[0]],

'Temperature std': [lpT[1]]})

df2_show = pd.DataFrame({'loop' : loopnumber,

'X' : recordX,

'Z' : stepsZ_list[i,0],

'E_pressure mean': [lpE[0]],

'E_pressure std': [lpE[1]],

'E_Cd mean': [lpE[2]],

'E_Cd std': [lpE[3]],

'E_estimated h': hE,

'E_thickness': thicknessE,

'E_operation time': endTimeE,

'S_pressure mean': [lpS[0]],

'S_pressure std': [lpS[1]],

'S_Cd mean': [lpS[2]],

'S_Cd std': [lpS[3]],

'S_estimated h': hS,

'S_thickness': thicknessS,

'S_operation time': endTimeS})

print (df2_show)

os.remove(filenameE)

os.remove(filenameS)

os.remove(filenameT)

time.sleep(stepsZ_list[i,9]) # Wait for next loop.

print ('Wait '+str(stepsZ_list[i,9])+' s for next loop.')

df = pd.concat([df,df2])

print (df)

fname = folder_data+time.strftime('%Y%m%d')+'_'+time.strftime('%H%M%S')+'_Round'+str(i+1)+'_results.csv'

df.to_csv(fname,index=False,sep=',')

zaber.rightToHome() # X-axis zaber goes home for ending.

time.sleep(2)

zaber.upToHome() # Z-axis zaber goes home for ending.

new_position = 0

zw = open(folder+'Zzaber_position.txt', "w")

zw.write(str(new_position))

zw.close()

print ('All done.')

####################### NIDAQ ###########################################################

def nidaq(filename, targetime):

niport = "Dev1/ai0"

sampling_rate = 1000 #Sampling rate

samples_per_channel = (int(targetime)+4)*sampling_rate #Samples per channel

with nidaqmx.Task() as task:

task.ai_channels.add_ai_voltage_chan(niport, terminal_config=TerminalConfiguration.RSE, min_val=-5.0, max_val=5.0, units=VoltageUnits.VOLTS)

task.timing.cfg_samp_clk_timing(sampling_rate, active_edge=Edge.RISING, sample_mode=AcquisitionType.FINITE, samps_per_chan=samples_per_channel)

d1 = np.linspace(0,samples_per_channel/sampling_rate,samples_per_channel)

print ('NiDAQ starts.\n')

d2 = task.read(number_of_samples_per_channel=samples_per_channel)

d3 = np.zeros((samples_per_channel,2))

d3[:,0] = d1

for i in range(len(d2)):

d3[i,1] = d2[i]*3544-8700

np.savetxt(filename, d3, delimiter=',')

print ('NiDAQ finishes.\n')

################# Harvard Syringe Pump ############################################

def syringe_pump(flowrate, direction, targetime):

synport = 'COM6'

diameter = 23 #mm

print ('Syringe pump starts ('+direction+' mode at '+str(flowrate)+' ml/min '+str(targetime)+'s). \n')

with harvard.Chain(synport) as chain:

p11 = harvard.Pump(chain, address=0)

p11.setdiameter(diameter)  # mm

if direction == 'E':

p11.setflowrateinfuse(flowrate)

p11.settargetime(targetime)

if targetime > 0:

p11.infuse()

else:

p11.setflowratewithdraw(flowrate)

p11.settargetime(targetime)

if targetime > 0:

p11.withdraw()

chain.close()

print ('Syringe pump finishes.\n')

############# Low-pass Filter ##############################

def filter_lowpass(filename,flowrate,targetime):

stat = np.zeros((8))

x = np.array(pd.read_csv(filename))

pstatNumber = 2000

pstat = np.zeros((pstatNumber,2))

for k in range(pstatNumber):

pstat[k,0] = x[k,0]

pstat[k,1] = x[k,1]

bstat, astat = signal.butter(3, 1/500, 'lowpass')

ystat = signal.filtfilt(bstat, astat, pstat[:,1])

pstatMean = np.mean(ystat)

pstatStd = np.std(ystat)

cut = (int(targetime)-1)*1000

start = 4000

cutx = np.zeros((cut,2))

for j in range(cut):

cutx[j,0] = x[start+j,0]

cutx[j,1] = x[start+j,1]

b, a = signal.butter(3, 1/500, 'lowpass')

y = signal.filtfilt(b, a, cutx[:,1])

m = flowrate/1000000*997.3/60 #Unit: kg/s

dt = 0.001

rho = 997.3

cd = 4*m/(3.1415*dt**2*(2*rho*abs(y-pstatMean))**0.5)

stat[0] = np.mean(y-pstatMean)

stat[1] = np.std(y-pstatMean)

stat[2] = np.mean(cd)

stat[3] = np.std(cd)

stat[4] = stats.sem(cd) # Standard error of measurement

stat[5] = len(cd[:]) # Number of samples

stat[6] = pstatMean # Static pressure mean

stat[7] = pstatStd # Static pressure standard deviation

return stat

############### Measurement ######################

def measure(flowrate, direction, targetime, filename, filenameThermo):

with concurrent.futures.ThreadPoolExecutor(max_workers=2) as executor:

#with concurrent.futures.ProcessPoolExecutor() as executor:

sp = executor.submit(syringe_pump, flowrate, direction, targetime)

nithermo = executor.submit(nidaqThermo, filenameThermo)

time.sleep(3)

ni = executor.submit(nidaq, filename, targetime)

############ Thermocouple nidaq ##############################

def nidaqThermo(filename):

niport = "Dev1/ai4"

sampling_rate = 100 #Sampling rate

samples_per_channel = sampling_rate #Samples per channel

with nidaqmx.Task() as task:

task.ai_channels.add_ai_voltage_chan(niport, terminal_config=TerminalConfiguration.DIFFERENTIAL, min_val=-5.0, max_val=5.0, units=VoltageUnits.VOLTS)

task.timing.cfg_samp_clk_timing(sampling_rate, active_edge=Edge.RISING, sample_mode=AcquisitionType.FINITE, samps_per_chan=samples_per_channel)

d1 = np.linspace(0,samples_per_channel/sampling_rate,samples_per_channel)

print ('Thermocouple starts.\n')

d2 = task.read(number_of_samples_per_channel=samples_per_channel)

d3 = np.zeros((samples_per_channel,2))

d3[:,0] = d1

for i in range(len(d2)):

d3[i,1] = d2[i]

np.savetxt(filename, d3, delimiter=',')

print ('Thermocouple finishes.\n')

############# Thermocouple ###################################

def temperature(filename):

sampling_rate = 100

data = np.zeros((2))

x = np.array(pd.read_csv(filename))

xCal = np.zeros((len(x[:,1])))

for i in range (len(xCal)):

xCal[i] = x[i,1]*1000

b, a = signal.butter(3, 1/(sampling_rate/2), 'lowpass')

y = signal.filtfilt(b, a, xCal)

data[0] = np.mean(y)

data[1] = np.std(y)

print ('Mean temperature: '+str(data[0])+', Std: '+str(data[1])+'.\n')

return data

if __name__ == '__main__':

with Zaber() as zaber:

if mode == 'cal':

stepsX_path = folder + stepsX_fileName

zaber.calibration(stepsX_path)

elif mode == 'mea':

stepsZ_path = folder + stepsZ_fileName

cal_path = folder + cal_fileName

zaber.sideways(stepsZ_path, cal_path)