Linear static behavior of curved beams coupled with strings representing also fiber-reinforced masonry arches

Sergio Chirivì1, Tim Ibell2, Alessandro Contento3 and Angelo Di Egidio1

1 DICEAA - University of L’Aquila, Italy

2 Department of Architecture and Civil Engineering - University of Bath, UK

3 Department of Civil and Environmental Engineering - University of Illinois at Urbana-Champaign, USA

Abstract

A linear elastic analytical model of curved beam with constant curvature, coupled with  curved strings has been developed. To model the connections between the strings and the beam, a continuous distribution of tangential and orthogonal linear elastic springs has been introduced. To evaluate the behavior of the system, a numerical integration of the field equations has been performed. A parametric analysis, capable of pointing out the influence on the behavior of the system of both the stiffness of the tangential connection between strings and beam and of the geometrical characteristic of the beam, has been performed. The results of the proposed model have been compared with those obtained from a finite element model. They show that the strings, in some cases, do not work as expected.

Keywords

Arch, string, coupling, analytical investigation.

- Introduction

The coupling between curved structural elements and reinforcing layers is very frequent in the retrofit of masonry arches or vaults of historical buildings. In the last years the FRP composites seem to be one of the best candidates to assume the role of ideal reinforcement, given their good combination of properties such as lightness, stiffness, corrosion resistance and ease of application compared with other methods. Many analyses have been carried out to better understand the role of composite materials as such the reinforcement. For this purpose, numerical and experimental tests have been carried out on unreinforced and reinforced masonry arches, confirming the effectiveness of FRP extrados strips both in terms of load bearing capacity and ultimate displacement (Cancelliere et al. (2010)). Other experimental analyses have shown that composite materials are significant in enhancing the capability to support horizontal loads, and in increasing the collapse load, stiffness and ductility (Briccoli Bati et al. (2007)). Moreover, strengthening at the intrados seems to be the most effective option to increase structural strength, whereas the application of fiber composites at the extrados provides higher deformation capacity prior to failure (Oliveira et al. (2010)). Considering a four-hinge mechanism, hinge positions on the arch under external loads were varied along the arch barrel to identify their optimum positions to achieve the minimum collapse load (Chen et al. (2007)). Furthermore, FRP allows the line of thrust to fall outside the thickness of the arch, increasing the capacity of the arch and reducing the lateral thrust transmitted to the piers (De Lorenzis et al. (2007)). Stress analyses were carried out considering the composite material as a linear elastic material and the perfect adhesion between the masonry and the FRP reinforcement (Marfia et al. (2008)). In other analyses, the performance of the interface between fiber composite and masonry is one of the key factors affecting the behaviour of strengthened structures. For this reason, experimental studies to evaluate the bond between FRP and masonry were carried out (Capozucca (2010); D’Ambrisi et al. (2013)).

Aim of this paper is to investigate the role of reinforcing fibers applied to curved  beams realized with homogeneous and isotropic materials. The results of this paper are not generally applicable to No-Tension-Materials.. However, the results obtained here could provide useful information in curved masonry structures where the prevailing stress is the compression, until the low tension strength of the material is reached.

A linear elastic analytical model of a curved beam with constant curvature coupled with curved strings is developed. A continuous distribution of tangential and normal linear elastic springs is used to model the connections between the strings and the beam. The kinematic, equilibrium and constitutive equations, capable of describing the behavior of the mechanical system, are obtained. A numerical integration of the field equations, written as a set of ordinary differential equations, is performed to obtain the solution to the elastic problem.

Two types of static excitation of the coupled system are taken into account. The first kind is a distributed load, while the other are base displacements of the arch. A parametric analysis, capable of pointing out the role of some geometrical or mechanical parameters characterizing the system is conducted. Specifically, the stiffness of the tangential connection between strings and beam and some geometrical characteristics of the arch are taken as variable parameters in the analyses.

The results provided by the proposed model are compared with those obtained by a finite element (FE) model. The good agreement between the analytical and the computational models allows the use of the FE model to investigate the case of strings that are not able to react to compression.

- Assumptions and motivation for the researh

In many bi-dimensional structures, two families of curved interacting beams can be recognized. Depending on the geometrical characteristics of the shell, often, arches constitute one of these families. These elements usually plays an important role in the statics (i.e. in barrel shells, spherical shells and others as in de Leo et al. (2014), (2015)).

It is important to remark that, for the purpose of this analysis, the shells are considered built with homogeneous, isotropic material; hence the structure works well both in compression and in tension. To improve the performance of these types of structures use of reinforcing fibers can be done. In recent years, the use of fiber composites has become common practice.

In masonry buildings, fiber reinforcement is often used for the strengthening of bi-dimensional curved structures, such as barrel and pavilion vaults. However, since the analyses performed in this investigation are conducted considering an elastic, linear behavior of the structure, they are  only partially applicable to masonry structures.

Although the results of this paper may seem far from the real behavior of masonry structures, under some assumption they could offer interesting indications about the use of fiber-reinforcement for masonry structural elements. For example, an arch-like structure works properly if only compressive stresses occur, due to the limited tensile strength of the masonry. If the tensile stresses exceed this limit, damage appears. However, until some damage is shown, the structure works in the linear, elastic field. Inside this operating range this paper could be useful to give some indications regarding the use of fiber-reinforcements.

- Analytical model

An arch is characterized by a few parameters (Figure 1(a, c)). Specifically, R is the radius of the arch, θ0 is the angle that measures the degree of flattening of the arch and h is the height of the section; C is the center of the arch. If , then the structure is a ‘semicircular’ arch, while if , it is a ‘segmental’ arch. The arch is modeled by using the classical mono-dimensional, polar model of curved planar beam. The fibers, positioned both on the extrados and on the intrados of the beam, are modeled as curved strings, connected to the arch through a continuous distribution of linear elastic springs (Figure 1(b)).

To span the curved domain of the two fiber layers and of the arch, three different local curvilinear coordinates are considered: cs and ci are the coordinates of the upper and lower fiber layers, respectively, while c is the coordinate of the arch (Figure 1(c)). All these coordinates can be expressed as functions of the sole angle ϑ through the following equations:

cs=R+h2ϑ

c=Rϑ	(1)

ci=R-h2ϑ

However, since in beams the dimensions of the section are small compared to their length (h2≪R), the two local coordinates of the fibers cs and ci can be substituted by the coordinates of the beam c.

  - Kinematic, equilibrium and constitutive equations

To describe the kinematics of the arch, the three classical displacement components ut, vt and φt are taken into account. For the fibers, modelled as curved strings, the normal and the tangential displacement components are considered: us, vs and ui, vi (Figure 1(c)). The kinematic equations read:

εt=ut'+vtR        	(2.1)

γt=-utR+vt'-φt 	(2.2)

kt=φt'	(2.3)

εfs=us'+vsR 	(2.4)

εfi=ui'+viR 	(2.5)

εns=vs-vt 	(2.6)

εts=us-ut+h2φt 	(2.7)

εni=vt-vi	(2.8)

εti=ut-ui+h2φt                   	(2.9)

where (.)' represents the derivative with respect to the curvilinear coordinate c. Equations 2.1 to 2.3 represent the classical kinematic equations of an arch. Equations 2.4 and 2.5 refer to the longitudinal elongation of the curved strings, representing the upper and lower fibers, respectively. The two sets of Eqns 2.6-2.7 and Eqns 2.8-2.9 describe the strains of the normal (εns, εni) and of the tangential (εts, εti) springs, representing the upper and lower connections between the fibers and arch, respectively.

The equilibrium equations of the arch coupled with the strings are obtained referring to the internal stresses and the external forces shown in Figure 2. The quantities Nt,Tt,Mt Ni, Ti, Mi describe the internal forces of the arch; Nfs and Nfi are the internal normal forces of the strings representing the upper and lower fibers, respectively. Finally, ts, ss and ti, si represent the stresses in the tangential and normal springs, which model the connections between the upper and lower fibers and the arch, respectively (see Figure 2(a)); pt and pn are the tangential and  normal components of the external load respectively, applied directly on the axis of the arch (see Figure 2(c)). In Figure 2(a), quantities with superscript plus are defined as (.)+≔.+d.dcdc. The equilibrium equations are found to be:

-Nt'-VtR-ts+ti=Pt	(3.1)

NtR-Vt'-ss+si=Pn	(3.2)

-Mt'+ti+tsh2-Vt=0	(3.3)

-Nfs'+ts=0	(3.4)

NfsR+ss=0	(3.5)

-Nfi'-ti=0	(3.6)

NfiR-si=0	(3.7)

Equations 3.1 to 3.3 represent the equilibrium of the internal and external forces acting on an infinitesimal portion of arch. Specifically, they describe the equilibrium of forces acting in the tangential direction, in the normal direction and of the moments, respectively. Equations 3.4 and 3.5 represent equilibrium of forces acting on an infinitesimal portion of the upper fiber, in the tangential and normal directions, respectively. Finally, the last two Eqns 3.6 and 3.7 describe the equilibrium of forces acting on an infinitesimal portion of the lower fiber.

It is worth noticing that the kinematic problem (Eqn 2) is the adjoint of the equilibrium problem (Eqn 3).

  - Assumptions and simplifications

Some internal constraints are introduced following suitable assumptions on the kinematical behavior of the mechanical system. Consequently a simplification of the mathematical problem occurs. However, the introduction of these constraints makes the equations yet capable of well describing the mechanical behavior of the system.

The first internal constraint considered is the classical shear indeformability of the beam. It leads to the vanishing of the shear strain γt. Specifically, it reads:

γt=0⇒φt=vt'-utR	(4)

The second internal constraint is the infinite stiffness of the springs in the direction normal to the axis of the arch, representing the connection between the strings and the arch. This constraint has been introduced to simulate the impossibility of the fibers detaching from the arch in the normal direction, and it leads to the vanishing of deformations εns and  εni, so that:

| εns=0⇒vs=vt εni=0⇒vi=vt | (5) |
| --- | --- |

By taking into account  Eqn 4 the kinematic equations (Eqn 2) become:

εt=ut'+vtR

kt=vt''-ut'R

εfs=us'+vtR

εfi=ui'+vtR	(6)

εts=us-ut+h2(vt'-utR)

εti=ut-ui+h2(vt'-utR)

The equilibrium equations, instead, must be condensed since some internal forces become reactive quantities. Specifically, the shear Vt can be obtained from Eqn 3.3, while the internal forces ss and si come from Eqn 3.5 and Eqn 3.7, respectively. The reactive quantities then read:

Vt=-Mt'+ti+tsh2

ss=-NfsR	(7)

si=NfiR

By substituting Eqn 7 into the equilibrium equations (Eqn 3), the final condensed equilibrium equations appear as:

-Nt'+(Mt'-ti+tsh2)1R-ts+ti=Pt

NtR+(Mt''-ti'+ts'h2)+NfsR+NfiR=Pn

-Nfs'+ts=0	(8)

-Nfi'-ti=0

Even after the introduction of the internal constraints, the kinematic and equilibrium problems appear as the transpose of each other.

A linear elastic behavior of the arch, of the fibers and of the connection springs has been considered. The constitutive laws read:

Nt=EAεt

Mt=EIkt

Nfs=kfsεfs	(9)

Nfi=kfiεfi

ts=gsεts

ti=giεti

where E is the Young modulus of the arch; A and I are the area and the moment of inertia of the section of the arch, respectively; kfs and kfi are the longitudinal stiffnesses of the upper and lower fibers; gs and gi are the stiffnesses of the tangential springs representing the upper and lower connections between fibers and arch, respectively.

The reactive quantities , ss and si, that can no longer be expressed through constitutive laws, can however be obtained from Eqn 7.

  - Field equations

By combining the internal constraints, given by Eqns 4 and 5, the kinematic equations (Eqn 6), the equilibrium equations (Eqns 7 and 8) and the constitutive laws (Eqn 9), the following set of first-order  differential equations is obtained:

ut'=NtEA-vtr

vt'=utr+φt

φt'=MtEI

us'=Nfskfs-vtr

ui'=Nfikfi-vtr

Nt'=-Vtr-gsus-ut+φth2-giut-ui+φth2-pt	(10)

Vt'=Ntr+Nfsr-pn

Mt'=gsus-ut+φth2-giut-ui+φth2h2-Vt

Nfs'=gsus-ut+φth2

Nfi'=giut-ui+φth2

The boundary conditions now read:

ut(Rϑ0)=0	ut(Rϑmax)=0

vt(Rϑ0)=0	vt(Rϑmax)=0

φt(Rϑ0)=0	φt(Rϑmax)=0	(11)

us(Rϑ0)=0	us(Rϑmax)=0

ui(Rϑ0)=0	ui(Rϑmax)=0

They refer to clamped boundaries of the arch and fully constrained fibers. Results have been obtained by numerical integration of Eqn 10, together with the relevant boundary conditions (Eqn 11).

- Computational model

A comparison with a finite element model is performed. To build the Finite Element (FE) model, use is made of standard 3- or 4-node shell elements to model the arch. The fiber reinforcements are modelled by using frame elements while the connections between the fibers and the arch are modelled through tangential and normal link-type elements. The use of shell elements permits the modelling  of fibers not connected to the axis of the arch, but to the extrados or the intrados of the beam, at a distance  from the axis (see Figure 1(c)).

Along the height, the section is meshed with four shell elements. Along the axis of the arch, instead, for the fibers and the arch, frame and shell elements of angular amplitude of  are used, respectively. For example, in a semicircular arch () where , 720 frame elements are used to model both the upper and the lower fibers; 4 × 720 shell elements are instead used to model the arch. Two tangential and normal link elements are collocated between the corresponding nodes of the fibers and the shells. Specifically, 722 link elements are used to model the tangential connection between the fibers and the arch, both at the extrados and the intrados of the beam. An equal number of links is used to model the normal connections.

These characteristics of the mesh have been determined by performing a preliminary convergence analysis of the results provided by the FE model (see Figure 3).

- Numerical simulations

The model developed in Section 3 allows the study of numerous types of problem. In the numerical simulations carried out, semicircular and segmental arches are studied, initially only under operational loads and then applying base displacements.

Only fill and live loads are considered (see Figure 2(b)), because fiber reinforcements are usually applied to structures already deformed under proper weight load. Therefore, only when fill and live loads are restored the stress state in the fibers changes.

5.1 Static operational loads

The system described by Eqn 10 with the relevant boundary conditions (Eqn 11) allow finding the unknown quantities of the problem. The simulations described below, refer to an arch built with materials that possess the same characteristics of masonry and carbon fibers. It must be remarked that the analyses performed here are able to describe only a linear elastic behavior of the system. Hence, results are valid until the low tensile strength of the masonry is reached. However, if the arch works well under operational loads, no tensile stress would manifest itself in the masonry. Moreover, it is worth noting that this is a specific example and the results cannot be generalized to masonry structures. It is useful to remark that the aim of the paper is the study of the role of the fiber-reinforcement of a curved homogeneous isotropic beam.

In the cases reported here, the following geometrical and mechanical characteristics have been used: geometrical radius 3 m, rectangular section of the arch of dimensions 0.30×0.50 m2 (width × height); Young’s modulus of the arch of 3.0×106 kN/m²; thickness of the carbon fibers of 0.378 mm; Young’s modulus of the carbon fibers of 0.19×109 kN/m²; fill load of 19.0 kN/m3; live load of 50.0 kN/m². Equation 15 gives the boundary conditions for the fibers.

In Figure 4 it is possible to see the deformed shape of a semicircular arch under operational loads, and the stresses on the arch and the fibers. Both the upper and the lower fibers are stretched in several areas. Specifically, the lower fibers exhibit tensile stresses on the extremities of the arch and near the key stone, as can be surmised by the deformed shape of the arch.

In the analysis of the role of the degree of flattening of the arch, two different types of flattening have been taken into account. In the first, the radius of the arch is kept constant for analysis purposes as the solution progresses.  A change of the degree of flattening leads to a change in the length of the arch. In the second, the length of the arch is kept constant and, therefore, a change of the degree of flattening implies a change of the radius of the arch. For the first kind of analyses a segmental arch with constant radius  and characteristics equal to those of the case of Figure 4 is studied. In contrast to the previous results shown in Figure 4, in this case both the upper and the lower fibers are in compression (Figure 5). For the second kind, an arch with the same previous flattening () and length, equal to the length  of the semicircular arch of radius , is considered. Also in this case, the upper and lower fibers are both compressed everywhere, in contrast to what is found for the semicircular arch of Figure 4 (the results are not reported here for brevity).

To evaluate the effects of the mechanical characteristics of the connections between the fibers and the arch, the same semicircular arch of Figure 3 is analyzed. The tangential stiffnesses of the connections () are now considered as variable parameters. When semicircular arches are investigated, the values of these parameters, provided by CNR-DT 200 R1/2012, are considered as upper values. The results presented in Figure 6 show that the longitudinal stresses in the fibers strongly depend on the tangential stiffness of the connections. Specifically, for low values of the upper and lower fibers are compressed everywhere, and only when these parameters are increased do tensile stresses appear. The curves of Figure 6, representing the longitudinal stresses in the fibers, asymptotically tend to the limit curve of the case of rigid tangential connections between the fibers and the arch.

The case of a segmental arch () and constant radius (R=3m) is also studied for different values of the stiffnesses of the tangential connections. Keeping the other geometrical characteristics equal to those of the arch considered in the previous analysis, it is possible to notice the complete absence of tensile stresses across all the stiffness values (Figure 7). The same analysis performed on a segmental arch with 30º of flattening and constant length is conducted (not reported here for brevity). Also in this case, no tensile stresses appear.

In both the previous cases, increasing the tangential stiffness of the connections, the curves representing the longitudinal stresses of the fibers asymptotically tend to the case of perfectly rigid connections between fibers and arch. Moreover, the stresses in the fibers of segmental arches (Figure 7) are less sensitive to the stiffness of the tangential connections, compared to those in the fibers of semicircular arches (Figure 6). In particular, in segmental arches with constant arc length, the stresses in the fibers appear almost independent of the stiffness of the connections.

5.2 Base displacements

In the following analyses, the effects on the behavior of the system of horizontal base displacements, introduced to simulate the displacements of the supports of the arch, are analyzed.

In Figure 8, the longitudinal stresses in the fibers of an arch subject to fill and live loads, together with a base displacement are shown. The same geometrical and mechanical characteristics of the semicircular arch of Figure 4 are considered. The results labelled with  refer to the case already shown in Figure 4, where no base displacements are applied. The presence of base displacements changes the stresses in the fibers. Specifically, the extension of the area and the value of the stresses in the fibers where a tensile stress occurs, increase. As expected, for higher values of displacements, higher values of tensile stresses are found.

Also in the case of a segmental arch with constant radius and with constant arc length the effects of the base displacements are considered (results are not reported here for brevity). Again, the presence of base displacements changes the stresses in the fibers. Even when there is a base displacement  in the segmental arch and the fibers are all compressed, it turns out that the presence of a base displacement is capable of inducing tensile stress in the fibers. Therefore, it is shown that displacements of arch supports cause tensile stresses to build up in the fibers and it is found that the tensile stresses rapidly rise as the displacements increase.

- Comparison between the results of the analytical and computational models

  - Test case

Several comparisons amongst the results of different cases, obtained from the analytical and FE models are performed. For the sake of brevity, only one case is discussed. The test case is one of those already discussed in Section 4. Specifically, the geometrical and the mechanical characteristics of the system are summarized in  Tab.1.

In Figure 9 the stresses in the upper (Nfs) and lower (Nfi) fibers are shown for a semicircular arch (). Solid lines refer to the results obtained from the FE model, while dashed lines refer to the results provided by the analytical model. There is a good agreement between the results provided by these two. The small differences that appear can be attributed to the shear indeformability of the arch in the analytical model, which is not introduced in the FE model. As can be observed, both in the upper and in the lower fibers, small ranges where tensile stresses manifest themselves exist. In particular, the lower fiber is stretched near the boundaries. This is because the fibers are fully constrained at the boundaries.

  - Non-linear behavior of the fibers

The good agreement between the analytical and the FE models allows the use of the FE model to investigate the non-linear behavior of the fibers under the assumption that they do not resist compression. Since the compression strength of the fibers is very small due to stability problems related to their very small thickness, usually, this small contribution is neglected. A non-linear computational analysis is conducted with the aim of understanding whether the conclusions reached from the above linear analyses are still applicable in the nonlinear field.

In Figure 10, comparison is shown between the results obtained from the analytical model, already shown in Figure 9 (dashed lines), and those obtained from the finite element model (solid lines), where the fibers are considered not to be able to resist compression. It is interesting to observe that the non-linear finite element model confirms the results of the linear analytical model. Specifically, the results are only slightly affected by the different constitutive laws of the fibers. The ranges where the fibers are stretched remain substantially unchanged. Only a slight enlargement of these ranges occurs together with a slight increase in the tensile stresses in the fibers.

As observed in Section 4, the ranges where the tensile stresses manifest themselves in the fibers reduce when the degree of flattening of the arch increases. To investigate whether this behavior manifests itself when fibers which are able to resist compression are considered, the case of the segmental arch with  and characteristics as in Tab.1, already considered in Section 4, is studied in detail. In Figure 11 comparison between the results obtained from the analytical model (dashed lines) and those obtained from the finite element model (solid lines), where the fibers are considered not to be able to resist compression, is shown. Also in this case the results obtained from the linear analytical model are substantially confirmed. The constitutive laws of the fibers plays a limited role on the outcome of the analysis. When the resistance to compression is not considered, only a very small range appears where the lower fibers are stretched appears compared with the linear analytical case where only compression stresses exist.

- Conclusions

A linear elastic analytical model of curved beam with constant curvature, coupled with curved strings, has been developed. The arch has been modelled using a classical mono-dimensional polar model of curved planar beam. A continuous distribution of tangential and normal linear elastic springs has been used to model the connections between the strings and the arch. A numerical integration of the field equations has been  performed. The strings considered are intended to represent fiber-reinforcements.

The main objective of this paper is to investigate the role of reinforcing fibers applied to curved beams realized with homogeneous and isotropic materials. However, the results obtained provide useful information for curved masonry structures subject to operational loads, until the low tension strength of the material is reached.

Two types of static excitations of the coupled system, arch-strings, have been taken into account. The first kind represents operational loads (fill loads, live loads), the other one, instead, base displacements of the arch. In the parametric analyses, the stiffness of the tangential connection between strings and arch, and the degree of flattening of the arch, have been taken as main variable parameters. Comparison of the results provided by the analytical model and those obtained from a finite element (FE) model has been performed.

Results show that the use of fiber reinforcement is effective mainly in arches where a base displacement occurs. In this case, fibers are stretched over vast areas of the arch. On the contrary, when the fibers are used to strengthen an arch-like structure, with the aim of improving its behavior under static loads, the fibers do not work as expected. In this case the ranges over which the fibers exhibit tensile stresses are very small. Their amplitude further reduces in segmental arches. In arches with a medium degree of flattening () it is found that tensile stress occurs only in a very small range. These results show that a lot of attention must be devoted to the appropriate use of fiber reinforcement.

Given the good agreement between the analytical and the computational models, the FE model has been used to investigate the case of fibers that do not resist compression. The results obtained show that, when only the tensile strength of the fibers is taken into account, only small quantitative changes in the results occur and, therefore, since no qualitative modifications exist, all the previous conclusions apply.

- Acknowledgment

The authors are very grateful to the ‘Filauro’ Italian Foundation for the economic support which was given to Ing. Sergio Chirivì, during his stay at the University of Bath in the UK.

References

# Briccoli Bati, S., Rovero, L. and Tonietti, U. (2007). “Strengthening Masonry Arches with Composite Materials”, Journal of composites for construction, Vol. 11, No. 1, pp. 33-41.

# Cancelliere, I., Imbimbo, M. and Sacco, E. (2010). “Experimental tests and numerical modeling of reinforced masonry arches”, Engineering Structures, Vol. 32, No. 3, pp. 776-792.

Capozucca, R. (2010). “Experimental FRP/SRP-historic masonry delamination”, Composite structures, Vol. 92, No. 4, pp. 891-903.

# Chen, Y., Ashour, A.F. and Garrity, S.W. (2007). “Modified four-hinge mechanism analysis for masonry arches strengthened with near-surface reinforcement”, Engineering structures, Vol. 29, No. 8, pp. 1864 - 1871.

D’Ambrisi, A., Feo, L. and Focacci, F. (2013). “Experimental and analytical investigation on bond between carbon-FRCM materials and masonry”, Composites Part B: Engineering, Vol. 46, pp. 15-20.

de Leo, A.M., Contento, A. and Di Egidio, A. (2015). “Semi-analitical approach to the study of static behaviour and buckling of shells with single constant curvature”, Continuum Mechanics and Thermodynamics, Special Issue: Nonlinearities, Bifurcation and Instabilities, (DOI 10.1007/s00161-014-0364-7, in press).

de Leo, A.M., Contento, A. and Di Egidio, A. (2014). “Mono-dimensional formulation of axial-symmetric spherical shells and characterization of the linear static behavior”, Meccanica, (submitted).

De Lorenzis, L., Dimitr,i R. and La Tegola, A. (2007). “Reduction of lateral thrust of masonry arches and vaults with FRP composites”, Construction and building materials, Vol. 21, No. 7, pp. 1415-1430.

Marfia, S., Ricamato, M. and Sacco, E. (2008). “Stress analysis of reinforced masonry arches”, International journal for computational method in engineering science and mechanics, Vol. 9, No. 3, pp. 77-90.

# Oliveira, D.V., Basilio, I. and Lourenco, P.B. (2010). “Experimental behavior of FRP strengthened masonry arches”, Journal of composites for construction, Vol. 14, No. 3, pp. 312-322.

CNR-DT 200 R1/2012. “Istruzioni per la Progettazione, l’Esecuzione ed il Controllo di Interventi di Consolidamento Statico mediante l’utilizzo di Compositi Fibrorinforzati”, (in Italian).

List of tables

Tab.1: Geometrical and mechanical characteristics of the test case

| Radius R | 3.0 m |
| --- | --- |
| Section of the arch | 0.30 × 0.50 m2 |
| Thickness of the fibers | 0.378 mm |
| Young modulus of the arch | 3.0 × 106 KN/m² |
| Young modulus of the fibers | 0.19 × 109 KN/m² |
| Tangential stiffness of the connections | 2.0 × 106 KN/m² |
| Fill load | 19.0 KN/m3 |
| Live load | 50.0 KN/m² |

Captions to figures

Figure 1: Kinematics of the coupled system, arch plus strings: (a) Geometrical parameters characterizing the arch; (b) Connection between the strings and the arch; (c) Displacement components.

Figure 2: Statics of the coupled system, arch plus strings: (a) Internal stresses; (b) Shape of the external loads; (c) Total external forces acting on the infinitesimal portion of the beam.

Figure 3: Finite element model.

Figure 4: Deformation and stresses in the fiber-reinforced arch: (a) deformed shape of the arch; (b) normal stress Nt and bending moment Mt  in the arch; longitudinal stress in the upper fiber Nfs and in the lower fiber Nfi ().

Figure 5: Stress in the upper fiber Nfs and the lower fiber Nfi in a segmental arch with R = const ().

Figure 6: Longitudinal stress in the upper (Nfs) and in the lower (Nfi) fibers of a semicircular arch  (   ).

Figure 7: Stress in the upper (Nfs) and in the lower (Nfi) fibers of a segmental arch with R=const ( ).

Figure 8: Effects of the base displacement: (a) Deformation of a semicircular arch subject to operational fill and live loads, and base displacement; (b) longitudinal stresses in the upper (Nfs) and lower (Nfi) fibers in a semicircular arch by varying the displacement d .

Figure 9: Stress in the upper (Nfs) and lower (Nfi) fibers in the test case (, solid lines: finite element model; dashed lines: analytical model).

Figure 10: Stress in the upper (Nfs) and lower (Nfi) fibers in the test case (, solid lines: finite element model with fiber not resisting compression; dashed lines: analytical model).

Figure 11: Stress in the upper (Nfs) and lower (Nfi) fibers in the test case (, solid lines: finite element model with fiber not resisting compression; dashed lines: analytical model).

List of figures

Figure 1

Figure 2

Figure 3

Figure 4

Figure 5

Figure 6

Figure 7

Figure 8

Figure 9

Figure 10

Figure 11