Benefits of coastal managed realignment for society: evidence from ecosystem service assessments in two UK regions

Michael A. MacDonald1, Chris de Ruyck1, Rob H. Field1,2, Alan Bedford3, Richard B. Bradbury 1,2,4

1 RSPB Centre for Conservation Science, The Lodge, Sandy SG19 2DL, United Kingdom

2 RSPB Centre for Conservation Science, The David Attenborough Building, Pembroke Street, Cambridge CB2 3QZ

3 Edge Hill University, St Helens Road, Ormskirk, L39 4QP, UK

4 Conservation Science Group, Department of Zoology, University of Cambridge, Downing Street, Cambridge CB2 3EJ, UK

ABSTRACT

Coastal managed realignment has the potential to deliver both nature conservation and other benefits to people, but these are rarely quantified. We used an ecosystem services assessment toolkit, TESSA, at two locations in the UK; Hesketh Outmarsh West (northwest England), where realignment has already been carried out, and the Inner Firth of Forth (central Scotland), where realignment is proposed for multiple sites. At the Inner Forth we focus on one site, Inch of Ferryton, in Clackmannanshire but also assess ecosystem services across several sites. Using dedicated data collection where possible, and site-appropriate existing data, we estimate the value of these sites to people in the realigned state compared with the most realistic alternative: continued agricultural production behind hard flood defences (agricultural state). Services assessed were climate change mitigation, agricultural production, nature-based recreation, and flood risk protection. At both sites agricultural production was estimated to be greater in the agricultural state, while other services were estimated to be greater in the realigned state. We are cautious about assigning overall monetary values based on biophysical attributes, particularly considering that climate change mitigation is highly sensitive to carbon prices, and that by necessity we were unable to quantify all services. Nevertheless, using a price for carbon that incorporates the societal cost of emissions, we estimate that the net annual provision of services is £262,935 (£1460.75/ha) at Hesketh Outmarsh West and £93,216 (£574.70/ha) at Inch of Ferryton. At both sites, sequestered carbon in accreting sediments outweighs greenhouse gas emissions from intertidal habitats, and the net value of this in turn outweighs the income foregone from crops and grazing. At Hesketh Outmarsh West the value of ecosystem services is increased by the reduction to flood risk arising from the coastal managed realignment action. Nature-based recreation is estimated to increase at both sites under realignment, with the visitor profile expected to be strongly local. Decisions about coastal management, including realignment, should incorporate information about the ecosystem services provided under different scenarios. This applies beyond coastal ecosystems, and the site-scale is often the most appropriate scale to carry out such assessments, as this is the scale at which decisions are often taken.

Keywords

Sediment accretion; Benefits transfer; Restoration; Recreation; Climate regulation; Agricultural production

# 1. Introduction

There have been considerable losses of coastal habitats around the world over recent decades (e.g. 50% of marshes, 35% of mangroves and 30% of reefs lost or degraded; MEA, 2005). In the United Kingdom, for example, large areas of intertidal habitat have been lost due to land claims for industrial development and agriculture (Jones et al., 2011). This is now being exacerbated by inundation due to coastal squeeze, in which intertidal habitat is constrained by the sea on one side and defensive walls on the other; this is having negative impacts on the characteristic biodiversity of the intertidal zone (Doody, 2004; McLusky et al., 1992; Royal Haskoning, 2006; Wolters et al., 2005).

Coastal salt marsh is protected under the European Habitats Directive, and the UK’s Post-2010 Biodiversity Framework aims to prevent net losses to the area of priority habitats, including salt marsh (DEFRA, 2011; JNCC and DEFRA, 2012). To help reverse losses, there have recently been several large coastal managed realignment (MR) projects in the UK that seek to restore intertidal habitat within historic land claim areas. MR is achieved by removing or breaching sea walls so the land behind them floods, allowing the inter-tidal habitat (mudflat, low salt marsh and high salt marsh) to migrate inland. An alternative approach is to install tidal exchange structures within sea-walls, to restore a natural tidal regime to a contained area, re-creating intertidal habitats, including coastal lagoons, in a process known as regulated tidal exchange (RTE).

While loss of intertidal habitat is a biodiversity conservation concern, intertidal habitats also provide other significant ecosystem services (ES), such as carbon sequestration in accreting sediments, flood defenses for coastal communities, hunting of wintering wildfowl, and nursery areas for commercially-caught fish (Boerema et al., 2016; da Silva et al., 2014; Foster et al., 2013); these may make the reintroduction of intertidal habitats attractive to decision makers and the general public. The role of MR is also therefore promoted as ecosystem-based adaptation in the face of climate change (Temmerman et al., 2013). Consideration of natural capital and ES provision can support arguments for conservation-oriented decision making (Balmford et al., 2011; Eastwood et al., 2016; Fisher et al., 2011; Mace, 2014; Mace et al., 2012), and it is increasingly being incorporated into government policy making on land-use (Albon et al., 2014; DEFRA, 2007; Natural Capital Committee, 2015). In 2016, the Natural Capital Coalition launched a protocol intended to act as a standardised framework for organisations to identify, measure and value their reliance and effects on natural capital. However, information about the relationships between ecosystem health, resilience, and service provision is often incomplete (UK National Ecosystem Assessment, 2011). The performance of MR specifically in delivering ES and biodiversity in England has been questioned (Esteves, 2013). Therefore, it is important that evidence for its effects on biodiversity and ES is collected, to inform the most appropriate environmentally, socially and economically sustainable decisions at such sites. We address the evidence gap for the effects of MR on ES in this study by quantifying ES provision at two UK sites under different scenarios, to measure the marginal benefit of carrying out MR and to reveal who gains and who loses from different management approaches. We focus on primary measurement of services wherever possible, to reduce reliance on benefits transfer approaches (da Silva et al., 2014; Luisetti et al., 2014). ES provision by a site in isolation is relatively meaningless, whereas comparing the actual or potential provision under different scenarios provides information that can be used in making management decisions (Balmford et al., 2002). In this study the comparison is between the ES provided under the realigned state and those provided under the most likely alternative land-use at the sites: the maintenance of hard sea defences, behind which relatively intensive agriculture continues (termed the agricultural state).

Fig. 1. Locations mentioned in the text at (a) the Inner Forth Estuary, and (b) Hesketh Outmarsh.

The mature salt marsh sampled at Skinflats is indicated by hatching. The symbol indicating Crossens Marsh is the approximate site location taken from the longitude and latitude in Ford et al. (2012)

2. Study area, materials and methods

This study used the TESSA (Toolkit for Ecosystem Services Site-based Assessment) approach (Blaen et al., 2016; Peh et al., 2013), in which the net benefits of a management intervention are assessed. TESSA is not the only tool available for such assessments (e.g. McInnes and Everard, 2017), but it has the advantage of providing site-scale quantitative assessments, and has been used previously at wetlands (Peh et al., 2014). TESSA focuses on those ES that are considered practical to quantify without excessive cost or expert knowledge, and as such there is possible bias from the exclusion of ES from the assessment (e.g. non-use values). However, in the absence of methods that allow for measurement of all ES, we must rely on such assessments, in the same way that researchers use surveys of a subset of taxa to represent biodiversity (e.g. MacDonald et al., 2012)

2.1 Study area

The sites differ in that one (Hesketh Outmarsh West on the Ribble estuary in north-west England) has already undergone MR; whereas at the other (the Inner Firth of Forth, in central Scotland) MR is projected for several sites. The sites were chosen because they are both candidates for future MR, and the results of this study will help to inform future decisions, and because the RSPB’s involvement in both sites facilitated access for field data collection and contact with site managers and stakeholders.

Across Hesketh Outmarsh as a whole (Fig 1), approximately 350 ha of salt marsh had been converted to agricultural land through a private land claim and the building of an outer sea defence wall. These were completed in the early 1980s, with areas inland continuing to be protected by the inner defence wall, maintained by the Environment Agency (EA). In 2006, the Royal Society for the Protection of Birds (RSPB) acquired 180 ha and a MR project was completed by 2009 by the EA, creating the Hesketh Outmarsh West Nature Reserve. This was to provide compensatory habitat for salt marsh lost elsewhere, to contribute to EA targets for habitat creation, and to support the case for strengthening the inner wall, which was done as part of the same project. The RSPB’s conservation aims include the re-creation of diverse coastal salt marsh habitat. There are plans to similarly realign the remaining 170 ha of the original land claim (Hesketh Outmarsh East), but this study considers the impact of the MR that has already taken place.

Four breaches were made to the outer wall, historic creek beds were dredged, lagoons were dug, and the inner wall was reinforced. In 2012-13 (the time of this assessment), the realigned site was still relatively dynamic, with sediments flowing into the site and infilling the creeks and lagoons, and salt marsh vegetation continuing to develop and mature (Bedford, 2012). Natural salt marsh borders the realigned area to the west and north, and has been grazed extensively by cattle over a long period; grazing by sheep (0.4 livestock units ha-1) has been introduced to the realigned area.

The Inner Forth is that part of the Firth of Forth estuary that is upstream of the Forth Bridges. It has a long history of agricultural and industrial land claims: 45-55% of the inter-tidal area is estimated to have been lost over the past 400 years (Maslen and Hansom, 1999). A small (3.7 ha) part of one site adjacent to the Clackmannan Bridge, Kennet Pans, has already undergone MR as part of the bridge’s construction in 2008 (Fig. 1). Elsewhere, RTE was introduced in 2010 at Skinflats RSPB reserve to allow tidal inundation and salt marsh development on approximately 10 ha; however, the RTE has not functioned as intended and the area is only allowed to flood approximately once a month. Research commissioned by the RSPB identified a further seven sites with potential for MR, totalling approximately 365 ha of new intertidal habitat, including a substantial part of the 162 ha Inch of Ferryton Farm site (RSPB, 2012)(Fig. 1.). Inch of Ferryton has undergone an options appraisal, a cost-benefit analysis and design development for a MR project (ABPmer, 2014); while the other six sites are not indicated on maps here as their candidacy for MR remains debatable. The present study considers Inch of Ferryton as the primary site for assessment, and considers the impact of potential MR at this site on ES. However, climate change mitigation and agricultural production could be estimated for all nine sites combined at the Inner Forth, and so we have done this, while still presenting the results for the Inch of Ferryton separately.

### 2.2 Description of the realigned and agricultural states

At both sites, the realigned state is considered to be one where MR has been carried out, with the subsequent establishment of intertidal habitat, generally salt marsh. At Hesketh Outmarsh West, this is represented by the current nature reserve, whereas at Inch of Ferryton, this is the potential state of the site if MR plans were realized. The same is true of other sites in the Inner Forth, except Kennet Pans and Skinflats, parts of which have already undergone MR or RTE, such that the current situation represents the realigned state. The agricultural state is considered to be continued agricultural production behind hard sea defences. At Hesketh Outmarsh West this assumes that the recent MR had not taken place, and that the land had continued under agricultural production, as is still the case at Hesketh Outmarsh East (Table 1). At the Inner Forth, this agricultural state is the current situation at all sites except for Skinflats and Kennet Pans, where the agricultural state assumes that these two sites had remained behind hard sea defences, as they were prior to the implementation of RTE and MR respectively. Note that this does not mean that 100% of the land at the Inner Forth is currently producing food: some existing intertidal habitat occurs.

Table 1. Area of habitat (ha) in realigned and agricultural states

| Site | Agricultural State |  | Realigned State |  | Total Area |
| --- | --- | --- | --- | --- | --- |
|  | Claimed Farmland | Intertidal Habitat | Claimed Farmland | Intertidal Habitat | Total Area |
| Hesketh Outmarsh West | 180 | 0 | 0 | 180^ | 180 |
| Inner Forth |  |  |  |  |  |
| Inch of Ferryton | 159.1 | 3.1# | 74.8 | 87.4 | 162.2 |
| Skinflats* | 78.9 | 124.2 | 4.9 | 198.2 | 203.1 |
| Kennet Pans* | 17.4 | 17.4 | 9.3 | 25.5 | 34.8 |
| Other sites | 285.6 | 52.4 | 64.8 | 273.2 | 338.0 |
| Total Inner Forth | 541.0 | 197.1 | 153.8 | 584.3 | 738.1 |

* Includes 10 ha already subject to RTE at Skinflats, and 3.7 ha already realigned at Kennet Pans

# All intertidal habitat at Inch of Ferryton in agricultural state is mudflat^

^ All intertidal habitat at Hesketh Outmarsh West in the realigned state is mid-high salt marsh

Areas of different habitats were estimated for the realigned and agricultural states using current habitat and elevation maps (Table 1). Intertidal vegetation communities are affected by elevation, salinity, inundation frequency, flow rate, and sediment load (Allen and Pye, 2009), leading to a graduation of habitat that we place in three classes: mudflat, low salt marsh, and mid-high salt marsh. Hesketh Outmarsh West does not contain any mudflat or low salt marsh, as this occurs to the seaward side of the site under consideration. The sites on the Inner Forth are adjacent to mud flat and currently have various proportions of low (Salicornia-dominated) and mid-high (Puccinellia-dominated) salt marsh (Gauld, 2014). We assumed that mudflat would occur between 0 and 2 m ODN (Ordnance Datum Newlyn) in the realigned areas, low salt marsh between 2 and 2.33 m ODN, and mid-high salt marsh between 2.34 and 4 m ODN (Blott and Pye, 2004), which matched on-site observations. One-metre contour maps were available, and we assumed that 33% of the 2 to 3 m ODN contour would become low salt marsh, and that land above 4 m ODN would become terrestrial habitat (Table 2). Agricultural land at Hesketh Outmarsh West was considered to be arable, as had been the case prior to MR, while at the Inner Forth the current proportion of crops/pasture in East Central sub-region was applied to the area of agricultural land (with the exception that rough grazing was excluded): winter wheat 2.6%, winter barley 2.3%, spring barley 15.7%, oats 7.0%, oilseed rape 0.4%, potatoes 0.01%, other crops 2.7%, grass under 5 years 30.8%, grass over 5 years 38.6% (Scottish Government Directorate for Environment and Forestry, 2014). Grass under five years old was considered to be silage rather than pasture.

### 2.3 Stakeholder engagement

The TESSA approach recommends engagement with stakeholders and the collection of primary data where possible (Peh et al., 2013). We therefore held discussions with site managers and stakeholders (representatives of statutory agencies, academics, local farmers) at Hesketh Outmarsh in October 2012, and the Inner Forth in April 2013. Stakeholders represented both on-site (i.e. local scale) and off-site (i.e. regional to national scale) ES providers and beneficiaries. This process allowed us to: identify the plausible alternative states at our sites; identify the services provided, how they might change in the different states, and who might benefit; and determine the most appropriate location and methods to measure these services in a way that would give locally relevant data. The delivery of the services in the realigned state could then be compared to the delivery in the agricultural state. The realigned and agricultural states are described above, and the services that we assessed are described below. We aimed to determine the stocks and flows of the services at the sites at a point in time, rather than to carry out a full cost-benefit analysis or to include time horizons or discount rates.

Table 2. Predicted additional intertidal habitat (ha) at the Inner Forth in the realigned state

| Site | Additional intertidal area | Mudflat | Low salt marsh | Mid-high salt marsh |
| --- | --- | --- | --- | --- |
| Inch of Ferryton | 84.3 | 52.2 | 10.1 | 22.0 |
| Skinflats | 74.0 | 7.1 | 9.6 | 57.3 |
| Kennet Pans | 8.1 | 7.1 | 0.3 | 0.7 |
| Other sites | 220.8 | 77.7 | 26.4 | 116.7 |
| Total | 387.2 | 144.1 | 46.4 | 196.7 |

2.4 Global climate regulation

Carbon stocks and greenhouse gas fluxes were estimated separately, using a combination of field measurements and reference to published values: fluxes were estimated using an ecosystem budget approach, where net sequestration or emission was the sum of rates of primary production, efflux of gases, and sediment accretion (Andersson and Sobek, 2006).

Stocks of above-ground biomass (AGB) for salt marsh were sampled by cutting all vegetation in 1 m2 quadrats (sample sizes are presented with the results) to ground level, weighing, then drying sub-samples to calculate a conversion factor for the dry mass of the entire sample. Quadrats were sampled in January 2012 at Hesketh Outmarsh West and in June-July 2013 at the Inner Forth. Measured values of carbon stocks for mudflat and salt marsh at Kennet Pans MR site were applied to these habitats at the Inner Forth, with the exception of salt marsh at Skinflats, where we also measured values. Below ground biomass (BGB) for salt marsh was estimated as 70.9% of AGB, (Hussey and Long, 1982). Values for arable land were considered to be zero due to harvesting and cultivation, with the exception of litter, which was assumed to be 0.47 t ha-1 (IPCC, 2006). For pasture we used published values of 1.08 t ha-1 for AGB and 6.58 t ha -1 for BGB (Anderson-Teixeira and DeLucia, 2011). Intertidal mudflat was considered not to hold any AGB or BGB.

Soil organic carbon (SOC) was sampled by taking cores with an auger to a maximum depth of 1 m, and taking three 1 cm3 subsamples from this at standard depths: 10 cm from surface; 10 cm above the base of the organic horizon or maximum auger depth; and a mid-point between these two; only two sub-samples were taken when the organic horizon was less than 20 cm. The depth to the bottom of the organic horizon was measured using metal rods of known length. Soil samples were chilled until analyzed using loss on ignition (440°C) measurements in the laboratory. The organic carbon proportion and bulk density (g/cm3) from each subsample was used to calculate the mean organic carbon density (OCD) for each habitat, and these values and soil organic horizon depth were used to calculate the SOC per hectare for each habitat. Per hectare carbon stocks in AGB, BGB and SOC were calculated for each habitat and these values were used to estimate carbon stocks based on the amount of each habitat predicted to be present. Annual carbon accumulation in salt marsh sediments was calculated using OCD data and site-specific accretion data at Hesketh Outmarsh West (Bedford, 2012) and the Inner Forth (Gauld, 2014).

Net greenhouse gas (GHG: CO2, CH4, N2O) flux values for salt marsh were taken from grazed salt marsh in a study carried out within 5 km of Hesketh Outmarsh West (Ford et al., 2012). Net GHG fluxes for mudflat were taken from mean values of CH4 and N2O values in a study of the Blackwater Estuary in eastern England, giving a value of 0.53 t CO2 eq ha -1 y-1 for MR mudflat and 1.08 t CO2 eq ha -1 y-1 for non-MR mudflat. Carbon sequestration (1.19 t CO2 eq ha -1 y-1) of pasture was an average value derived from several reviews (O'Brien et al., 2014). Emissions of N2O arising from inorganic fertilizer application to agricultural land were estimated to be 0.0117 of the annual nitrogen application (USDA, 1994). Site-specific fertilizer application data were available for Hesketh Outmarsh, while at the Inner Forth, we used crop-specific fertilizer application rates (British Survey of Fertiliser Practice, 2014). Using the proportion of crops present (listed above) we calculated an average emission rate for all farmland of 0.48 t CO2 eq/ha. Values for each habitat were then used to generate site-scale emission rates for the two states, based on the distribution of habitats. Annual methane emissions from livestock were taken to be 0.008 t per sheep, and 0.057 t per cow (IPCC, 2006). Average stock rates for Scotland for 2012-13 were used: sheep 1.17 ha-1, and cattle 1.4 ha-1 (Scottish Government Directorate for Environment and Forestry, 2014), and were applied to pasture only, as livestock emissions on salt marsh are already incorporated into the GHG fluxes for that habitat (Ford et al., 2012). As with fertiliser application, the proportion of land under pasture (38.6%) was used to calculate a farmland emission value of 0.93 t CO2 eq/ha. Values of emissions from livestock, and sequestration and emissions for land cover/use were summed to give a total value of Global Warming Potential (GWP100) for each site in the two states, based on the habitats and animals present, expressed in tonnes CO2 eq y -1, assuming GWP100 values of 1 for 1 t CO2, 25 for 1 t CH4 and 298 for 1 t N2O (Forster et al., 2007). By convention, net atmospheric warming is presented as a positive value and net cooling as a negative value.

2.5 Agricultural production

The revenue derived from farming the sites in the agricultural state was calculated using regional data from the Farm Business Survey for 2013-14 (Rural Business Research, 2016; Scottish Government Directorate for Environment and Forestry, 2014): cereal farms in northwest England for Hesketh Outmarsh West, and general cropping in Scotland for the Inner Forth. The average per ha income value was used, minus the single farm payment and agri-environment subsidies, and multiplied by the area of land that would be farmed in the agricultural state. Using farm business survey data was more practical than determining costs by interviewing farmers, due to sensitivity issues of disclosing financial information of individual farm business.

2.6 Recreation

Data on visitors to Hesketh Outmarsh West pre- and post-MR were collected by RSPB reserve staff using car and stile counter equipment. These counts were used to generate annual visitor numbers for the realigned and agricultural states. Stile counts began in January 2008, and car park counts in March 2008. These counts were used up to September 2008, when the first breach was made. Counts were available for other RSPB sites in the area, and so Hesketh counts were calibrated against those at the Sandgrounders Hide at the Marshside Reserve (October-December 2008 for stile counts; January-March and October-December 2008 for car counts; Fig. 1) to give an annual count of visitors. Post-MR counts were made from October 2011 to September 2012. Data on visitor origins were available from surveys (n = 339; one international visitor was excluded for these purposes) conducted in June 2012 at the nearby (11.7 km distant) Marshside RSPB Reserve, which was considered likely to have a similar visitor profile to Hesketh Outmarsh West. Using postcode data, the mean visitor travel distance (return journey) was multiplied by £0.72 km-1 for visitors travelling by car, which corresponds to the amount that UK HMRC allow to be paid in expenses for business travel, and which incorporates car maintenance and other costs in addition to direct fuel costs. This was used to calculate the mean travel cost per visit, which was then multiplied by the number of annual visitors before and after MR at Hesketh Outmarsh West to give a value for nature based recreation in the realigned and agricultural states.

At the Inner Forth, we limited our analysis of recreation values to Inch of Ferryton, due to the impracticability of assessing these at multiple sites across the landscape. Site visitor surveys (Supplementary Information) were carried out from May to September 2013, and in September 2014, at Bothkennar Pools, a salt marsh lagoon and wooded area with an access trail that is 7 km from Inch of Ferryton, on the southern shore of the Inner Forth (Fig. 1.). This represented a similar site, in terms of habitat, access and facilities, to that expected to be produced by MR at Inch of Ferryton. Again using postcode of origin data, the mean visitor travel spend was calculated as above. Counts of visitors were made on six days in August, and the per hour counts of visitors for this month were scaled up to an eight hour day, and then calibrated against those from the Eden Estuary Centre Observatory for the other months of the year, to calculate an annual visitation rate. The Eden Estuary Centre is approximately 60 km northeast of Inch of Ferryton, and was considered likely to have a similar trend in visitor numbers across the year, as birds visiting in autumn and winter are a key draw for visitors, so that the second half of the year sees more visitors than the first. There were assumed to be no visitors to Inch of Ferryton pre-MR, as there is nothing to attract people and there are no paths across the site.

2.7 Flood and Storm Surge Protection

MR is expected to provide flood protection to the Ribble and Forth estuaries upstream of the sites, by providing extra holding capacity for water. The marginal effect of these realignments on flood risk upstream requires complex hydrological modelling that was beyond this study. However, existing modelled data were available for both sites, and these results are used for this ES assessment. At Hesketh Outmarsh West, where tidal flooding is the major risk, the capacity of the site relative to the estuary’s total tidal volume was calculated (North West & North Wales Coastal Group, 2009). At the Inner Forth, flood risk arising from several management options (do nothing, maintain current situation, maximum MR, constrained MR and RTE) was assessed as part of a wider cost-benefit analysis (ABPmer, 2014).

We also examined local flood risk mitigation potential at Hesketh Outmarsh West, where MR was carried out in conjunction with improvements to the inner seawall, to improve flood protection to 1050 ha of land from 1 in 50 years to 1 in 200 years chance of inundation (Simpson and Harvey, 2005). Salt marsh reduces wave attenuation (Möller and Spencer, 2002), which should reduce the maintenance costs for the improved inner wall; however, we were unable to quantify this reduction. At Hesketh Outmarsh West, the process of funding improvements to the inner wall meant that without the creation of salt marsh (meeting requirements of the Water Framework Directive), this work would not have met the criteria for the Flood Defence Grant in Aid (Environment Agency, 2014). Therefore, although flood protection provided by the inner wall would not usually be considered an ES, we argue that the restored salt marsh does provide an ES by enabling the repair of the inner wall.

The total value of residential property protected by the inner wall was estimated by multiplying the number of properties within the 1050 ha flood risk zone by the average house price in Hesketh Bank over the last 5 years, calculated using the Zoopla.com website (2013). The annual value of agricultural production within the flood risk zone was estimated by using Farm Business Survey data from 2012, assuming that all annual agricultural output would be lost in the event of a flood year. The average farm business income per hectare (excluding subsidies) was multiplied by the area affected. The annual value of the flood wall in the realigned state was calculated as the difference in protection provided by the new wall and the previous wall; i.e. 1/200 of the value subtracted from 1/50 of the value.

2.8 Wildfowling and fish production

Wildfowling is a common activity at Hesketh Outmarsh, where it only occurs adjacent to the MR site, and at the Inner Forth, where wildfowlers have legal access to shoot from the mean high water mark. We consider the value of wildfowling to be primarily recreational: quarry may be eaten, but this is not the main reason for the activity. While interviews with wildfowlers were carried out on site to determine motivation, we were unable to obtain data on the number of wildfowlers, their visit frequency, or the annual take from either area.

Salt marshes are known to provide fish nursery habitat (Colclough et al., 2005; Laffaille et al., 2000). However, quantifying the contribution of MR to human fisheries by dedicated data collection was beyond the scope of the current study, although McLusky et al. (1992) estimated that fish populations in the Forth would be 66% greater if historic land claims during the last 200 years hadn’t occurred. The value of salt marsh for one species of fish has been estimated at one site in the UK (Luisetti et al., 2011), and this has been transferred elsewhere (da Silva et al., 2014), but we were not sufficiently confident to apply this approach to our sites.

# 3. Results

3.1 Carbon Stocks

Values for carbon stocks are shown in Table 3, including values for historic salt marsh at Hesketh Outmarsh West. Estimates of above-ground carbon stocks per ha for salt marsh were similar at Hesketh Outmarsh West and the Inner Forth, and varied between habitats at each site. By far the greatest component (over 98%) of the total carbon stock at the sites is held in the soil. The per-hectare values of soil organic carbon were most strongly determined by the depth of the organic horizon, rather than the carbon concentration and soil bulk density. The net change in carbon stocks at Hesketh Outmarsh West, for the purposes of this study, is the difference between the realigned salt marsh and agricultural land each occupying the 180 ha of the site (i.e. 6 343 t), while the net difference at the Inner Forth is in favour of the agricultural scenario (5 805 t: 1 902 t at Inch of Ferryton alone; Table 3). We estimate if all of Hesketh Outmarsh West was mature salt marsh (as it would have been pre-land claim), it would have current carbon stocks 23 195 t greater than the existing realigned salt marsh.

Table 3. Values of carbon stocks (t) for each habitat type, and total carbon stocks for each site in the realigned and agricultural states. Values in bold are derived from our own data, sample sizes in brackets. Areas for Inch of Ferryton are presented in Tables 1 and 2.

|  | Hesketh Outmarsh West |  |  | Inner Forth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Agricultural Land | Historic Salt Marsh | Realigned Salt Marsh | Agricultural Land1 | Mudflat | RTE (Skinflats) | Realigned SM | Historic SM (Low) | Historic SM (High) |
| AGB (t ha-1) | 0 | 2.50 (20) | 1.47 (25) | 0.42 | 0 | 0.67 (18) | 1.75 (8) | 1.23 (16) | 2.13 (16) |
| BGB (t ha-1) | 0 | 1.77 | 1.04 | 2.54 | 0 | 0.48 | 1.24 | 0.87 | 1.51 |
| Soil OC (t ha-1) | 151.7 (3) | 312.0 (7) | 184.9 (6) | 118 (20) | 107 (3) | 133 (18) | 93 (5) | 431 (34) | 431 (34) |
| Area (realigned state; ha) | 0 | 0 | 180 | 153.8 | 285.1 | 66.9 | 176.2 | 11.7 | 44.4 |
| Area (agricultural state; ha) | 180 | 0 | 0 | 541 | 141 | 0 | 0 | 11.7 | 44.4 |
| Total stock (realigned state; t) | 33 734 |  |  | 99 682 (Inch of Ferryton = 17 826) |  |  |  |  |  |
| Total stock (agricultural state; t) | 27 391 |  |  | 105 487 (Inch of Ferryton = 19 728) |  |  |  |  |  |
| Difference (t) | 6 305 |  |  | -5 805 (Inch of Ferryton = -1 902) |  |  |  |  |  |
| Total stock (180 ha historic saltmarsh) | 56 929 |  |  |  |  |  |  |  |  |

1 Values for AGB and BGB are 0.386 x published values, as pasture is estimated to be 38.6% of agricultural land

3.2 Greenhouse Gas Fluxes

The intertidal habitats present at both Hesketh Outmarsh West and the Inner Forth are estimated to emit more greenhouse gases than the agricultural land against which they are compared (Table 4). This is outweighed by the carbon sequestered due to sediment accretion, except for the historical salt marsh at Hesketh Outmash West, where accretion rates are lower. Per-hectare net emissions due to agriculture are higher at Hesketh Outmarsh West than the Inner Forth due to greater application rates of inorganic fertilizer, and because some of the agricultural land at the Inner Forth is pasture, which sequesters carbon in addition to emitting GHGs. Accretion rates at the Inner Forth are higher than at Hesketh Outmarsh West and the presence of a large amount of intertidal habitat in the agricultural scenario means that the net global warming potential in both states at this site is negative, although Inch of Ferryton in the agricultural state is a net emitter, as very little intertidal habitat is currently present (Table 4). The annual net benefits to climate change mitigation are greater in the realigned state at both Hesketh Outmarsh West (1 005 t CO2 eq), and at the Inner Forth (6 334 t CO2 eq; Inch of Ferryton alone 1 822 t CO2 eq).

3.3 Agricultural production

The Farm Business Survey data for cereal farms in the northwest region (16 farms in sample), found that the average farm business income per hectare (excluding subsidies, but including diversification out of agriculture) was £18 (Rural Business Research, 2016). Therefore, the predicted farm business income generated by arable cropping within the realigned area at Hesketh Outmarsh West is £3,240.

Table 4. Fluxes (combination of emission and sequestration) of greenhouse gases (CO2 equivalent/year) at Hesketh Outmarsh and Inner Forth in different habitats and different states. Net sequestration is given as negative values, net emission as positive values.

|  | Hesketh Outmarsh West |  |  | Inner Forth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Agricultural Land | Historic Salt Marsh | Realigned Salt Marsh | Agricultural Land | Mudflat (MR) | Mudflat (non-MR) | RTE (Skinflats) | Realigned SM | Historic SM (Low) | Historic SM (High) |
| Sources of fluxes |  |  |  |  |  |  |  |  |  |  |
| Emissions from land (t ha y-1) | 0 | 20.9 | 20.9 | -0.461 | 0.53 | 1.08 | 20.9 | 20.9 | 20.9 | 20.9 |
| Emissions from fertiliser (t ha y-1) | 2.71 | 0 | 0 | 0.48 | 0 | 0 | 0 | 0 | 0 | 0 |
| Emissions from livestock (t ha y-1) | 0 | 0 | 0 | 0.93 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sequestration from sediment accretion (t ha y-1) | 0 | -10.88 | -23.77 | 0 | -29.9 | -29.9 | -29.7 | -25.9 | -40.9 | -40.9 |
| Area (realigned state; ha) | 0 | 0 | 180 | 153.8 | 144.1 | 141 | 66.9 | 176.2 | 11.7 | 44.5 |
| Area (agricultural state; ha) | 180 | 0 | 0 | 541 | 0 | 141 | 0 | 0 | 11.7 | 44.5 |
| Total GWP (realigned state; t CO2 eq y-1) | -517 |  |  | -11 006 (Inch of Ferryton = -1 760) |  |  |  |  |  |  |
| Total emission (agricultural state; t CO2 eq y-1) | 488 |  |  | -4 672 (Inch of Ferryton = 62) |  |  |  |  |  |  |
| Difference | 1 005 |  |  | 6 334 (Inch of Ferryton = 1 822) |  |  |  |  |  |  |

1 This value represents the pasture sequestration rate of 1.19 t ha-1 (O'Brien et al., 2014) multiplied by the proportion of agricultural land that is estimated to be pasture (38.6%)

This value is forfeited in the realigned area. For the Inner Forth, the average annual income (excluding subsidies, including diversification) of general cropping farms was £87.64 ha-1 (Scottish Government Directorate for Environment and Forestry, 2014), and so the predicted income for the remaining farmland in the realigned state is £13,479, compared with £47,411 for the agricultural state, a difference of £33,931. For Inch of Ferryton alone, these figures are £6,555 and £13,944 respectively, a difference of £7,389.

3.4 Recreation

The annual number of car visits to Hesketh Outmarsh West more than doubled after MR (from 2 158 to 4 949), while the number of stile counts nearly halved (from 1 014 to 596). Making the conservative assumption that each car contained a single visitor, the total annual number of visits increased by 2 373. Postcode data from Marshside Reserve indicated a mean travel spend per car visit of £18.27, so that the estimated annual value of the site in terms of travel cost (vehicle travel only) rose from £39,448 to £90,468.

Thirty-nine interviews were conducted at Bothkennar Pools on the Firth of Forth. Annual visit numbers based on counts of visitors were estimated to be 2 562, which we assumed would be similar to the number of visits received by the Inch of Ferryton. Using the travel costs for those arriving by car (n=34) or bus (n=2), indicated a mean travel spend of £3.71, or an annual total of £9,505 for all visitors. Inch of Ferryton site is currently (i.e. in the agricultural state) considered to receive no visitors for the reasons described above, and therefore the value for recreation is zero.

3.5 Flood and Storm Surge Protection

At Hesketh Outmarsh West, the annual value of flood protection provided by the improved flood defenses to residential property and agricultural production within the flood risk zone was estimated to be £164,905 (Table 5). This assumes that in the case of a flood all properties would become uninhabitable. MR at Hesketh Outmarsh West is estimated to increase the tidal capacity of the Ribble Estuary by 0.1% on a mean spring tide, and by 1.2 % on a tide reaching the level of highest astronomical tide (North West & North Wales Coastal Group, 2009). The modelled potential flood risk benefits of flood storage by intertidal habitats at Inch of Ferryton are positive, though minor: reductions in absolute high water levels of 2-3 cm downstream and 3-5 cm upstream in 1:200 year surge events, and more modest benefits for other events (ABPmer, 2014). The MR options considered include construction of new 1:25 year standard embankments, but unlike at Hesketh Outmarsh West, approval for their construction is not tied to the re-creation of inter-tidal habitat; therefore we do not assess their contribution to flood alleviation as an ES.

Table 5. Annual value of flood defences in the realigned state, Hesketh Outmarsh

| # Houses | Average House Price 2008-2012 | Total Value | Annual flood risk with old defences | Annual flood risk with new defence | Difference |
| --- | --- | --- | --- | --- | --- |
| 62 | £175 470 | £10 879 140 | £217 583 | £54 396 | £163 187 |
| Agricultural land (ha) | Annual net income (per ha) | Total net income | Annual flood risk with old defences | Annual flood risk with new defence | Difference |
| 1000 | £159 | £159 000 | £2 513 | £795 | £1 718 |

3.6 Summary

Provision of ES was greater in the realigned state than agricultural state, with the exception of agricultural production (Fig. 2). Proportionally, the greatest differences were agricultural production and flood risk protection at Hesketh Outmarsh West, and global climate mitigation and recreation at the Inner Forth. Carbon stocks, which arise from differences in carbon accretion and sequestration, were lower in the realigned state at the Inner Forth, though not at Hesketh Outmarsh West.

(a)

(b)

Fig. 2. Proportional differences (relative to realigned state, which = 100%) in ES provision at (a) Hesketh Outmarsh West and (b) Inch of Ferryton

(C stocks = t C; GHG flux = t CO2 eq y-1; Agricultural production = net annual revenues in £; Flood risk = annual value of property protection; Recreation = visits y-1.)

In order to estimate the value of climate change mitigation a range of prices are available on emissions trading markets. Using the average trading prices of the EU emissions trading scheme for 2013 (€4.00 t-1 CO2 eq, or £3.40 on average exchange rates; CCC, 2014) gives a net annual value of £3,417 at Hesketh Outmarsh West and £21,536 at the Inner Forth (£6,195 at the Inch of Ferryton). However, these prices do not represent the societal costs of emissions, and carbon-pricing guidance from the UK government suggests using the non-traded price for carbon, and recommends a range of £25-75 t-1CO2 eq (DECC 2010), which we consider to be more appropriate for an ES assessment. Taking the mid-point of the guide prices gives an annual net value of MR of £50,250 at Hesketh Outmarsh West, and £316,700 at the Inner Forth (£91,100 at Inch of Ferryton). Including other ES, we estimate a marginal monetary value of £262,935 for Hesketh Outmarsh West (180 ha site) and £93,216 for Inch of Ferryton (162 ha site).

4. Discussion

Assessing these two intertidal sites demonstrates the versatility of using TESSA as a framework for assessment of the relative merits of different land-use decisions (Peh et al., 2013), as at one site the management intervention has already been carried out, while at the other the assessment is largely forward-looking. This assessment of the ES at these two estuarine sites used a combination of dedicated primary data collection, existing on-site data, and appropriate values from similar habitats and sites, to provide an assessment of the effects of MR. We have focused on the biophysical values, because monetary values for these will change over time, though we recognise that money is a useful common unit to compare different services. The choice of ES to assess was guided both by decisions regarding which were provided by the sites in each state, and by considerations of those that were practical to quantify. There are others, such as wildfowling and nutrient burial in sediments, that we did not feel able to quantify; their absence would affect the single figure attached to ES in this assessment. However, as discussed below, we feel that the disaggregated quantification of the individual ES is likely to be more important.

We focussed where possible on collection and use of existing primary data to determine values at our sites. Though use of benefits transfer approaches would have enabled us to consider a wider range of services and associated benefits, we decided to limit the range of services in our assessment because of the well-known issues (such as commodity inconsistency and lack of site similarity) that can introduce errors into benefits transfer (e.g. Johnston and Rosenberger, 2010). In particular, we were not confident that, for services such as the nursery grounds benefit to fish production, function transfers had been sufficiently well elucidated to capture criteria for site similarity and therefore to enable transfer validity (Luisetti et al., 2014).  Other assessments of the merits of MR have assigned values to a site largely or entirely by benefits transfer, which can  lead to higher estimates of value. For example, a marginal value of ES of £491,155 to £913,752 was estimated for the Steart Peninsula MR, a site of 470 ha, where 252 ha of intertidal habitat will be created (da Silva et al., 2014). On the Dutch-Belgian border, the net value of ES provided by intertidal habitat created by MR was valued at €35 000/ha, with large contributions to this value from nitrogen removal (Boerema et al., 2016). By contrast, our estimate of the net annual provision of services, using the same carbon price as da Silva et al. (2014) is £262,935 (£1460.75/ha) at Hesketh Outmarsh West and £93,216 (£574.70/ha) at Inch of Ferryton. The former value is higher due to the reduction in flood risk provided by the improved embankment, which we consider an ES in this assessment. It is also important to note that ES provision may not scale linearly with area.

Despite our concerns about the use of benefits transfer across a broader range of services, we did use it to provide estimates of GHG fluxes and some of the carbon stocks. This was partly practical: we did not have resources to measure GHG fluxes directly at our sites. However, existing literature (e.g. Luisetti et al., 2014) also suggests that biophysical estimates for carbon stock and GHG fluxes in saltmarsh systems can be reasonably transferable within a country (though this becomes more challenging at larger scales). We could therefore have chosen to use benefits transfer for all carbon stock estimates; however, we collected primary data where possible in order to minimise errors in our estimates. We did this because other site-based ES assessments have found that carbon values were critically important in determining the aggregate financial value of sites (Birch et al., 2014; Blaen et al., 2015; Peh et al., 2014). Error in carbon values is introduced by measurement error in the biophysical measurements, and in the range of carbon market prices. The greater the uncertainty associated with biophysical estimates, the less easy to determine how economically valuable a strategy may be. Intertidal sediment accretion is dynamic and site-dependent, such that while applying values from other sites is not wrong (our accretion values fall within the same ranges as those from other realigned sites, e.g. Burden et al., 2013; Spencer et al., 2012), the use of site-specific data in this study is a strength.

The majority of our measurements were of flows of services. Carbon stock is an exception to this. Though many mainstream economists would reject the inclusion in an analysis like this of stock values, they can be compared at two points in time, or between two alternative states, as a heuristic to help appreciate the change in natural capital stock that, in this case, results over time from the balance of GHG fluxes and carbon accretion (Luisetti et al., 2014). There were contrasting patterns in the carbon stocks at the two sites. The realigned scenario at Hesketh Outmarsh West held more soil carbon than the agricultural scenario, primarily due to a deeper organic soil horizon. At the Inner Forth, however,  the carbon content of agricultural soil was greater than that recorded in the newly-realigned site at Kennet Pans, as the latter had undergone sediment loss post-MR. It could be argued that it would have been more appropriate to use carbon stock values from mature salt marsh, as this is the desired end product of MR, and these were more than twice as great as those of the agricultural state at Hesketh Outmarsh West. However, this was rejected on three counts. First, the approach was intended to be conservative in estimating the benefits of nature-friendly management, so that the claims of such management are not undermined by suggestions of inflated estimates. Second, adjusting carbon stocks would result in adjusting accretion rates, as these also differ between newly-formed and mature salt marsh (Bedford, 2012). Third, the habitats provided by MR may indeed not be equivalent to mature salt marsh, at least from an ecological point of view (Mossman et al., 2012), and possibly in biophysical characteristics (Burden et al., 2013; Garbutt and Wolters, 2008). Given this, we expect that over the longer term the expected outcome in terms of carbon stocks may lie somewhere between newly-realigned and historically-undisturbed salt marsh.

The two sites in the current study demonstrate the different paths that MR may take, even at sites where MR has taken place relatively recently. At the Inner Forth, our data suggest that there is a loss of carbon from newly-realigned areas due to initial soil loss. For this reason, our carbon stock estimates for mudflat may appear to be low. However, the accretion process means that at current rates this loss of carbon should be replaced within six years; from then on the site will be in credit, even without taking into account the avoided emissions from reduction of agricultural activities. By contrast, at Hesketh Outmarsh West, where the realigned area has been protected by surrounding mature salt marsh, the carbon stocks are already greater than would be the case if the site had remained agricultural land, and are still accumulating carbon in the soil by accretion. Over time, these accretion rates are likely to decline, approaching those of the mature salt marsh, where the rate is less than half that of the realigned salt marsh (Bedford, 2012). When this occurs, the salt marsh is likely to be a net emitter of GHG, which highlights the sensitivity of assessments such as these to the selection of appropriate data. The importance of successional, compared to mature, intertidal habitats in providing ES has been highlighted elsewhere (Boerema et al., 2016). We used values of GHG flux for salt marsh that were measured very close to Hesketh Outmarsh West (Ford et al., 2012). However, these efflux values are much higher than those measured elsewhere in the UK (Andrews et al., 2006; Blackwell et al., 2010), which in turn are higher than those found in other studies, where more saline salt marshes may only release small amounts of these gases or even be sinks in some circumstances (Adams et al., 2012; Chmura et al., 2011; Weston et al., 2014). Therefore, it is possible that the net benefit in terms of sequestration may be higher than the figures we report; again, we take a conservative approach to estimating the benefits of MR.

The flood protection benefits of the MR at Hesketh Outmarsh West are considerable. As previously stated, flood barriers would not usually be considered an ES; however, at this site the provision of salt marsh habitat contributed to the justification for improving the inner defences. There is an additional, unquantified, reduction in maintenance costs for the inner seawall due to the presence of additional salt marsh. At the Inner Forth, minor benefits to flood risk would arise from extra capacity to hold water at Inch of Ferryton, should MR occur there. Similarly minor benefits arise from the increased holding capacity of the realigned area at Hesketh Outmarsh West.

Other studies of the recreation value of realigned sites, including Inch of Ferryton itself, have predicted higher values, but estimates of visitor numbers were not based on local data (ABPmer, 2014; da Silva et al., 2014; Everard, 2009). The estimated benefits in the current study are more modest, possibly because of the limited facilities present at both Hesketh Outmarsh West and at the projected Inch of Ferryton MR site. It may also reflect conservative assumptions in estimating visitor numbers to a potential Inch of Ferryton visitor site, based on surveys at Bothkennar Pools. Nevertheless, this site is likely to be a valuable resource for local people: none of the Bothkennar Pools interview respondents lived further than 40 km away. At Hesketh Outmarsh West there appears to be some indication that the nature of visitors has changed, with more people arriving by car. This may also reflect changes to access to the site, with there no longer being a circular walking route around the perimeter of the site. The visitor profile at this site is likely to remain strongly local, as 76% of visitors to Marshside Reserve were day visitors from their home. Visitors travelling by car from greater distances provide higher monetary values for site recreation, but the importance of the site to local people should not be underestimated. The count of visits, which is much higher in the realigned state in both cases, is probably a better measure of the sites’ worth than the travel cost estimate, which measures the minimum value that a visitor assigns to the site.

5. Conclusion

In this study we have attempted to document the marginal differences in provision of a range of ES resulting from the decision to realign the coast, using site-specific data where possible. The approach we have taken may appeal to others wishing to provide evidence for decision-makers: relative to the resource requirements of projects, dedicated field measurements can be done both quickly and cheaply, assuming a general understanding of the science involved, but no specific expertise. It was not intended to be a full cost-benefit analysis; something that has been done at the Inch of Ferryton site in the Inner Forth in any case (ABPmer, 2014), again using estimates drawn from a wide range of sources, and elsewhere using data on relatively few services (Luisetti et al., 2011; Shepherd et al., 2007; Turner et al., 2007).

Decisions regarding MR are context and scale-dependent and are likely to be made for many reasons: political, ecological, pragmatic. A single bottom line figure that incorporates the monetary value of services may be of limited use, as the monetary value of climate change mitigation is likely to dwarf that of other services, given the range of carbon market values and the relative size of GHG fluxes compared with other service flows. National food security may trump other concerns, as may the provision of a nature reserve to a community otherwise deprived of recreational resources. At both our study sites, the main service foregone is the provision of agricultural goods when farmland is inundated. Financially, this loss is outweighed by the benefits of other services provided by the intertidal habitat. The beneficiaries of the different services will differ, from the global population in the case of climate change mitigation, to relatively local groups in the case of recreation and flood risk prevention, to potentially a handful of individual farmers, although it can be argued that food production provides wider benefits to society. Conversely, a project that provides benefits to society at large (e.g. in the form of flood protection) may have high costs for individual stakeholders (e.g. farmers who lose productive land). This in turn requires decision-makers to consider how to compensate those who may lose out financially, or who provide flows of ES through management that doesn’t maximize their financial return.

The results of this assessment can help to inform decision making in coastal areas of the UK; in particular to support MR as providing benefits to people as well as to nature. The absolute figures provided here are derived from conservative assumptions, in order not to create false arguments for MR, but the direction of changes in ES provision strengthens the case for the plurality of societal benefits that might be realised from planned and targeted MR.

6. Acknowledgments

RSPB site managers and local staff facilitated the carrying out of this work, particularly Tony Baker, David Anderson and Yvonne Boles. Jethro Gauld provided sediment accretion data for the Inner Firth of Forth. Ranald Strachan at the Eden Estuary Centre provided visitor data for calibration. Two reviewers provided comments that improved this article.

7. References

ABPmer, 2014. Inch of Ferryton Habitat Creation Options Appraisal Study: Stage 1 Final Report.

Adams, C.A., Andrews, J.E., Jickells, T., 2012. Nitrous oxide and methane fluxes vs. carbon, nitrogen and phosphorous burial in new intertidal and saltmarsh sediments. Science of the Total Environment 434, 240-251.

Albon, S., Balana, B., Brooker, R., Eastwood, A., 2014. A systematic evaluation of Scotland’s Natural Capital Asset Index. Scottish Natural Heritage Commissioned Report No. 751.

Allen, J.R.L., Pye, K., 2009. Coastal Saltmarshes: Their nature and Importance, in: Allen, J.R.L., Pye, K. (Eds.), Saltmarshes, Morphology conservation and Engineering Significance. Cambridge University Press, New York.

Anderson-Teixeira, K.J., DeLucia, E.H., 2011. The greenhouse gas value of ecosystems. Global Change Biology 17, 425-438.

Andersson, E., Sobek, S., 2006. Comparison of a mass balance and an ecosystem model approach when evaluating the carbon cycling in a lake ecosystem. Ambio 35, 476-483.

Andrews, J.E., Burgess, D., Cave, R.R., Coombes, E.G., Jickells, T.D., Parkes, D.J., Turner, R.K., 2006. Biogeochemical value of managed realignment, Humber estuary, UK. Science of the Total Environment 371, 19-30.

Balmford, A., Bruner, A., Cooper, P., Costanza, R., Farber, S., Green, R.E., Jenkins, M., Jefferiss, P., Jessamy, V., Madden, J., Munro, K., Myers, N., Naeem, S., Paavola, J., Rayment, M., Rosendo, S., Roughgarden, J., Trumper, K., Turner, R.K., 2002. Economic reasons for conserving wild nature. Science 297.

Balmford, A., Fisher, B., Green, R., Naidoo, R., Strassburg, B., Kerry Turner, R., Rodrigues, A.L., 2011. Bringing Ecosystem Services into the Real World: An Operational Framework for Assessing the Economic Consequences of Losing Wild Nature. Environmental and Resource Economics 48, 161-175.

Bedford, A., 2012. Hesketh Out Marsh West Monitoring Report 3 2010/11. Report to the Royal Society for the Protection of Birds. Edge Hill University.

Birch, J.C., Thapa, I., Balmford, A., Bradbury, R.B., Brown, C., Butchart, S.H.M., Gurung, H., Hughes, F.M.R., Mulligan, M., Pandeya, B., Peh, K.S.H., Stattersfield, A.J., Walpole, M., Thomas, D.H.L., 2014. What benefits do community forests provide, and to whom? A rapid assessment of ecosystem services from a Himalayan forest, Nepal. Ecosystem Services 8, 118-127.

Blackwell, M.S.A., Yamulki, S., Bol, R., 2010. Nitrous oxide production and denitrification rates in estuarine intertidal saltmarsh and managed realignment zones. Estuarine, Coastal and Shelf Science 87, 591-600.

Blaen, P.J., Jia, L., Peh, K.S.H., Field, R.H., Balmford, A., MacDonald, M.A., Bradbury, R.B., 2015. Rapid Assessment of Ecosystem Services Provided by Two Mineral Extraction Sites Restored for Nature Conservation in an Agricultural Landscape in Eastern England. PLoS ONE 10, e0121010.

Blaen, P.J., MacDonald, M.A., Bradbury, R.B., 2016. Ecosystem services provided by a former gravel extraction site in the UK under two contrasting restoration states. Conservation and Society 14, 48-56.

Blott, S.J., Pye, K., 2004. Application of lidar digital terrain modelling to predict intertidal habitat development at a managed retreat site: Abbotts Hall, Essex, UK. Earth Surface Processes and Landforms. Earth Surface Processes and Landforms 29, 893-905.

Boerema, A., Geerts, L., Oosterlee, L., Temmerman, S., Meire, P., 2016. Ecosystem service delivery in restoration projects: the effect of ecological succession on the benefits of tidal marsh restoration. Ecology and Society 21, Article No. 10.

British Survey of Fertiliser Practice, 2014. Fertiliser use on farm crops for crop year 2013.

Burden, A., Garbutt, R.A., Evans, C.D., Jones, D.L., Cooper, D.M., 2013. Carbon sequestration and biogeochemical cycling in a saltmarsh subject to coastal managed realignment. Estuarine, Coastal and Shelf Science 120, 12-20.

CCC, 2014. Meeting Carbon Budgets – 2014 Progress Report to Parliament. Committee on Climate Change July 2014, London.

Chmura, G.L., Kellman, L., Guntenspergen, G.R., 2011. The greenhouse gas flux and potential global warming feedbacks of a northern macrotidal and microtidal salt marsh. Environmental Research Letters 6.

Colclough, S., Fonseca, L., Astley, T., Thomas, K., Watts, W., 2005. Fish utilisation of managed realignments. Fisheries Management and Ecology 12, 351-360.

da Silva, L.V., Everard, M., Shore, R.G., 2014. Ecosystem services assessment at Steart Peninsula, Somerset, UK. Ecosystem Services 10, 19-34.

DEFRA, 2007. An introductory guide to valuing ecosystem services. DEFRA, London.

DEFRA, 2011. Biodiversity 2020: A strategy for England’s wildlife and ecosystem services.

Doody, J.P., 2004. ‘Coastal squeeze’— an historical perspective. Journal of Coastal Conservation 10, 129-138.

Eastwood, A., Brooker, R., Irvine, R.J., Artz, R.R.E., Norton, L.R., Bullock, J.M., Ross, L., Fielding, D., Ramsay, S., Roberts, J., Anderson, W., Dugan, D., Cooksley, S., Pakeman, R.J., 2016. Does nature conservation enhance ecosystem services delivery? Ecosystem Services 17, 152-162.

Environment Agency, 2014. Calculate Grant in Aid funding for flood and coastal erosion risk management projects. Guidance for risk management authorities.

Esteves, L.S., 2013. Is managed realignment a sustainable long-term coastal management approach? Journal of Coastal Research 65 (Sp. 1), 933-938.

Everard, M., 2009. Ecosystem services case studies. Environment Agency, Bristol.

Fisher, B., Bradbury, R., Andrews, J., Ausden, M., Bentham-Green, S., White, S., Gill, J., 2011. Impacts of species-led conservation on ecosystem services of wetlands: understanding co-benefits and tradeoffs. Biodiversity and Conservation 20, 2461-2481.

Ford, H., Garbutt, A., Jones, L., Jones, D.L., 2012. Methane, carbon dioxide and nitrous oxide fluxes from a temperate salt marsh: Grazing management does not alter Global Warming Potential. Estuarine Coastal and Shelf Science 113, 182-191.

Forster, P., Ramaswamy, V., Artaxo, P., Berntsen, T., Betts, R., Fahey, D.W., Haywood, J., Lean, J., Lowe, D.C., Myhre, G., 2007. Changes in atmospheric constituents and in radiative forcing. Chapter 2, Climate Change 2007. The Physical Science Basis.

Foster, N.M., Hudson, M.D., Bray, S., Nicholls, R.J., 2013. Intertidal mudflat and saltmarsh conservation and sustainable use in the UK: A review. Journal of Environmental Management 126, 96-104.

Garbutt, A., Wolters, M., 2008. The natural regeneration of salt marsh on formerly reclaimed land. Applied Vegetation Science 11, 335-344.

Gauld, J.G., 2014. Evaluating the Impact of Saltmarsh Restoration upon Carbon Sequestration in Coastal Ecosystems. MSc Thesis, University of Edinburgh.

Hussey, A., Long, S.P., 1982. Seasonal changes in weight of above-ground and below-ground vegetation and dead plant material in a salt marsh at Colne Point, Essex. Journal of Ecology 70, 757-771.

IPCC, 2006. 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Intergovernmental Panel on Climate Change.

JNCC and DEFRA, 2012. UK Post-2010 Biodiversity Framework. JNCC and DEFRA (on behalf of the Four Countries’ Biodiversity Group).

Johnston, R.J., Rosenberger, R.S., 2010. Methods, trends and controversies in contemporary benefit transfer. Journal of Economic Surveys 24, 479-510.

Jones, L., Angus, A., Cooper, A., Doody, P., Everard, M., Garbutt, A., Gilchrist, P., Hansom, J., Nicholls, R., Pye, K., Ravenscroft, N., Rees, S., Rhind, P., Whitehouse, A., 2011. Coastal Margins, in: Assessment, U.N.E. (Ed.), The UK National Ecosystem Assessment Technical Report, UNEP-WCMC, Cambridge.

Laffaille, P., Feunteun, E., Lefeuvre, J.C., 2000. Composition of Fish Communities in a European Macrotidal Salt Marsh (the Mont Saint-Michel Bay, France). Estuarine, Coastal and Shelf Science 51, 429-438.

Luisetti, T., Turner, R.K., Bateman, I.J., Morse-Jones, S., Adams, C., Fonseca, L., 2011. Coastal and marine ecosystem services valuation for policy and management: Managed realignment case studies in England. Ocean and Coastal Management 54, 212-224.

Luisetti, T., Turner, R.K., Jickells, T., Andrews, J., Elliott, M., Schaafsma, M., Beaumont, N., Malcolm, S., Burdon, D., Adams, C., Watts, W., 2014. Coastal Zone Ecosystem Services: From science to values and decision making; a case study. Science of the Total Environment 493, 682-693.

MacDonald, M.A., Maniakowski, M., Cobbold, G., Grice, P.V., Anderson, G.Q.A., 2012. Effects of agri-environment management for stone curlews on other biodiversity. Biol. Conserv. 148, 134-145.

Mace, G.M., 2014. Whose conservation? Science 345, 1558-1560.

Mace, G.M., Norris, K., Fitter, A.H., 2012. Biodiversity and ecosystem services: a multilayered relationship. Trends in ecology & evolution 27, 19-26.

Maslen, J., Hansom, J., 1999. Use of GIS to map land claim and identify potential areas for coastal managed realignment in the Forth Estuary.  Management report. Confidential report by GeoWise Ltd. & Coastal Research Group, Glasgow University to Scottish Natural Heritage. SNH, Edinburgh.

McInnes, R.J., Everard, M., 2017. Rapid Assessment of Wetland Ecosystem Services (RAWES): An example from Colombo, Sri Lanka. Ecosystem Services 25, 89-105.

McLusky, D.S., Bryant, D.M., Elliott, M., 1992. The impact of land-claim on macrobenthos, fish and shorebirds on the Forth Estuary, eastern Scotland. Aquatic Conservation: Marine and Freshwater Ecosystems 2, 211-222.

MEA, 2005. Millennium Ecosystem Assessment: Chapter 19 – Coastal Systems.  . World Resources Institute, Washington, DC.

Möller, I., Spencer, T., 2002. Wave dissipation over macro-tidal saltmarshes: effects of marsh edge typology and vegetation change. Journal of Coastal Research 36, 506-521.

Mossman, H.L., Davy, A.J., Grant, A., 2012. Does managed coastal realignment create saltmarshes with ‘equivalent biological characteristics’ to natural reference sites? J. Appl. Ecol. 49, 1446-1456.

Natural Capital Committee, 2015. The State of Natural Capital: Protecting and Improving Natural Capital for Prosperity and Wellbeing. Third report to the Economic Affairs Committee. Natural Capital Committee.

North West & North Wales Coastal Group, 2009. North West England and North Wales Shoreline Management Plan SMP2.

O'Brien, D., Capper, J.L., Garnsworthy, P.C., Grainger, C., Shalloo, L., 2014. A case study of the carbon footprint of milk from high-performing confinement and grass-based dairy farms. Journal of Dairy Science 97, 1835-1851.

Peh, K.S.-H., Balmford, A., Bradbury, R.B., Brown, C., Butchart, S.H.M., Hughes, F.M.R., Stattersfield, A., Thomas, D.H.L., Walpole, M., Bayliss, J., Gowing, D., Jones, J.P.G., Lewis, S.L., Mulligan, M., Pandeya, B., Stratford, C., Thompson, J.R., Turner, K., Vira, B., Willcock, S., Birch, J.C., 2013. TESSA: A toolkit for rapid assessment of ecosystem services at sites of biodiversity conservation importance. Ecosystem Services 5, 51-57.

Peh, K.S.H., Balmford, A., Field, R.H., Lamb, A., Birch, J.C., Bradbury, R.B., Brown, C., Butchart, S.H.M., Lester, M., Morrison, R., Sedgwick, I., Soans, C., Stattersfield, A.J., Stroh, P.A., Swetnam, R.D., Thomas, D.H.L., Walpole, M., Warrington, S., Hughes, F.M.R., 2014. Benefits and costs of ecological restoration: Rapid assessment of changing ecosystem service values at a UK wetland. Ecology and Evolution 4, 3875-3886.

Royal Haskoning, 2006. Coastal squeeze, saltmarsh loss and Special Protection Areas. English Nature Research Reports No 710, Peterborough.

RSPB, 2012. Inner Forth Futurescape, Feasibility Study. Final Report.

Rural Business Research, 2016. Farm Business Survey 2013/14.

Scottish Government Directorate for Environment and Forestry, 2014. Economic Report on Scottish Agriculture 2014 Edition.

Shepherd, D., Burgess, D., Jickells, T., Andrews, J., Cave, R., Turner, R.K., Aldridge, J., Parker, E.R., Young, E., 2007. Modelling the effects and economics of managed realignment on the cycling and storage of nutrients, carbon and sediments in the Blackwater estuary UK. Estuarine, Coastal and Shelf Science 73, 355-367.

Simpson, C., Harvey, R., 2005. Hesketh Out Marsh West Realignment Scheme: Scoping Report. Environment Agency.

Spencer, T., Friess, D.A., Moller, I., Brown, S.L., Garbutt, R.A., French, J.R., 2012. Surface elevation change in natural and re-created intertidal habitats, eastern England, UK, with particular reference to Freiston Shore. Wetlands Ecology and Management 20, 9-33.

Temmerman, S., Meire, P., Bouma, T.J., Herman, P.M.J., Ysebaert, T., de Vriend, H.J., 2013. Ecosystem-based coastal defence in the face of global change. Nature 504, 79-83.

Turner, R.K., Burgess, D., Hadley, D., Coombes, E., Jackson, N., 2007. A cost–benefit appraisal of coastal managed realignment policy. Global Environmental Change 17, 397-407.

UK National Ecosystem Assessment, 2011. The UK National Ecosystem Assessment Technical Report. UNEP-WCMC, Cambridge.

USDA, 1994. Inventory Of U.S. Greenhouse Gas Emissions And Sinks: 1990-1993, EPA-230-R-94-014. U.S. Environmental Protection Agency, Office of Policy, Planning and Evaluation, Washington, DC.

Weston, N.B., Neubauer, S.C., Velinsky, D.J., Vile, M.A., 2014. Net ecosystem carbon exchange and the greenhouse gas balance of tidal marshes along an estuarine salinity gradient. Biogeochemistry 120, 163-189.

Wolters, M., Bakker, J.P., Bertness, M.D., Jefferies, R.L., Möller, I., 2005. Saltmarsh erosion and restoration in south-east England: squeezing the evidence requires realignment. J. Appl. Ecol. 42, 844-851.