# Metabolic and Molecular Imaging with Hyperpolarised Tracers

Jason Graham Skinner1, Luca Menichetti2,3, Alessandra Flori3,4, Anna Dost1, Andreas Benjamin Schmidt1,7, Markus Plaumann5, Ferdia Aiden Gallagher6, Jan-Bernd Hövener7

1Department of Radiology, Medical Physics, Medical Center - University of Freiburg, Faculty of Medicine, University of Freiburg, Freiburg, Germany

2Institute of Clinical Physiology, National Research Council (CNR), Pisa, Italy

3Fondazione CNR / Regione Toscana G. Monasterio, Pisa, Italy

4Institute of Life Sciences, Scuola Superiore Sant'Anna, Pisa, Italy

5Institute of Biometrics and Medical Informatics, Otto-von-Guericke University Magdeburg, Magdeburg, Germany

6Department of Radiology, University of Cambridge, Cambridge, UK

7Section  Biomedical Imaging and MOIN CC, University Medical Center Schleswig Holstein, University of Kiel, Kiel, Germany

Short title:  Molecular Imaging with HP Tracers

Corresponding Authors:

Jason Graham Skinner

Email: Jason.skinner@uniklinik-freiburg.de

0049 1764 382 1672

Jan-Bernd Hövener

Email: jan.hoevener@rad.uni-kiel.de

0049 431 880 5832

# Abstract

Since reaching the clinic, Magnetic Resonance Imaging (MRI) has become an irreplaceable radiological tool because of the macroscopic information it provides across almost all organs and soft tissues within the human body, all without the need for ionizing radiation. The sensitivity of MR, however, is too low to take full advantage of the rich chemical information contained in the MR signal. Hyperpolarisation techniques have recently emerged as methods to overcome the sensitivity limitations by enhancing the MR signal by many orders of magnitude compared to the thermal equilibrium, enabling a new class of metabolic and molecular X-nuclei based MR tracers capable of reporting on metabolic processes at the cellular level. These hyperpolarised (HP) tracers have the potential to elucidate the complex metabolic processes of many organs and pathologies, with studies so far focusing on the fields of oncology and cardiology. This review presents an overview of hyperpolarisation techniques that appear most promising for clinical use today, such as Dissolution Dynamic Nuclear Polarisation (d-DNP), Parahydrogen Induced Hyperpolarisation (PHIP), Brute Force Hyperpolarisation, and Spin-exchange Optical Pumping (SEOP), before discussing methods for tracer detection, emerging metabolic tracers and applications, and progress in preclinical and clinical application.

# Introduction

Since reaching the clinic, Magnetic Resonance Imaging (MRI) has become an irreplaceable radiological tool because of the macroscopic information it provides across almost all organs and soft tissues within the human body, all without the need for ionizing radiation. In conventional MRI, the hydrogen nucleus (H-1) - or proton - is the most sensitive stable nucleus due to its high gyromagnetic ratio and abundance within the body. The sensitivity of MR, however, is too low to take full advantage of the rich chemical information contained in the MR signal. For instance, while nuclear magnetic resonance (NMR) spectroscopy is a workhorse in chemistry, in vivo MR spectroscopy (MRS) is limited to few clinical applications and a spatial, temporal, and chemical resolution of cm3, minutes, and mM [1]. The promise of a non-invasive, spatially resolved chemical analysis, however, is immense. Hyperpolarisation techniques have recently emerged as methods to overcome these sensitivity limitations by enhancing the MR signal by many orders of magnitude compared to the thermal equilibrium [2–4], and has enabled a new class of metabolic and molecular MR tracers capable of reporting on metabolic processes at the cellular level [5–7].

Hyperpolarisation consists of generating a so-called ‘hyperpolarised’ (HP) state, in which the population difference between nuclear energy levels is enhanced by several orders of magnitude (illustrated for spin ½ nuclei in Fig. 1). This population difference, or polarisation, is expressed as P = (N+-N-)/(N++N-), where N+ and N- are the numbers of spins in the spin up and spin down populations respectively, and P is directly proportional to the MR signal. The HP state is a non-equilibrium condition of the spin system; therefore, the nuclear spins will relax back to their Boltzmann thermal equilibrium distribution over time with a corresponding loss of the HP signal enhancement. This decay rate is governed by the longitudinal (T1) relaxation time, which varies from a few seconds to a few hours, depending on the nucleus, molecule, and the surrounding environmental conditions [8]. Typically, for metabolic applications, the C-13 nucleus is hyperpolarised because of its favourable life time and chemical shift dispersion compared to hydrogen.

In a typical HP metabolic imaging experiment, a metabolic substrate isotopically enriched with a magnetically active nucleus, usually C-13, is hyperpolarised and rapidly injected in vivo to track metabolism in real time (Fig. 2).  In vivo, the HP nucleus is rapidly exchanged between the substrate and its metabolic products during enzyme-mediated metabolic reactions, and by means of chemical shift resolution and the large signal enhancement, these metabolites can be distinguished. To maximize the signal available for the experiment, the hyperpolariser should be close to the MR scanner, and transfer rapid [11–14].

HP tracers have rapidly become a promising molecular imaging tool for non-invasive in vivo metabolic studies due to the unique metabolic insights they provide. Ionising radiation, like in very sensitive Positron Emission Tomography (PET), is not required [15]. The goal of this review is to present an overview of HP molecular and metabolic imaging, tracer production, detection, and progress in preclinical and clinical application.

# Hyperpolarisation Technology

Here we describe dissolution-Dynamic Nuclear Polarisation (d-DNP) [2, 6, 7], Para-Hydrogen (pH2) Induced Polarisation (PHIP) [4, 17], Brute Force Hyperpolarisation [18], and Spin-Exchange Optical Pumping (SEOP) [3, 19, 20].

## Dissolution Dynamic Nuclear Polarisation (d-DNP)

While DNP history started already in the 1950’s [21], the recent interest in solid state NMR and dissolution-DNP experiments was enabled by the development of improved microwave technologies and more efficient mono- and bi‐radicals [22]. The potential of DNP in modern MRI and solid-state NMR was demonstrated with d-DNP by Golman and co-workers [2], and by low temperature magic angle spinning DNP (MAS-DNP) by Griffin and co-workers [23]. The first d-DNP setup for preclinical studies [2] was at 3.35 T with the ready availability of microwave sources at 94 GHz with maximum powers in the 100–400 mW range, while nowadays commercial polarizers operates at 5 T (SPINlab, General Electrics). At the typical temperature of ~ 1.4 K, DNP takes place transferring the high electronic polarisation of a paramagnetic agent - i.e. unpaired electrons added in the form of radical molecules at mM concentrations - to magnetically active nuclei in the sample, such as 13C (Fig. 3a), irradiating the EPR lineshape under static conditions [24].

At this time, three distinct mechanisms are usually involved to describe polarisation transfer in DNP, whose contribution depends on the EPR linewidths of the radical in comparison to the nuclear Larmor frequency, among other factors. In particular, the Solid Effect deals with a two-spin system by taking into account the interaction between the electronic and the nuclear spin; the Cross Effect accounts for the polarisation transfer from two coupled electrons, whose resonance frequency (ω0S1,2) are separated by the Larmor frequency of the nuclei (ω0I) that are to be polarised, ω0S1 - ω0S2 = ω0I [25]. The Thermal Mixing mechanism exploits thermodynamic concepts and describes the polarisation transfer in terms of heat exchange between different reservoirs [26–29].

The sample formulation and radical stability affects the polarisation performance: the unpaired electrons should be homogenously dispersed (oligomers and clustering are detrimental) into the sample matrix and a suitable glassy agent (such as dimethylsulfoxide, glycerol) are usually added to the formulation. The presence of small amounts of lanthanide chelates, such as Gd3+, can also further enhance the polarisation [30]. A fast dissolution occurs by rapid melting of the sample through direct contact with a superheated (180-200 °C) and pressurised (10 bar with helium) aqueous medium to produce solutions at temperatures and pH values suitable for injection in vivo, while preserving most of the hyperpolarisation [2].

In more recent years, the interest in hyperpolarising water (i.e. of protons diluted in heavy water) led to promising results: polarisation, dissolution and radical extraction for water/DMSO-d6 samples were shown to provide a polarisation and T1 suitable for in vivo MR perfusion imaging [31–33]. Many efforts are currently underway to improve the polarisation efficiency for H-1 and C-13 nuclei with the synthesis of new radicals and better design of polarizers [24]. Currently, [13C]pyruvate is the lead candidate molecule for clinical HP metabolic imaging not only due to its importance in metabolic pathways, but also because it has important chemico-physical properties, such as a long T1 relaxation time, and excellent glass-forming properties required for solid-state DNP. Nevertheless, DNP of other nuclei has been demonstrated, including N-15 [2, 34], Si-29 [35], Y-89 [36], H-1 [37], Xe-129 [38], F-19 [39], Li-6 [40, 41] and, more recently, P-31 [42].

d-DNP is currently expensive because of its need for dedicated instrumentation including a superconducting magnet, and requires the addition of a radical (that is subsequently removed before injection into patients [32]). Recently, however, d-DNP of pyruvate was achieved using photo-induced radicals of pyruvate [43, 44], potentially eliminating the need for an extra radical and its subsequent removal prior to in vivo application, and increases the polarisation lifetime such that sample transport becomes feasible [45]. The long polarisation time (~ 1 h) limits the d-DNP throughput, although this limitation has partially been overcome with setups that polarize several samples at a time [32, 46]. Notably, d-DNP–polarised samples are the only liquid HP tracers that were applied in humans to date [47–50] and commercial setups for sterile use are now available.

In recent years, there have been a number of significant developments in DNP technology [51]. These include the hyperpolarisation of C-13 labelled pyruvic acid by high-field d-DNP in 2016 [52] and the high-field liquid-state DNP of microliter samples using polar solvents such as water or fluorobenzene [53]. C-13 hyperpolarisation of around 30% was demonstrated in the liquid-state by using a photo-induced nonpersistent radicals microwave modulation [54], and, in addition to the progresses in methology, various molecules were investigated as a relaxation boosting agents for d-DNP at high fields, e.g., metallofullerenes such as Gd2@C79N [55].

## Parahydrogen Induced Polarisation (PHIP)

Parahydrogen Induced Polarisation (PHIP) exploits the pure but MR-invisible spin singlet order of parahydrogen (pH2). pH2 denotes one of four combinations of the nuclear spins of the dihydrogen molecule (one para, three ortho). The para-fraction of hydrogen gas is relatively easily to enrich to ~ 100 % by catalytic conversion at low temperature, ~ 23 K. If stored appropriately, in the absence of the catalyst (e.g., in an aluminum flask with little paramagnetic impurities), the lifetime of the para-state is several days to weeks [56] and is thus “spin order on demand”.

This spin order is transferred to a target molecule, either by pair-wise addition (hydrogenation, PASADENA [57–59], ALTADENA [57, 59, 60]) or reversible exchange (SABRE [17, 61]). Usually, the H-1 spin order is converted by magnetic field cycling [62, 63] or spin-order-transfer (SOT) pulse sequences [64, 65] into net polarisation of an X-nucleus with longer relaxation times or “long-lived states”.

pH2-based methods enable the production of aqueous HP solutions within seconds [66] at a fraction of the cost of a d-DNP setup [67–70],The production and subsequent use of PHIP HP tracers has even been demonstrated within the bore of MRI scanner without a dedicated polarizer [71, 72]. Compared to d-DNP, however, the pool of usable agents is restricted. Until recently, the agents that could be polarised with pH2 were limited to molecules with an unsaturated precursor. This limitation was eased dramatically by SABRE [17, 73–79], SABRE-RELAY [80] and PHIP-SAH [64–66]. To date, high C-13 HP of pyruvate by pH2 was demonstrated and used to monitor metabolism [86].

Besides the limitation of suitable tracers, the removal of the hydrogenation catalyst may be the most significant challenge of PHIP methods. Although promising approaches using either heterogeneous hydrogenation [87, 88] or biphasic solutions [89] have been published, so far they have resulted in lower polarisations due to less efficient hydrogenation and relaxation during either the catalyst removal or during phase separation, respectively.

On the path towards routine in vivo application of pH2 polarised tracers, the following important milestones have been achieved: hyperpolarisation and in vivo application of [1-13C]succinate [90], [1-13C]diethylsuccinate [91], and [1-13C]phospholactate [92]. PHIP-SAH polarised [1-13C]pyruvate was injected into mice  and 3.5 ± 0.5 % [1-13C]pyruvate polarisation was detected at the heart and in vivo metabolism was observed [86]. HP [1-13C]hydroxyenthyl-propionate was polarised within an MRI scanner using only the scanner hardware, and the tracer’s in vivo distribution was measured 10 - 15 s after production and injection [93]. SABRE has been achieved in numerous aqueous solutions for 1H and X-nuclei, predominantly N-15 [75-78].  [1H]HP using SABRE has been generated in several molecules, including DNA building blocks in biochemistry and some disease-treating drugs [94, 95]. SABRE-SHEATH enabled the HP > 10% of heteronuclei [96–98], and the biomolecule [15N]nicotinamide has been polarised to P = 7% in aqueous solution [96, 99]. Continuous hyperpolarisation of a biomolecule in aqueous solution was enabled by SABRE [95, 100].

## Brute Force Hyperpolarisation

The simplest approach to achieve a high sample thermal equilibrium polarisation is to apply strong magnetic fields and low temperatures. This method was first used for the HP of noble gases and D2 gas [101, 102] and later to obtain a liquid, aqueous solution containing metabolic tracers [18]. For both cases the sample is frozen during HP and melted or evaporated (in the case of hyperpolarised gases) before usage. He-3 gas, for instance, is evaporated at 6-7 K and extraction can be done much faster than the longitudinal relaxation time of the He-3 liquid of T1=300s (at 1bar pressure) [101]. Note that Xe-129 has a much higher boiling point of T≈165K. In case of the liquid metabolic tracers, a frozen sample is ejected from the polariser after HP and quickly dissolved in warm H2O. In the only published examples, P<1% is achieved for pyruvate at 14 T and ~ 2.3 K [18] which is currently too low for in vivo metabolic imaging.

The advantage of these methods is that theoretically, every molecule can be hyperpolarised by “brute force”, and although there are some limiting factors [103], the range of suitable molecules is vast with the HP of [1-13C]pyruvate, [1-13C]sodium lactate, [1-13C]acetic acid and the above mentioned gases having been demonstrated [18, 101, 102]. Furthermore, no microwave irradiation or co-solvation of radicals or catalyst is needed.

If kept at moderate temperatures and magnetic field strengths after hyperpolarisation, HP samples can be transported as T1 is long under these conditions. For [1-13C]pyruvate at 2 T longitudinal relaxation times of ~5 h and ~20 h were reported at 30 K and 6 K, respectively [18]. For xenon at a temperature of 77 K and in a magnetic field of 100 mT, T1 is of the order of 2.5 hours [104, 105]. At 4.2 K it is already more than 60 hours [102]. These long lifetimes may enable the off-site production and delivery of HP tracers, similar to PET-tracers - a concept that in principle could be applied to any polarisation method free of in-sample relaxation agents.

Particularly the production of liquid, metabolic HP tracers using the brute force method is relatively new and has yet to be fully explored. C-13 enhancement factors of > 1500 and ~ 150 have been demonstrated at 1 T and 9.4 T [103] respectively, which is still too low for biomedical applications. Higher polarisation levels could be achieved at even lower temperatures and higher fields but this poses technical challenges and increases the required time for HP.

## Spin-exchange Optical Pumping (SEOP)

In spin-exchange optical pumping, circularly polarised light is used to selectively optically pump (typically) rubidium (Rb) electrons, generating a highly spin polarised ground state of Rb gas. These atoms subsequently transfer spin polarisation through spin-exchange collisions with noble gas nuclei, such as Xe-129 [106], Ne-21 [107], Kr-83 [108], Xe-131 [109], and He-3 [110]. The resulting signal enhancement has permitted a wide range of applications, and for Xe-129, which can be used as a molecular biosensor [111, 112], this has included: studies of the structure of molecules and proteins [113], bacterial spores [114], and metal organic frameworks (MOFs) [115]; in vivo MR imaging of the lungs [116], brain [117], and brown adipose tissues [118],  and experiments with genetically encoded gas vesicles [119].

Like all HP methods, HP Xe-129 applications benefit from a high level of polarisation at high concentrations. For SEOP, this is challenging due to the high Rb spin destruction cross-section of Xe-129, which quickly suppresses the Rb polarisation as the Xe-129 concentration is increased, limiting the obtainable xenon polarisation [19, 106]. Two approaches to dealing with this difficult issue have emerged: ‘continuous flow’ (CF) SEOP [19], and 'stopped flow’ (SF) SEOP [120], with both methods providing enough polarised gas for lung imaging applications [120, 121]. Recently, the discovery of spin destructive Rb clusters forming CF polarisers has provided a pathway to increase the current capabilities of such systems [122]. Optimisation of the magnetic fields may yield further gains [123]. SF has achieved polarisations of ~ 74 % at 1000 Torr Xe [120], and ~ 90 % at 300 Torr Xe [20], both while back-filled to 2000 Torr with N2, in ~litre quantities, with the largest gains in magnetisation coming from the higher Xe density mixes due to its proportionality with the product of polarisation and concentration [20, 124].

While HP Xe-129 imaging of the lungs is currently the most promising biomedical application of this technology - as it allows regional variations in lung function to be detected which is not possible with existing approaches - HP Xe-129 biosensors are showing great promise as a tracer for molecular imaging [125, 126].

# Hyperpolarised Magnetic Resonance Spectroscopic Imaging (MRSI) and Metabolic Imaging

The polarisation of HP agents relaxes back to thermal equilibrium very rapidly, losing the entire signal enhancement endowed by the hyperpolarisation process. Depending on the orientation of the magnetisation, the loss is governed by T1 or T2. Imaging speed and efficient use of the finite magnetisation is therefore a critical factor. In general, rf-heavy sequences or those with high flip angles should be avoided because every excitation used during the course of a pulse sequence will draw on the finite pool of HP magnetisation - although sequences that continually re-focus transverse magnetisation using rf pulses, such as steady-state free precision sequences [127], can mitigate this effect. Additionally, spectroscopically selective pulses can spare magnetisation of the injected tracer and increase SNR on metabolites of interest [128].

To obtain information on enzyme-catalysed metabolism, dynamic, spectrally resolved data are required such that the dynamics of the metabolic conversions can be obtained (such as those in Fig. 2), and analysed to yield values of the conversion rate constants [129].  In the simplest but most sensitive implementation, the data is acquired by pulse-acquisition experiments with a local coil. For improved spatial resolution, specialised acquisition methods such as chemical shift imaging (CSI) are needed (Fig. 6a).

Imaging within in vivo experiments is usually done using single-shot acquisition: echo-planar spectroscopic imaging (EPSI) [133-136] or center-out k-space encoding (spiral read-out) can be customised for fast k-space coverage. In this case, spiral CSI (SPCSI) (Fig. 5) and strategies based on multi-echo chemical shift species separation, such as in IDEAL spiral CSI (ISPCSI) [130], are among the most effective approaches.

Additionally, alternatives were evaluated such as balanced-Steady-State Free Precession (SSFP) Imaging [127, 131, 132], Stimulated Echo (STEAM) [133, 134] or Free-Breathing Measurements [135], particularly effective for cardiac studies. Compressed sensing [128, 136, 137], that improves the spatial resolution without increasing the acquisition time, as well as parallel imaging [138], which provides accelerated signal acquisition, were also reported. Finally, single-shot spatiotemporal encoding (SPEN) was more recently proposed because it is able to simultaneously provide spatial and chemical shift information within a time period of a few hundreds of milliseconds [139].

# Biomedical Applications of Hyperpolarised Metabolic and Molecular Tracers

[1-13C]Pyruvate is the most commonly used molecule in vivo because it has a central role in cell metabolism, at the crossroads of anaerobic and aerobic metabolism and is either exchanged into lactate or forms Acetyl-CoA and carbon dioxide [140]. The metabolic conversion of pyruvate into its main downstream metabolites lactate, bicarbonate and alanine (Fig. 4) can be tracked in vivo and in real-time, to investigate the flux of key enzymes such as Lactate Dehydrogenase (LDH), Pyruvate Dehydrogenase (PDH) or Alanine Transaminase (ALT), in different metabolic pathways and pathological conditions [141]. HP-pyruvate has been mainly used as a probe of glycolytic metabolism in cancer and in the heart, as well as in other tissues, such as the brain and liver. This section describes investigations that have been conducted with [1-13C]pyruvate and other emerging substrates.

## Probing Lactate Dehydrogenase (LDH)

LDH catalyses the reversible metabolic conversion of pyruvate into lactate in the cytosol. Due to the Warburg Effect, cancer cells show increased glycolytic activity and glucose uptake in aerobic conditions, resulting in the production of lactate. Intravenous administration of HP [1-13C]pyruvate results in an enhanced spectroscopic signal of [1-13C]lactate in tumours due to enzymatic exchange via LDH that can be correlated both with tumour staging [5, 128, 142] and with treatment response [47, 143].

LDH-catalyzed metabolic conversion of [1-13C]pyruvate into [1-13C]lactate has also been investigated in the field of cardiology since both lactate pool size and production depend on the metabolic status of the heart. Due to the onset of acidosis and to the shift towards anaerobic metabolism, the lactate concentration is increased early after ischaemia, resulting in a larger lactate pool. An increase in LDH flux has been detected in vitro in the isolated rat heart [144] and in pigs after ischaemia-reperfusion injury [145]. A significant increase in [1-13C]lactate was also detected in pigs subjected to cardiac inotropic stress induced by intravenous dobutamine administration [146], suggesting an increase of the flux through LDH in conditions of higher energy demand. Conversely, in a porcine model of dilated cardiomyopathy induced by chronic rapid right ventricular pacing, no variation was detected in the [1-13C]lactate/[1-13C]pyruvate signal ratio, corresponding to an unchanged LDH flux [147].

It should be noted, too, that lactate or phospholactate was polarised by DNP [148] and PHIP [149], respectively. Because the lactate pool is much larger in vivo than the pyruvate pool, only little signal of hyperpolarised pyruvate was observed in these experiments. A deuterium label was used to facilitate the analysis of the data.

## Probing Pyruvate Dehydrogenase (PDH)

HP [1-13C]pyruvate can also be used to probe citric acid cycle carbon flux. In vivo metabolic conversion into [1-13C]bicarbonate provides information about the activity/flux of the multienzyme complex PDH that regulates the irreversible decarboxylation of pyruvate to acetyl-CoA (ACoA), the energetic fuel of the Krebs cycle (Fig. 4). The PDH complex has a key role in the balance between glucose and FA oxidation, in competition for the ACoA supply in cardiac cells [150]. Therefore, the flux through PDH should be highly flexible in response to different substrate availability in order to meet the haemodynamic demand in different physiological states or pathological conditions. Merritt et al. first showed that the rate of appearance of HP H13CO3- and HP 13CO2, after injection of HP [1-13C]pyruvate in the isolated rat heart could be directly related to the flux through PDH [151]. The enzymatic flux through PDH has also been investigated in vivo, in small and large animal models, by detecting the metabolic conversion of HP [1-13C]pyruvate into [1-13C]bicarbonate [152]. For instance, transient impairments of PDH complex have been recorded in pigs after acute ischaemia and reperfusion [145], and a reduction in PDH flux was also shown in hyperthyroid hearts in rats [153], which was related to both increased myocardial FA oxidation and to stimulation of PDK4 gene expression by the thyroid hormone T3.

HP [1-13C]lactate has been also evaluated as an alternative probe of PDH flux [155]. Because of the elevated endogenous concentration, lactate has a superior safety profile and is better tolerated at higher concentrations. Potentially it could be used to probe LDH activity in an alternative way as well as tissue utilisation of lactate as a fuel source.

HP probes can be used to map pH in vivo and in real-time, by looking at the HP [13C]bicarbonate/[13C]CO2 ratio [156, 157]. Assuming that the rate of exchange of the 13C label between the two molecules is rapid, catalysed by the enzyme carbonic anhydrase (CA), the pH can be estimated by calculating the ratio of HP [13C]bicarbonate to 13CO2 peak areas [156]. This was first demonstrated in tumour studies with the infusion of the HP [13C]bicarbonate (Fig. 6(b)) [156], and has also been used to probe the activity of the enzyme CA itself [158]. It is also relevant for cardiac studies, as myocardial ischemia is characterised by a rapid onset of acidosis due to increased anaerobic glycolysis in condition of poor coronary perfusion [159]. In vivo pH was also recently probed with HP [1,5-13C]zymonic acid, a substrate whose 13C resonances shift up to 3 ppm per pH unit in the physiologic range [160].

## Probing Alanine Transaminase (ALT)

Pyruvate can be also generated through the catabolism of various amino acids, including alanine, one of the major gluconeogenic precursors: pyruvate and alanine are the central substrates in the alanine cycle, a recycling and scavenging pathway linking muscle and liver metabolism. Real-time assessment of ALT flux can be investigated by tracking the rate of production of HP [1-13C]pyruvate-derived [1-13C]alanine [151], to potentially explore the transamination activity and alanine pool size. This is relevant for liver disease [163] because liver is a primary site of ALT enzyme activity.

A few studies have investigated the reversible nature of the ALT catalysed conversion and detected [1-13C]pyruvate spectroscopic signal after the injection of HP [1-13C]alanine [164]. In the heart, the exchange rate of the 13C label from [1-13C]pyruvate to [1-13C]alanine was increased in hypertrophy in a rat model of hyperthyroidism [165] and in conditions of increased cardiac workload as reported in pigs and in rats, after dobutamine administration [146, 166].

## Investigation of Krebs Cycle Intermediates

Further characterisation of PDH flux is possible by injection of HP [2-13C]pyruvate to investigate the real-time incorporation of pyruvate-derived ACoA into the Krebs cycle. In this case, the labelling in the C2 position is exchanged with ACoA during the PDH mediated conversion of [2-13C]pyruvate, thus allowing the tracking of Krebs cycle intermediates such as citrate and glutamate. The observation of the [2-13C]pyruvate metabolic products is linked to the contribution of pyruvate incorporation into the TCA cycle and allows for exploration of mitochondrial substrate selection.

The injection of HP [1,2-13C]pyruvate was also performed in pigs to allow simultaneous in vivo investigation of PDH flux, Krebs cycle flux and pH [167]. An enhancement of PDH flux and incorporation into the Krebs cycle intermediates was also found in the spontaneously hypertensive rat heart after injection of HP [1-13C]pyruvate and [2-13C]pyruvate, demonstrating a switch of cardiac metabolism from fatty-acids towards glucose oxidation [168]. In contrast, early stage heart failure resulted in a decrease of pyruvate flux through PDH and subsequent incorporation into the Krebs cycle [169–171]. The [1-13C]acetylcarnitine (ALCAR) produced from the [2-13C]pyruvate-derived ACoA, through a metabolic reaction catalysed by the enzyme carnitine acetyltransferase (CAT), can be also detected. In a study performed in rats using HP [2-13C]pyruvate, the authors describe a dynamic cycling mechanism of [2-13C]pyruvate-derived ACoA into the ALCAR pool (through the enzyme CAT) before entering the Krebs cycle, thus underlining a ‘fine-tuning’ mechanism of CAT-mediated disposal of ACoA [170].

## In vivo Fatty-Acid (FA) Metabolism

HP 13C-MRI has been used to probe in vivo Fatty-Acide (FA) metabolism. The metabolic fate of FA inside the cell is highly regulated: once taken up by the cytosol, FA are esterified to fatty acyl-CoA and converted to long-chain ALCAR, which is located in the outer mitochondrial membrane, prior to entering the mitochondria. Acyl-CoA finally undergoes to β-oxidation to generate ACoA (in competition with glucose-derived ACoA) and the reducing equivalents NADH and FADH2.

[1-13C]Acetate was among the first FA metabolism HP probes. Recalling the β-oxidation theory of fat metabolism, [1-13C]acetic acid attracted great interest as a probable intermediary metabolite in fat oxidation: indeed, FA are the main source of myocardial energy supply in the healthy adult heart [150], and HP [1-13C]acetate has been evaluated as probe of FA metabolic pathway in cardiac applications [172, 173], while a few studies focus on skeletal muscle [16], brain [174] and kidneys [175].

The study of metabolic conversion into the 13C-ACoA-derived 13C-ALCAR, and the incorporation into the Krebs cycle through ACoA labelling faced sensitivity limitations in vivo. As compared to [1-13C]pyruvate, HP acetate formulations provide lower [13C]-concentration, as well as shorter T1 and lower polarisation level. All these factors together reduce the available SNR.

Moreover, the highly regulated FA metabolism likely results in a slower metabolic conversion and incorporation into the Krebs cycle. Because of the reduced sensitivity, in vivo studies with HP acetate can take advantage of the increased spectral resolution and SNR provided by high field NMR systems [176–178]. It should be noted, though, that the direct detection of hyperpolarised Acetyl-Coa has yet to be reproduced by other groups.

It has been suggested that [1-13C]acetate uptake in cells occurs passively by diffusion from plasma through a concentration gradient. The dependence from the injected HP acetate dose was investigated in rats: indeed, a dose-dependent correlation between the ALCAR/acetate ratio and the injected acetate was found. In parallel, no dependence was observed for the signal of the Krebs cycle metabolite [5-13C]citrate, which could thus provide a reliable estimation of the flux through Krebs cycle in cardiac cells [177].

Medium-chain FA analogues, such as [1-13C]butyrate or [1-13C]octanoate [179], have also been hyperpolarised. The spectroscopic signal of the products [1-13C]ALCAR and [1-13C]acetoacetate, as well as the Krebs cycle intermediates [5-13C]glutamate and [5-13C]citrate, could be reliably detected in cardiac studies in small animals [180–182]. The co-infusion of HP [1-13C]butyrate and [1-13C]pyruvate in rats provided a first proof of concept for the in vivo investigation of myocardial substrate selection in real-time [179].

[1-13C]diethyl succinate, hyperpolarised through PHIP and DNP [183], was applied in vivo as a CA targeting the TCA cycle with mixed results [91]. Observed resonances were assigned to metabolites or mono-esters. Likely, 1-13C-succinate-D2 is the metabolic substrate that was hyperpolarised most often with pH2. Despite a somewhat lower polarisation performance (≈ 8 %) as compared d-DNP, in one in vivo study, several resonances were observed and assigned to metabolites at 4.7 T.

## Redox Probes

HP probes have been used to detect redox reactions: both the reduced and oxidised form of vitamin C, i.e. [1-13C]ascorbic acid (13C-AA) and [1-13C]dehydroascorbic acid (13C-DHA) have been tested.  Several diseases are characterised by an altered production of reactive oxygen species (ROS), leading to a dysregulation of the cellular redox potential and to an increased tissue oxidative stress. An enhanced oxidative stress is considered a hallmark of different cancer phenotypes [184]; moreover, high levels of oxidative stress can be found in infectious diseases, chronic inflammation, and diabetes [185].

Studies with HP 13C-DHA have been carried out to investigate the status of oxidative stress of a tissue through the observation of its reducing capacity, reflected by the local concentration of reducing agents (especially glutathione). Preliminary studies have been conducted in tumour cell cultures and in small animal models. In these studies, the conversion of 13C-DHA into 13C-AA can be observed and semi-quantitatively assessed through the measured VitC / (DHA + VitC) ratio. This parameter, which provides indications of the tissue redox capacity, has been mainly related to the glutathione tissue concentration, and to a lesser extent to the glucose transporter (in particular to GLUT4) [186], the rate of NADPH production, the flux through the cytosolic pentose phosphate pathway [187],  and the expression of the enzyme NAPDH oxidase 4 (Nox4) [188].

[13C]AA has also been hyperpolarised to track the in vivo oxidation into [13C]DHA and to provide direct evidence of the tissue redox status (Fig 6(c)). HP [13C]AA is an attractive alternative to [13C]DHA as it is less toxic; however, a shorter T1 and reduced polarisation have been recorded [162]. A different internalisation mechanism has been also pointed out for the two molecules [189]: while AA is transported inside the cell via the slower sodium-dependent vitamin C transporter (SVCT1-2), DHA entry occurs through the faster GLUT-mediated transport. These factors could reduce the in vivo utility of HP [13C]AA. Extracellular oxidation of 13C-AA and intracellular reduction of 13C-DHA were observed in cells. [13C]DHA reduction was detected in vivo. In contrast, no detectable oxidation of [13C]AA was measured in the same tumours, consistent with the fact that tumours maintain a reduced microenvironment.

## Molecular Imaging with Xe-129 Biosensors

HP Xe-129 functionalised host-guest biosensors [111] – biomolecular cages functionalised with specific targeting moieties with which Xe can freely exchange (Fig. 7) - are emerging as another potent class of molecular biosensors that, when combined with HyperCEST detection schemes [112], can report on picomolar cell concentrations with high specificity [111, 125]. Many biological and molecular targets can be envisaged and a number have already been pursued including e.g. avadin [111, 190], human carbonic anhydrase II [191, 192], cell surface glycans [193], a number of receptors such as integrin (known to be upregulated in some cancers) [194], EGFR [195] and transferrin [196] – even cancer cells [125, 195]. Recently, important in vitro and in vivo demonstrations have included the in vitro use of MS2 viral capsids as macromolecular biosensor hosts (Fig. 7b) for lymphoma cell detection at picomolar concentrations [125], the first in vivo detection of a Xe-129 biosensor: (non-functionalised) cucurbit[6]uril (CB6) [126], and the in vitro use of a fluorescent biosensor for mitochondria detection [197].

In the Xe-129 biosensor concept, many of Xe-129’s remarkable properties are exploited to create a sensitive biomolecular spy: HP Xe-129 has a distinct resonance in the gas phase which is manifest as a single peak in an NMR spectrum. When free Xe-129 exchanges with a functionalised biosensor population, two further distinct peaks will appear in the spectrum, originating from its respective exchange with target-bound and unbound biosensor populations. These further peaks result from the exquisite chemical shift sensitivity of Xe-129 to its environment, and provide a means to distinguish between the populations. Extremely low concentrations of the target-bound, functionalised biosensors can be detected using Hyper-CEST, which takes the existing concept of CEST – detection of the saturation of a dilute spin pool A (the target-bound functionalised cages) via the monitoring of the depletion of an exchange-coupled and significantly larger spin pool B (free HP Xe-129) - but combines it with a HP spin pool B, thereby substantially enhancing the sensitivity.

Diseases are complex in nature, often presenting with many unique biomarkers simultaneously. Xenon biosensors may ultimately offer unique and powerful diagnostic opportunities due to the potential for the deployment of multiple biosensors at the same time [199], each uniquely targeting different biomarkers of the same disease simultaneously (and potentially detected simultaneously) - a concept known a multiplexing [111, 198, 199]. However translation to in vivo imaging is faced by a number of challenges including RF tissue heating, delivery of both HP Xe-129 and the biosensors to the site of interest, and unknowns regarding biocompatibility of some sensors [200].

# Human Applications

## Hyperpolarised Tracers for Metabolic Imaging

Hyperpolarisation has been translated into humans using d-DNP HP 13C-pyruvate as the injected substrate. The first study by Nelson, et al. used a prototype hyperpolariser and demonstrated the feasibility of the method to reveal real-time lactate formation in prostate cancer [48]. There is initial evidence from this study that metabolic imaging may detect tumours that are occult on conventional imaging; current studies are addressing whether the method can better stratify tumour aggressiveness based on lactate formation, as has been demonstrated in animal models [142]. There is preliminary evidence demonstrating that HP [1-13C]lactate labelling may decrease following therapy in prostate cancer (Fig. 8(a)) [47] which builds on similar pre-clinical data in several tumour models [143]. More recently, the metabolism of HP [1-13C]pyruvate has also been imaged in glioma [49]. Therefore, the technique may have a role in the stratification of tumour aggressiveness and the early detection of treatment response [201].

Given the high metabolic activity of the heart, HP [13C]pyruvate has also shown promise in probing cardiac disease. A single human study has been published to date showing the formation of both [1-13C]lactate and [113C]bicarbonate in the normal human heart following the injection of [1-13C]pyruvate (Fig. 8(b)) [50]. Ongoing studies are using the technique to probe alterations in cardiac metabolism in human disease.

## Hyperpolarised Gas Imaging

Imaging of the lungs with HP gases offers the possibility of detecting regional variations in ventilation and lung function which as early features of pulmonary disease. The majority of pulmonary imaging with HP gases has been undertaken using He-3 given its strong MR signal, but more recently there has been an increase in the use of Xe-129 to assess lung ventilation, microstructure and regional gas exchange [202]. This has been driven by the decreased availability of He-3 and improvements in Xe-129 gas technology. HP gas imaging has been used to assess healthy subjects, smokers and patients with asthma and chronic obstructive pulmonary disease (COPD) [203–205]. In particular, the high level of regional variability in patients with COPD can be used to monitor early disease and as a biomarker of response to therapy [206], for example in the context of clinical trials. While HP gas MRI is not used as a routine clinical tool, it has the potential to be used in stratified patient populations to assist clinical decision making.

# Conclusion

HP imaging technologies offer great potential to provide important new insights into metabolic changes that occur in human disease and following treatment. However, there are many challenges in translating these methods into humans, including the need for dedicated hardware and a requirement for significant technical expertise across a broad range of disciplines such as physics, chemistry, biology and medicine. Importantly, the need to produce agents suitable for human use may require investment in dedicated pharmacy facilities and there significant regulatory hurdles that need to be overcome which can be very specific to individual countries. Key to the successful translation of the technique will be the development of methods to accurately and reproducibly quantify the metabolism of HP substrates in vivo [129]. Despite these challenges, HP MRI has been used in a number of different clinical settings including oncology, cardiology and respiratory medicine.

# Acknowledgements

We gratefully thank the European Society for Molecular Imaging for their support and the possibility of establishing a study group for Hyperpolarisation as a platform for scientific exchange within the society and beyond. We gratefully acknowledge funding providers: Marie Sklodowska-Curie grant No. 642773 (JGS, AD, JBH, JH); CNR-Fondazione Toscana Gabriele Monasterio, and the Italian Multi-sited Multi-Modal Molecular Imaging site of Eurobioimaging in Pisa (LM and AF); Heinrich-Böll-Stiftung P131623 (AS); DFG BE 1824/12-1 (MP); Cancer Research UK and the National Institute of Health Research Biomedical Research Centre (FG); DFG Emmy Noether Programme, award No HO-4604/2-1, the Faculty of Medicine of the CAU, EXC 306 and GRK 2154 Materials for Brain (JBH).

Conflict of Interest

The authors declare no conflict of interest.

References

1. 	Xu V, Chan H, Lin AP, et al (2008) MR spectroscopy in diagnosis and neurological decision-making. Semin Neurol 28:407–422.

2. 	Ardenkjær-Larsen JH, Fridlund B, Gram A, et al (2003) Increase in signal-to-noise ratio of &gt; 10,000 times in liquid-state NMR. Proc Natl Acad Sci 100:10158–10163.

3. 	Walker TG, Happer W (1997) Spin-exchange optical pumping of noble-gas nuclei. Rev Mod Phys 69:629–642.

4. 	Bowers CR, Weitekamp DP (1987) Parahydrogen and synthesis allow dramatically enhanced nuclear alignment. J Am Chem Soc 109:5541–5542.

5. 	Kurhanewicz J, Vigneron DB, Brindle K, et al (2011) Analysis of Cancer Metabolism by Imaging Hyperpolarized Nuclei: Prospects for Translation to Clinical Research. Neoplasia 13:81–97 .

6. 	Comment A, Merritt ME (2014) Hyperpolarized Magnetic Resonance as a Sensitive Detector of Metabolic Function. Biochemistry (Mosc) 53:7333–7357.

7. 	Keshari KR, Wilson DM (2014) Chemistry and biochemistry of 13 C hyperpolarized magnetic resonance using dynamic nuclear polarization. Chem Soc Rev 43:1627–1659.

8. 	Keshari KR, Sriram R, Koelsch BL, et al (2013) Hyperpolarized 13C-pyruvate magnetic resonance reveals rapid lactate export in metastatic renal cell carcinomas. Cancer Res 73:529–538.

9. 	Schroeter A, Rudin M, Gianolio E, et al (2017) MRI. In: Small Animal Imaging. Heidelberg, Dordrecht, London, New York, Berlin: Springer,  pp 227–324

10. 	Hövener J-B, Lange T, Leibfritz D (2016) Metabolic Magnetic Resonance. In: Visualization of the Brain and its Pathologies - Technical and Neurosurgical Aspects, Eds. Samii A, Nabavi A, Fahlbusch R. Uelvesbüll: Der Andere Verlag, pp 3–32

11. 	Chattergoon N, Martínez-Santiesteban F, Handler WB, et al (2013) Field dependence of T 1 for hyperpolarized [1- 13 C]pyruvate: FIELD DEPENDENCE OF T 1 FOR HYPERPOLARIZED PYRUVATE. Contrast Media Mol Imaging 8:57–62.

12. 	Cheng T, Mishkovsky M, Bastiaansen JAM, et al Automated transfer and injection of hyperpolarized molecules with polarization measurement prior to in vivo NMR. NMR Biomed 26:1582–1588.

13. 	Bowen S, Hilty C (2010) Rapid sample injection for hyperpolarized NMR spectroscopy. Phys Chem Chem Phys 12:5766–5770.

14. 	Shang H, Skloss T, von Morze C, et al (2016) Handheld electromagnet carrier for transfer of hyperpolarized carbon-13 samples: Electromagnet Carrier for Hyperpolarized 13 C Samples. Magn Reson Med 75:917–922.

15. 	Comment A, Merritt ME (2014) Hyperpolarized Magnetic Resonance as a Sensitive Detector of Metabolic Function. Biochemistry (Mosc) 53:7333–7357.

16. 	Bastiaansen JAM, Cheng T, Mishkovsky M, et al (2013) In vivo enzymatic activity of acetylCoA synthetase in skeletal muscle revealed by 13C turnover from hyperpolarized [1-13C]acetate to [1-13C]acetylcarnitine. Biochim Biophys Acta BBA - Gen Subj 1830:4171–4178.

17. 	Adams RW, Aguilar JA, Atkinson KD, et al (2009) Reversible Interactions with para-Hydrogen Enhance NMR Sensitivity by Polarization Transfer. Science 323:1708–1711.

18. 	Hirsch ML, Smith BA, Mattingly M, et al (2015) Transport and imaging of brute-force 13 C hyperpolarization. J Magn Reson 261:87–94.

19. 	Driehuys B, Cates GD, Miron E, et al (1996) High‐volume production of laser‐polarized 129 Xe. Appl Phys Lett 69:1668–1670.

20. 	Nikolaou P, Coffey AM, Walkup LL, et al (2013) Near-unity nuclear polarization with an open-source 129Xe hyperpolarizer for NMR and MRI. Proc Natl Acad Sci 110:14150–14155.

21. 	Atsarkin VA, Kessenikh AV (2012) Dynamic Nuclear Polarization in Solids: The Birth and Development of the Many-Particle Concept. Appl Magn Reson 43:7–19.

22. 	F. Pinto L, Marín-Montesinos I, Lloveras V, et al (2017) NMR signal enhancement of >50000 times in fast dissolution dynamic nuclear polarization. Chem Commun 53:3757–3760.

23. 	Hall DA, Maus DC, Gerfen GJ, et al (1997) Polarization-Enhanced NMR Spectroscopy of Biomolecules in Frozen Solution. Science 276:930–932.

24. 	Jähnig F, Kwiatkowski G, Ernst M (2016) Conceptual and instrumental progress in dissolution DNP. J Magn Reson 264:22–29.

25. 	Macholl S, Jóhannesson H, Henrik Ardenkjaer-Larsen J (2010) Trityl biradicals and 13C dynamic nuclear polarization. Phys Chem Chem Phys 12:5804–5817.

26. 	Guarin D, Marhabaie S, Rosso A, et al (2017) Characterizing Thermal Mixing Dynamic Nuclear Polarization via Cross-Talk between Spin Reservoirs. J Phys Chem Lett 8:5531–5536.

27. 	Wenckebach WT (2017) Dynamic nuclear polarization via thermal mixing: Beyond the high temperature approximation. J Magn Reson 277:68–78.

28. 	Hovav Y, Feintuch A, Vega S (2013) Theoretical aspects of dynamic nuclear polarization in the solid state – spin temperature and thermal mixing. Phys Chem Chem Phys 15:188–203.

29. 	Colombo Serra S, Rosso A, Tedoldi F (2012) Electron and nuclear spin dynamics in the thermal mixing model of dynamic nuclear polarization. Phys Chem Chem Phys 14:13299–13308.

30. 	Ardenkjaer-Larsen JH, Macholl S, Jóhannesson H (2008) Dynamic Nuclear Polarization with Trityls at 1.2 K. Appl Magn Reson 34:509–522.

31. 	Vuichoud B, Bornet A, de Nanteuil F, et al Filterable Agents for Hyperpolarization of Water, Metabolites, and Proteins. Chem – Eur J 22:14696–14700.

32. 	Ardenkjaer-Larsen JH, Leach AM, Clarke N, et al (2011) Dynamic nuclear polarization polarizer for sterile use intent. NMR Biomed 24:927–932.

33. 	Lipsø KW, Bowen S, Rybalko O, Ardenkjær-Larsen JH (2017) Large dose hyperpolarized water with dissolution-DNP at high magnetic field. J Magn Reson 274:65–72.

34. 	Cudalbu C, Comment A, Kurdzesau F, et al (2010) Feasibility of in vivo 15 N MRS detection of hyperpolarized 15N labeled choline in rats. Phys Chem Chem Phys 12:5818–5823.

35. 	Cassidy MC, Chan HR, Ross BD, et al (2013) In vivo magnetic resonance imaging of hyperpolarized silicon particles. Nat Nanotechnol 8:363–368.

36. 	Lumata L, Merritt M, Malloy C, et al (2012) Fast Dissolution Dynamic Nuclear Polarization NMR of 13C-Enriched 89Y-DOTA Complex: Experimental and Theoretical Considerations. Appl Magn Reson 43:69–79.

37. 	Ardenkjaer-Larsen JH, Laustsen C, Bowen S, Rizi R (2014) Hyperpolarized H 2 O MR angiography: Hyperpolarized 1 H 2 O MR Angiography. Magn Reson Med 71:50–56.

38. 	Comment A, Jannin S, Hyacinthe J-N, et al (2010) Hyperpolarizing Gases via Dynamic Nuclear Polarization and Sublimation. Phys Rev Lett 105:018104.

39. 	Lee Y, Zeng H, Ruedisser S, et al (2012) Nuclear Magnetic Resonance of Hyperpolarized Fluorine for Characterization of Protein–Ligand Interactions. J Am Chem Soc 134:17448–17451.

40. 	Heeswijk RB van, Uffmann K, Comment A, et al Hyperpolarized lithium-6 as a sensor of nanomolar contrast agents. Magn Reson Med 61:1489–1493.

41. 	Balzan R, Mishkovsky M, Simonenko Y, et al Hyperpolarized 6Li as a probe for hemoglobin oxygenation level. Contrast Media Mol Imaging 11:41–46.

42. 	Nardi-Schreiber A, Gamliel A, Harris T, et al (2017) Biochemical phosphates observed using hyperpolarized 31P in physiological aqueous solutions. Nat Commun 8:

43. 	Eichhorn TR, Takado Y, Salameh N, et al (2013) Hyperpolarization without persistent radicals for in vivo real-time metabolic imaging. Proc Natl Acad Sci 110:18064–18069.

44. 	Capozzi A, Hyacinthe J-N, Cheng T, et al (2015) Photoinduced Nonpersistent Radicals as Polarizing Agents for X-Nuclei Dissolution Dynamic Nuclear Polarization. J Phys Chem C 119:22632–22639.

45. 	Capozzi A, Cheng T, Boero G, et al (2017) Thermal annihilation of photo-induced radicals following dynamic nuclear polarization to produce transportable frozen hyperpolarized 13C-substrates. Nat Commun 8:15757.

46. 	Batel M, Krajewski M, Weiss K, et al (2012) A multi-sample 94GHz dissolution dynamic-nuclear-polarization system. J Magn Reson 214:166–174.

47. 	Aggarwal R, Vigneron DB, Kurhanewicz J (2017) Hyperpolarized 1-[13C]-Pyruvate Magnetic Resonance Imaging Detects an Early Metabolic Response to Androgen Ablation Therapy in Prostate Cancer. Eur Urol 72:1028–1029.

48. 	Nelson SJ, Kurhanewicz J, Vigneron DB, et al (2013) Metabolic Imaging of Patients with Prostate Cancer Using Hyperpolarized [1-13C]Pyruvate. Sci Transl Med 5:198ra108-198ra108.

49. 	Park I, Larson PEZ, Gordon JW, et al (2018) Development of methods and feasibility of using hyperpolarized carbon-13 imaging data for evaluating brain metabolism in patient studies. Magn Reson Med.

50. 	Cunningham CH, Lau JY, Chen AP, et al (2016) Hyperpolarized 13C Metabolic MRI of the Human Heart: Initial Experience. Circ Res CIRCRESAHA.116.309769.

51. 	Plainchont B, Berruyer P, Dumez J-N, et al (2018) Dynamic Nuclear Polarization Opens New Perspectives for NMR Spectroscopy in Analytical Chemistry. Anal Chem 90:3639–3650.

52. 	I. Yoshihara HA, Can E, Karlsson M, et al (2016) High-field dissolution dynamic nuclear polarization of [1- 13 C]pyruvic acid. Phys Chem Chem Phys 18:12409–12413.

53. 	Yoon D, Dimitriadis AI, Soundararajan M, et al (2018) High-Field Liquid-State Dynamic Nuclear Polarization in Microliter Samples. Anal Chem 90:5620–5626.

54. 	Capozzi A, Karlsson M, Petersen JR, et al (2018) Liquid-State 13C Polarization of 30% through Photoinduced Nonpersistent Radicals. J Phys Chem C 122:7432–7443.

55. 	Wang X, E. McKay J, Lama B, et al (2018) Gadolinium based endohedral metallofullerene Gd 2 @C 79 N as a relaxation boosting agent for dissolution DNP at high fields. Chem Commun 54:2425–2428.

56. 	Wagner S (2014) Conversion rate of para-hydrogen to ortho-hydrogen by oxygen: implications for PHIP gas storage and utilization. Magn Reson Mater Phys Biol Med 27:195–199.

57. 	Natterer J, Bargon J (1997) Parahydrogen induced polarization. Prog Nucl Magn Reson Spectrosc 31:293–315

58. 	Bowers CR, Weitekamp DP (1987) Parahydrogen and synthesis allow dramatically enhanced nuclear alignment. J Am Chem Soc 109:5541–5542.

59. 	Kuhn LT (2013) Hyperpolarization Methods in NMR Spectroscopy. Berlin, Heidelberg: Springer.

60. 	Pravica MG, Weitekamp DP (1988) Net NMR alignment by adiabatic transport of parahydrogen addition products to high magnetic field. Chem Phys Lett 145:255–258

61. 	Hövener J-B, Schwaderlapp N, Borowiak R, et al (2014) Toward Biocompatible Nuclear Hyperpolarization Using Signal Amplification by Reversible Exchange: Quantitative in Situ Spectroscopy and High-Field Imaging. Anal Chem 86:1767–1774.

62. 	Jóhannesson H, Axelsson O, Karlsson M (2004) Transfer of para-hydrogen spin order into polarization by diabatic field cycling. Comptes Rendus Phys 5:315–324.

63. 	Bommerich U, Trantzschel T, Mulla-Osman S, et al (2010) Hyperpolarized 19F-MRI: parahydrogen-induced polarization and field variation enable 19F-MRI at low spin density. Phys Chem Chem Phys 12:10309.

64. 	Goldman M, Jóhannesson H, Axelsson O, Karlsson M (2005) Hyperpolarization of 13C through order transfer from parahydrogen: A new contrast agent for MRI. Magn Reson Imaging 23:153–157.

65. 	Bär S, Lange T, Leibfritz D, et al (2012) On the spin order transfer from parahydrogen to another nucleus. J Magn Reson 225:25–35.

66. 	Bhattacharya P, Chekmenev EY, Reynolds WF, et al (2011) PHIP Hyperpolarized MR Receptor Imaging In Vivo: A Pilot Study of 13C Imaging of Atheroma in Mice. NMR Biomed 24:1023–1028.

67. 	Waddell KW, Coffey AM, Chekmenev EY (2011) In Situ Detection of PHIP at 48 mT: Demonstration Using a Centrally Controlled Polarizer. J Am Chem Soc 133:97–101.

68. 	Coffey AM, Shchepin RV, Truong ML, et al (2016) Open-Source Automated Parahydrogen Hyperpolarizer for Molecular Imaging Using 13 C Metabolic Contrast Agents. Anal Chem 88:8279–8288.

69. 	Coffey AM, Shchepin RV, Feng B, et al (2017) A pulse programmable parahydrogen polarizer using a tunable electromagnet and dual channel NMR spectrometer. J Magn Reson 284:115–124.

70. 	Hövener J-B, Bär S, Leupold J, et al (2013) A continuous-flow, high-throughput, high-pressure parahydrogen converter for hyperpolarization in a clinical setting: A HIGH-THROUGHPUT PARAHYDROGEN CONVERTER FOR HYPERPOLARIZATION. NMR Biomed 26:124–131.

71. 	Schmidt AB, Berner S, Schimpf W, et al (2017) Liquid-state carbon-13 hyperpolarization generated in an MRI system for fast imaging. Nat Commun 8:14535.

72. 	Kovtunov KV, Kidd BE, Salnikov OG, et al (2017) Imaging of Biomolecular NMR Signals Amplified by Reversible Exchange with Parahydrogen Inside an MRI Scanner. J Phys Chem C 121:25994–25999.

73. 	Buckenmaier K, Rudolph M, Back C, et al (2017) SQUID-based detection of ultra-low-field multinuclear NMR of substances hyperpolarized using signal amplification by reversible exchange. Sci Rep 7:13431.

74. 	Barskiy DA, Kovtunov KV, Koptyug IV, et al (2014) The Feasibility of Formation and Kinetics of NMR Signal Amplification by Reversible Exchange (SABRE) at High Magnetic Field (9.4 T). J Am Chem Soc 136:3322–3325.

75. 	Knecht S, Kiryutin AS, Yurkovskaya AV, Ivanov KL (2018) Mechanism of spontaneous polarization transfer in high-field SABRE experiments. J Magn Reson 287:74–81.

76. 	Mewis RE, Green RA, Cockett MCR, et al (2015) Strategies for the Hyperpolarization of Acetonitrile and Related Ligands by SABRE. J Phys Chem B 119:1416–1424.

77. 	Moreno KX, Nasr K, Milne M, et al (2015) Nuclear Spin Hyperpolarization of the Solvent Using Signal Amplification by Reversible Exchange (SABRE). J Magn Reson San Diego Calif 1997 257:15–23.

78. 	Olaru AM, Roy SS, Lloyd LS, et al (2016) Creating a hyperpolarised pseudo singlet state through polarisation transfer from parahydrogen under SABRE. Chem Commun 52:7842–7845.

79. 	Olaru AM, Burt A, Rayner PJ, et al (2016) Using signal amplification by reversible exchange (SABRE) to hyperpolarise 119Sn and 29Si NMR nuclei. Chem Commun 52:14482–14485.

80. 	Roy SS, Appleby KM, Fear EJ, Duckett SB (2018) SABRE-Relay: A Versatile Route to Hyperpolarization. J Phys Chem Lett 9:1112–1117.

81. 	Reineri F, Boi T, Aime S (2015) ParaHydrogen Induced Polarization of 13C carboxylate resonance in acetate and pyruvate. Nat Commun 6:

82. 	Cavallari E, Carrera C, Aime S, Reineri F (2017) 13 C MR Hyperpolarization of Lactate by Using ParaHydrogen and Metabolic Transformation in Vitro. Chem - Eur J 23:1200–1204.

83. 	Cavallari E, Carrera C, Aime S, Reineri F (2018) Studies to enhance the hyperpolarization level in PHIP-SAH-produced C13-pyruvate. J Magn Reson 289:12–17.

84. 	Trantzschel T, Bernarding J, Plaumann M, et al (2012) Parahydrogen induced polarization in face of keto–enol tautomerism: proof of concept with hyperpolarized ethanol. Phys Chem Chem Phys 14:5601.

85. 	Körner M, Sauer G, Heil A, et al (2013) PHIP-label: parahydrogen-induced polarization in propargylglycine-containing synthetic oligopeptides. Chem Commun 49:7839.

86. 	Cavallari E, Carrera C, Sorge M, et al (2018) The 13 C hyperpolarized pyruvate generated by ParaHydrogen detects the response of the heart to altered metabolism in real time. Sci Rep 8:8366.

87. 	Koptyug IV, Kovtunov KV, Burt SR, et al (2007) para-Hydrogen-Induced Polarization in Heterogeneous Hydrogenation Reactions. J Am Chem Soc 129:5580–5586.

88. 	Glöggler Stefan, Grunfeld Alexander M., Ertas Yavuz N., et al (2015) A Nanoparticle Catalyst for Heterogeneous Phase Para‐Hydrogen‐Induced Polarization in Water. Angew Chem Int Ed 54:2452–2456.

89. 	Reineri Francesca, Viale Alessandra, Ellena Silvano, et al (2011) Use of Labile Precursors for the Generation of Hyperpolarized Molecules from Hydrogenation with Parahydrogen and Aqueous‐Phase Extraction. Angew Chem Int Ed 50:7350–7353.

90. 	Hövener J-B, Chekmenev EY, Harris KC, et al (2009) Quality assurance of PASADENA hyperpolarization for <Superscript>13</Superscript>C biomolecules. Magn Reson Mater Phys Biol Med 22:123–134.

91. 	Zacharias NM, Chan HR, Sailasuta N, et al (2012) Real-Time Molecular Imaging of Tricarboxylic Acid Cycle Metabolism in Vivo by Hyperpolarized 1-13C Diethyl Succinate. J Am Chem Soc 134:934–943.

92. 	Shchepin RV, Pham W, Chekmenev EY (2014) Dephosphorylation and biodistribution of 1-13C-phospholactate in vivo. J Label Compd Radiopharm 57:517–524.

93. 	Schmidt AB, Berner S, Braig M, et al (2018) In vivo 13C-MRI using SAMBADENA. PLOS ONE 13:e0200141.

94. 	Zeng H, Xu J, Gillen J, et al (2013) Optimization of SABRE for polarization of the tuberculosis drugs pyrazinamide and isoniazid. J Magn Reson 237:73–78.

95. 	Hoevener J-B, Schwaderlapp N, Lickert T, et al (2013) A hyperpolarized equilibrium for magnetic resonance. Nat Commun 4:

96. 	Colell JFP, Emondts M, Logan AWJ, et al (2017) Direct Hyperpolarization of Nitrogen-15 in Aqueous Media with Parahydrogen in Reversible Exchange. J Am Chem Soc 139:7761–7767.

97. 	Theis T, Truong ML, Coffey AM, et al (2015) Microtesla SABRE Enables 10% Nitrogen-15 Nuclear Spin Polarization. J Am Chem Soc 137:1404–1407.

98. 	Colell JFP, Logan AWJ, Zhou Z, et al (2017) Generalizing, Extending, and Maximizing Nitrogen-15 Hyperpolarization Induced by Parahydrogen in Reversible Exchange. J Phys Chem C 121:6626–6634.

99. 	Rovedo P, Knecht S, Bäumlisberger T, et al (2016) Molecular MRI in the Earth’s Magnetic Field Using Continuous Hyperpolarization of a Biomolecule in Water. J Phys Chem B 120:5670–5677.

100. 	Hövener J-B, Knecht S, Schwaderlapp N, et al (2014) Continuous Re-hyperpolarization of Nuclear Spins Using Parahydrogen: Theory and Experiment. ChemPhysChem 15:2451–2457.

101. 	Frossati G (1998) Polarization of <Superscript>3</Superscript>He, D<Subscript>2</Subscript> and (eventually) <Superscript>129</Superscript>Xe Using Low Temperatures and High Magnetic Fields. J Low Temp Phys 111:521–532.

102. 	Krjukov EV, O’Neill JD, Owers-Bradley JR (2005) Brute Force Polarization of <Superscript>129</Superscript>Xe. J Low Temp Phys 140:397–408.

103. 	Hirsch ML, Kalechofsky N, Belzer A, et al (2015) Brute-Force Hyperpolarization for NMR and MRI. J Am Chem Soc 137:8428–8434.

104. 	Cates GD, Benton DR, Gatzke M, et al (1990) Laser production of large nuclear-spin polarization in frozen xenon. Phys Rev Lett 65:2591–2594.

105. 	Gatzke M, Cates GD, Driehuys B, et al (1993) Extraordinarily slow nuclear spin relaxation in frozen laser-polarized $^{129}\mathrm{Xe}$. Phys Rev Lett 70:690–693.

106. 	Walker TG, Happer W (1997) Spin-exchange optical pumping of noble-gas nuclei. Rev Mod Phys 69:629–642.

107. 	Chupp TE, Coulter KP (1985) Polarization of Ne 21 by Spin Exchange with Optically Pumped Rb Vapor. Phys Rev Lett 55:1074–1077.

108. 	Pavlovskaya GE, Cleveland ZI, Stupic KF, et al (2005) Hyperpolarized krypton-83 as a contrast agent for magnetic resonance imaging. Proc Natl Acad Sci U S A 102:18275–18279.

109. 	Stupic KF, Cleveland ZI, Pavlovskaya GE, Meersmann T (2011) Hyperpolarized 131Xe NMR spectroscopy. J Magn Reson 208:58–69.

110. 	Chupp TE, Wagshul ME, Coulter KP, et al (1987) Polarized, high-density, gaseous He 3 targets. Phys Rev C 36:2244–2251.

111. 	Spence MM, Rubin SM, Dimitrov IE, et al (2001) Functionalized xenon as a biosensor. Proc Natl Acad Sci 98:10654–10657.

112. 	Schröder L, Lowery TJ, Hilty C, et al (2006) Molecular imaging using a targeted magnetic resonance hyperpolarized biosensor. Science 314:446–449.

113. 	Desvaux H, Gautier T, Le Goff G, et al (2000) Direct evidence of a magnetization transfer between laser-polarized xenon and protons of a cage-molecule in water. Eur Phys J D 12:289–296.

114. 	Bai Y, Wang Y, Goulian M, et al (2014) Bacterial spore detection and analysis using hyperpolarized 129 Xe chemical exchange saturation transfer (Hyper-CEST) NMR. Chem Sci 5:3197–3203

115. 	Hoffmann HC, Brunner E (2015) Studies of Metal–Organic Frameworks: Xenon for Probing Framework Porosity, Breathing and Gating Behavior. In: Hyperpolarized xenon-129 magnetic resonance: concepts, production, techniques, and applications. Eds. Meersmann T, Brunner E. Cambridge: Royal Society Of Chemistry, , pp 234-248.

116. 	Dregely I, Mugler JP, Ruset IC, et al (2011) Hyperpolarized Xenon-129 gas-exchange imaging of lung microstructure: First case studies in subjects with obstructive lung disease. J Magn Reson Imaging 33:1052–1062.

117. 	Rao M, Stewart NJ, Norquay G, et al (2016) High resolution spectroscopy and chemical shift imaging of hyperpolarized 129Xe dissolved in the human brain in vivo at 1.5 tesla. Magn Reson Med 75:2227–2234.

118. 	Branca RT, He T, Zhang L, et al (2014) Detection of brown adipose tissue and thermogenic activity in mice by hyperpolarized xenon MRI. Proc Natl Acad Sci U S A 111:18001–18006.

119. 	Shapiro MG, Ramirez RM, Sperling LJ, et al (2014) Genetically encoded reporters for hyperpolarized xenon magnetic resonance imaging. Nat Chem 6:629–634.

120. 	Nikolaou P, Coffey AM, Walkup LL, et al (2014) XeNA: An automated ‘open-source’ 129Xe hyperpolarizer for clinical use. Magn Reson Imaging 32:541–550.

121. 	Ebner L, He M, Virgincar RS, et al (2017) Hyperpolarized 129Xenon Magnetic Resonance Imaging to Quantify Regional Ventilation Differences in Mild to Moderate Asthma: A Prospective Comparison Between Semiautomated Ventilation Defect Percentage Calculation and Pulmonary Function Tests. Invest Radiol 52:120.

122. 	Freeman MS, Emami K, Driehuys B (2014) Characterizing and modeling the efficiency limits in large-scale production of hyperpolarized $^{129}\mathrm{Xe}$. Phys Rev A 90:023406.

123. 	Burant A, Branca RT (2016) Diffusion-mediated 129 Xe gas depolarization in magnetic field gradients during continuous-flow optical pumping. J Magn Reson 273:124–129.

124. 	Goodson BM, Whiting N, Newton H, et al (2015) Chapter 6:Spin-exchange Optical Pumping at High Xenon Densities and Laser Fluxes: Principles and Practice. In: Hyperpolarized Xenon-129 Magnetic Resonance. pp 96–121

125. 	Jeong K, Netirojjanakul C, Munch HK, et al (2016) Targeted molecular imaging of cancer cells using MS2-based 129Xe NMR. Bioconjug Chem 27:1796–1801

126. 	Hane FT, Li T, Smylie P, et al (2017) In vivo detection of cucurbit [6] uril, a hyperpolarized xenon contrast agent for a xenon magnetic resonance imaging biosensor. Sci Rep 7:41027

127. 	Leupold J, Månsson S, Petersson JS, et al (2009) Fast multiecho balanced SSFP metabolite mapping of 1H and hyperpolarized 13C compounds. Magn Reson Mater Phys Biol Med 22:251–256.

128. 	Larson PEZ, Hu S, Lustig M, et al (2011) Fast dynamic 3D MR spectroscopic imaging with compressed sensing and multiband excitation pulses for hyperpolarized 13C studies. Magn Reson Med 65:610–619.

129. 	Daniels CJ, McLean MA, Schulte RF, et al (2016) A comparison of quantitative methods for clinical imaging with hyperpolarized 13C-pyruvate. NMR Biomed 29:387–399.

130. 	Schulte RF, Sperl JI, Weidl E, et al (2013) Saturation-recovery metabolic-exchange rate imaging with hyperpolarized [1-13C] pyruvate using spectral-spatial excitation. Magn Reson Med 69:1209–1216.

131. 	Morze C von, Larson PEZ, Hu S, et al (2011) Imaging of blood flow using hyperpolarized [13C]Urea in preclinical cancer models. J Magn Reson Imaging 33:692–697.

132. 	Månsson S, Petersson JS, Scheffler K (2012) Fast metabolite mapping in the pig heart after injection of hyperpolarized 13C-pyruvate with low-flip angle balanced steady-state free precession imaging. Magn Reson Med 68:1894–1899.

133. 	Larson PEZ, Hurd RE, Kerr AB, et al (2013) Perfusion and diffusion sensitive 13C stimulated-echo MRSI for metabolic imaging of cancer. Magn Reson Imaging 31:635–642.

134. 	Swisher CL, Larson PEZ, Kruttwig K, et al (2013) Quantitative measurement of cancer metabolism using stimulated echo hyperpolarized carbon-13 MRS. Magn Reson Med 71:1–11.

135. 	Lau AZ, Chen AP, Barry J, et al (2013) Reproducibility study for free‐breathing measurements of pyruvate metabolism using hyperpolarized 13C in the heart. Magn Reson Med 69:1063–1071.

136. 	Hu S, Lustig M, Chen AP, et al (2008) Compressed sensing for resolution enhancement of hyperpolarized 13C flyback 3D-MRSI. J Magn Reson 192:258–264.

137. 	Hu S, Lustig M, Balakrishnan A, et al (2010) 3D compressed sensing for highly accelerated hyperpolarized 13C MRSI with in vivo applications to transgenic mouse models of cancer. Magn Reson Med 63:312–321.

138. 	Shin PJ, Larson PEZ, Ohliger MA, et al (2014) Calibrationless parallel imaging reconstruction based on structured low-rank matrix completion. Magn Reson Med 72:959–970.

139. 	Schmidt R, Frydman L (2014) New spatiotemporal approaches for fully refocused, multislice ultrafast 2D MRI. Magn Reson Med 71:711–722.

140. 	Zhou L, Cabrera ME, Okere IC, et al (2006) Regulation of myocardial substrate metabolism during increased energy expenditure: insights from computational studies. Am J Physiol-Heart Circ Physiol 291:H1036–H1046.

141. 	Gallagher FA, Bohndiek SE, Kettunen MI, et al (2011) Hyperpolarized 13C MRI and PET: In Vivo Tumor Biochemistry. J Nucl Med 52:1333–1336.

142. 	Albers MJ, Bok R, Chen AP, et al (2008) Hyperpolarized 13C Lactate, Pyruvate, and Alanine: Noninvasive Biomarkers for Prostate Cancer Detection and Grading. Cancer Res 68:8607–8615.

143. 	Day SE, Kettunen MI, Gallagher FA, et al (2007) Detecting tumor response to treatment using hyperpolarized 13C magnetic resonance imaging and spectroscopy. Nat Med 13:1382–1387.

144. 	Yoshihara HAI, Bastiaansen JAM, Berthonneche C, et al (2015) An intact small animal model of myocardial ischemia-reperfusion: Characterization of metabolic changes by hyperpolarized 13C MR spectroscopy. Am J Physiol Heart Circ Physiol 309:H2058-2066.

145. 	Golman Klaes, Petersson J. Stefan, Magnusson Peter, et al (2008) Cardiac metabolism measured noninvasively by hyperpolarized 13C MRI. Magn Reson Med 59:1005–1013.

146. 	Menichetti Luca, Frijia Francesca, Flori Alessandra, et al (2012) Assessment of real‐time myocardial uptake and enzymatic conversion of hyperpolarized [1‐13C]pyruvate in pigs using slice selective magnetic resonance spectroscopy. Contrast Media Mol Imaging 7:85–94.

147. 	Schroeder MA, Lau AZ, Chen AP, et al (2013) Hyperpolarized 13C magnetic resonance reveals early- and late-onset changes to in vivo pyruvate metabolism in the failing heart. Eur J Heart Fail 15:130–140.

148. 	Kennedy BWC, Kettunen MI, Hu D-E, Brindle KM (2012) Probing Lactate Dehydrogenase Activity in Tumors by Measuring Hydrogen/Deuterium Exchange in Hyperpolarized l-[1-13C,U-2H]Lactate. J Am Chem Soc 134:4969–4977.

149. 	Shchepin RV, Coffey AM, Waddell KW, Chekmenev EY (2014) Parahydrogen Induced Polarization of 1-13C-Phospholactate-d2 for Biomedical Imaging with >30,000,000-fold NMR Signal Enhancement in Water. Anal Chem 86:5601–5605.

150. 	Stanley WC, Recchia FA, Lopaschuk GD (2005) Myocardial substrate metabolism in the normal and failing heart. Physiol Rev 85:1093–1129.

151. 	Merritt ME, Harrison C, Storey C, et al (2007) Hyperpolarized 13C allows a direct measure of flux through a single enzyme-catalyzed step by NMR. Proc Natl Acad Sci 104:19773–19777.

152. 	Schroeder MA, Cochlin LE, Heather LC, et al (2008) In vivo assessment of pyruvate dehydrogenase flux in the heart using hyperpolarized carbon-13 magnetic resonance. Proc Natl Acad Sci 105:12051–12056.

153. 	Atherton HJ, Dodd MS, Heather LC, et al (2011) The Role of PDH Inhibition in the Development of Hypertrophy in the Hyperthyroid Rat Heart: A Combined MRI and Hyperpolarized MRS Study. Circulation 123:2552–2561.

154. 	Flori A, Frijia F, Lionetti V, et al (2012) DNP Methods for Cardiac Metabolic Imaging with Hyperpolarized [1-13C]pyruvate Large Dose Injection in Pigs. Appl Magn Reson 43:299–310.

155. 	Chen Albert P., Lau Justin Y.C., Alvares Rohan D.A., Cunningham Charles H. (2014) Using [1‐13C]lactic acid for hyperpolarized 13C MR cardiac studies. Magn Reson Med 73:2087–2093.

156. 	Gallagher FA, Kettunen MI, Day SE, et al (2008) Magnetic resonance imaging of pH in vivo using hyperpolarized 13C-labelled bicarbonate. Nature 453:940–943.

157. 	Gallagher FA, Kettunen MI, Brindle KM (2011) Imaging pH with hyperpolarized 13C. NMR Biomed 24:1006–1015.

158. 	Gallagher FA, Sladen H, Kettunen MI, et al (2015) Carbonic Anhydrase Activity Monitored In Vivo by Hyperpolarized 13C-Magnetic Resonance Spectroscopy Demonstrates Its Importance for pH Regulation in Tumors. Cancer Res 75:4109–4118.

159. 	Rider OJ, Tyler DJ (2013) Clinical Implications of Cardiac Hyperpolarized Magnetic Resonance Imaging. J Cardiovasc Magn Reson 15:93.

160. 	Düwel S, Hundshammer C, Gersch M, et al (2017) Imaging of pH in vivo using hyperpolarized 13C-labelled zymonic acid. Nat Commun 8:15126.

161. 	Day SE, Kettunen MI, Cherukuri MK, et al (2011) Detecting response of rat C6 glioma tumors to radiotherapy using hyperpolarized [1-13C]pyruvate and 13C magnetic resonance spectroscopic imaging. Magn Reson Med 65:557–563.

162. 	Bohndiek SE, Kettunen MI, Hu D, et al (2011) Hyperpolarized [1-13C]-Ascorbic and Dehydroascorbic Acid: Vitamin C as a Probe for Imaging Redox Status in Vivo. J Am Chem Soc 133:11795–11801.

163. 	Hu S, Chen AP, Zierhut ML, et al (2009) In Vivo Carbon-13 Dynamic MRS and MRSI of Normal and Fasted Rat Liver with Hyperpolarized 13C-Pyruvate. Mol Imaging Biol 11:399–407.

164. 	Hu S, Zhu M, Yoshihara HAI, et al (2011) In vivo measurement of normal rat intracellular pyruvate and lactate levels after injection of hyperpolarized [1-(13)C]alanine. Magn Reson Imaging 29:1035–1040.

165. 	Atherton HJ, Dodd MS, Heather LC, et al (2011) Role of Pyruvate Dehydrogenase Inhibition in the Development of Hypertrophy in the Hyperthyroid Rat HeartClinical Perspective: A Combined Magnetic Resonance Imaging and Hyperpolarized Magnetic Resonance Spectroscopy Study. Circulation 123:2552–2561.

166. 	Josan S, Park JM, Hurd R, et al (2013) In vivo investigation of cardiac metabolism in the rat using MRS of hyperpolarized [1-13C] and [2-13C]pyruvate. NMR Biomed 26:

167. 	Chen Albert P., Hurd Ralph E., Schroeder Marie A., et al (2011) Simultaneous investigation of cardiac pyruvate dehydrogenase flux, Krebs cycle metabolism and pH, using hyperpolarized [1,2‐13C2]pyruvate in vivo. NMR Biomed 25:305–311.

168. 	Dodd MS, Ball DR, Schroeder MA, et al (2012) In vivo alterations in cardiac metabolism and function in the spontaneously hypertensive rat heart. Cardiovasc Res 95:69–76.

169. 	Schroeder MA, Ali MA, Hulikova A, et al (2013) Extramitochondrial domain rich in carbonic anhydrase activity improves myocardial energetics. Proc Natl Acad Sci 110:E958–E967.

170. 	Schroeder MA, Atherton HJ, Dodd MS, et al (2012) The Cycling of Acetyl-Coenzyme A Through Acetylcarnitine Buffers Cardiac Substrate SupplyClinical Perspective: A Hyperpolarized 13C Magnetic Resonance Study. Circ Cardiovasc Imaging 5:201–209.

171. 	Schroeder MA, Atherton HJ, Ball DR, et al (2009) Real-time assessment of Krebs cycle metabolism using hyperpolarized 13C magnetic resonance spectroscopy. FASEB J 23:2529–2538.

172. 	Koellisch Ulrich, Gringeri Concetta V., Rancan Giaime, et al (2014) Metabolic imaging of hyperpolarized [1‐13C]acetate and [1‐13C]acetylcarnitine ‐ investigation of the influence of dobutamine induced stress. Magn Reson Med 74:1011–1018.

173. 	Flori Alessandra, Liserani Matteo, Frijia Francesca, et al (2014) Real‐time cardiac metabolism assessed with hyperpolarized [1‐13C]acetate in a large‐animal model. Contrast Media Mol Imaging 10:194–202.

174. 	Mishkovsky M, Comment A, Gruetter R (2012) In Vivo Detection of Brain Krebs Cycle Intermediate by Hyperpolarized Magnetic Resonance. J Cereb Blood Flow Metab 32:2108–2113.

175. 	Mikkelsen EFR, Mariager CØ, Nørlinger T, et al (2017) Hyperpolarized [1-13C]-acetate Renal Metabolic Clearance Rate Mapping. Sci Rep 7:16002.

176. 	Jensen PR, Peitersen T, Karlsson M, et al (2009) Tissue-specific Short Chain Fatty Acid Metabolism and Slow Metabolic Recovery after Ischemia from Hyperpolarized NMR in Vivo. J Biol Chem 284:36077–36082.

177. 	Bastiaansen JAM, Cheng T, Lei H, et al (2015) Direct noninvasive estimation of myocardial tricarboxylic acid cycle flux in vivo using hyperpolarized 13C magnetic resonance. J Mol Cell Cardiol 87:129–137.

178. 	R. Jensen P, Meier S, H. Ardenkjær-Larsen J, et al (2009) Detection of low-populated reaction intermediates with hyperpolarized NMR. Chem Commun 0:5168–5170.

179. 	Yoshihara H, Bastiaansen JA, Karlsson M, et al (2015) Myocardial fatty acid metabolism probed with hyperpolarized [1-13C]octanoate. J Cardiovasc Magn Reson 17:O101.

180. 	Ball Daniel R., Rowlands Ben, Dodd Michael S., et al (2014) Hyperpolarized butyrate: A metabolic probe of short chain fatty acid metabolism in the heart. Magn Reson Med 71:1663–1669.

181. 	Bastiaansen JA, Merritt ME, Comment A (2015) Real time measurement of myocardial substrate selection in vivo using hyperpolarized <Superscript>13</Superscript>C magnetic resonance. J Cardiovasc Magn Reson 17:O15.

182. 	Bastiaansen JAM, Merritt ME, Comment A (2016) Measuring changes in substrate utilization in the myocardium in response to fasting using hyperpolarized [1-13C]butyrate and [1-13C]pyruvate. Sci Rep 6:

183. 	Billingsley KL, Josan S, Park JM, et al Hyperpolarized [1,4-13C]-diethylsuccinate: a potential DNP substrate for in vivo metabolic imaging. NMR Biomed 27:356–362.

184. 	Jakob U, Reichmann D (2013) Oxidative stress and redox regulation. Dodrecht Heidelberg New York London: Springer.

185. 	Maritim AC, Sanders RA, Watkins JB Diabetes, oxidative stress, and antioxidants: A review. J Biochem Mol Toxicol 17:24–38.

186. 	Keshari KR, Sai V, Wang ZJ, et al (2013) Hyperpolarized [1-13C]Dehydroascorbate MR Spectroscopy in a Murine Model of Prostate Cancer: Comparison with 18F-FDG PET. J Nucl Med 54:922–928.

187. 	Timm KN, Hu D-E, Williams M, et al (2017) Assessing Oxidative Stress in Tumors by Measuring the Rate of Hyperpolarized [1-13C]Dehydroascorbic Acid Reduction Using 13C Magnetic Resonance Spectroscopy. J Biol Chem 292:1737–1748.

188. 	Keshari KR, Wilson DM, Sai V, et al (2015) Noninvasive In Vivo Imaging of Diabetes-Induced Renal Oxidative Stress and Response to Therapy Using Hyperpolarized 13C Dehydroascorbate Magnetic Resonance. Diabetes 64:344–352.

189. 	N. Carroll V, Truillet C, Shen B, et al (2016) [ 11 C]Ascorbic and [ 11 C]dehydroascorbic acid, an endogenous redox pair for sensing reactive oxygen species using positron emission tomography. Chem Commun 52:4888–4890.

190. 	Lowery TJ, Garcia S, Chavez L, et al (2006) Optimization of xenon biosensors for detection of protein interactions. ChemBioChem 7:65–73

191. 	Aaron JA, Chambers JM, Jude KM, et al (2008) Structure of a 129Xe-cryptophane biosensor complexed with human carbonic anhydrase II. J Am Chem Soc 130:6942–6943

192. 	Chambers JM, Hill PA, Aaron JA, et al (2008) Cryptophane xenon-129 nuclear magnetic resonance biosensors targeting human carbonic anhydrase. J Am Chem Soc 131:563–569

193. 	Witte C, Martos V, Rose HM, et al (2015) Live‐cell MRI with Xenon Hyper‐CEST Biosensors Targeted to Metabolically Labeled Cell‐Surface Glycans. Angew Chem Int Ed 54:2806–2810

194. 	Seward GK, Bai Y, Khan NS, Dmochowski IJ (2011) Cell-compatible, integrin-targeted cryptophane-129Xe NMR biosensors. Chem Sci 2:1103–1110

195. 	Palaniappan KK, Ramirez RM, Bajaj VS, et al (2013) Molecular Imaging of Cancer Cells Using a Bacteriophage‐Based 129Xe NMR Biosensor. Angew Chem 125:4949–4953

196. 	Boutin C, Stopin A, Lenda F, et al (2011) Cell uptake of a biosensor detected by hyperpolarized 129Xe NMR: The transferrin case. Bioorg Med Chem 19:4135–4143

197. 	Zeng Q, Guo Q, Yuan Y, et al (2017) Mitochondria Targeted and Intracellular Biothiol Triggered Hyperpolarized 129Xe Magnetofluorescent Biosensor. Anal Chem 89:2288–2295

198. 	Schröder L (2013) Xenon for NMR biosensing–Inert but alert. Phys Medica Eur J Med Phys 29:3–16

199. 	Klippel S, Freund C, Schröder L (2014) Multichannel MRI labeling of mammalian cells by switchable nanocarriers for hyperpolarized xenon. Nano Lett 14:5721–5726

200. 	Wang Y, Dmochowski IJ (2016) An Expanded Palette of Xenon-129 NMR Biosensors. Acc Chem Res 49:2179–2187.

201. 	Zaccagna F, Grist JT, Deen SS, et al (2018) Hyperpolarized carbon-13 magnetic resonance spectroscopic imaging: a clinical tool for studying tumour metabolism. Br J Radiol 20170688.

202. 	Stewart NJ, Chan H-F, Hughes PJC, et al (2018) Comparison of 3 He and 129 Xe MRI for evaluation of lung microstructure and ventilation at 1.5T. J Magn Reson Imaging JMRI.

203. 	Kirby M, Ouriadov A, Svenningsen S, et al (2014) Hyperpolarized 3He and 129Xe magnetic resonance imaging apparent diffusion coefficients: physiological relevance in older never‐ and ex‐smokers. Physiol Rep 2: .

204. 	Kirby M, Svenningsen S, Kanhere N, et al (2013) Pulmonary ventilation visualized using hyperpolarized helium-3 and xenon-129 magnetic resonance imaging: differences in COPD and relationship to emphysema. J Appl Physiol 114:707–715.

205. 	Svenningsen Sarah, Kirby Miranda, Starr Danielle, et al (2013) Hyperpolarized 3He and 129Xe MRI: Differences in asthma before bronchodilation. J Magn Reson Imaging 38:1521–1530.

206. 	Horn FC, Marshall H, Collier GJ, et al (2017) Regional Ventilation Changes in the Lung: Treatment Response Mapping by Using Hyperpolarized Gas MR Imaging as a Quantitative Biomarker. Radiology 284:854–861.

Figure Legends

Figure 1: Schematic view of spin-½ nuclei populating two energy levels in a magnetic field at thermal equilibrium (left) and after hyperpolarisation (right). At thermal equilibrium, both energy levels are populated according to the Boltzmann distribution. At clinical field strengths, thermal energy dominates the population (left), and both energy levels are almost equally populated. Only the population difference contributes effectively to the MR signal, indicated in red. The goal of hyperpolarisation is to increase the population of one energy level so that the MRI signal is also increased (right). Reproduced with permission [9, 10].

Figure 2: Schematic view of a typical hyperpolarisation experiment. After the production of the HP tracer in the polariser, the solution is injected into the bloodstream and distributed in vivo. Labelled [1-13C]acetate is shown as an example and undergoes metabolism to [1-13C]acetylcarnitine and [5-13C]citrate. The signals of the injected and metabolised molecules can be detected by spectroscopic or imaging methods in real time. Reproduced with permission [9, 10, 16].

Figure 3: Schematic views of Dynamic Nuclear Polarisation (a, DNP), parahydrogen (pH2) - based methods Parahydrogen And Synthesis Allow Dramatically Enhanced Nuclear Alignment (b, PASADENA) and Signal Amplification By Reversible Exchange (c, SABRE) - a pH2-based technique that enables reproducible hyperpolarisation - and Spin-Exchange Optical Pumping (d, SEOP). Reproduced with permission [9, 10].

Figure 4: To the left: schematic representation of the metabolic pathways involving pyruvate: the reduction into lactate, the transamination into alanine, and the conversion into bicarbonate can be tracked using hyperpolarised [1-13C]pyruvate, while the conversion into Acetyl CoA and subsequent incorporation into the Krebs cycle can be studied with hyperpolarised [2-13C]pyruvate (below). To the right, a schematic representation of the highly regulated metabolic pathway involving free-fatty-acid (FFA) is shown. In the figure, the enzymes catalysing the different metabolic reactions are highlighted in light green.

Figure 5: In vivo metabolic maps of HP [1-13C]pyruvate, [1-13C]lactate, [1-13C]bicarbonate obtained in the pig heart at 3 T after injection of 230 mM HP [1-13C]pyruvate. Metabolic maps were obtained using 3D-IDEAL spiral CSI sequence (FOV = 30 cm2, slab thickness = 100 mm). Modified from [154]. Reproduced with permission.

Figure 6: a) Imaging response to radiotherapy in a rat glioma model: Images of hyperpolarised 13C-pyruvate (iii) and lactate (iv) before (top) and 96 h after radiotherapy (bottom). The lactate signal was reduced following exposure to 15 Gy radiation in spite an apparent increase in tumour size in (ii) [161]. b) HP 13C pH imaging in a tumour bearing mouse: The tumour region is outlined in red in (i), a proton MRI. (ii) shows a pH map of the same animal calculated from the ratio of the H13CO3– (iii) and 13CO2 (iv) voxel intensities in 13C chemical shift images acquired ~10 s after intravenous injection of ~100 mM hyperpolarised H13CO3-. pH is shown to be reduced in the tumour region [156]. c) Imaging redox status in murine lymphoma models: 13C MR spectra acquired from a 6-mm tumour slice following injection of 30 mM [1-13C]ascorbic acid (i) and [1-13C]dehydroascorbic acid (ii). Significant flux was observed from [1-13C]-DHA (175 ppm) to [1-13C]-AA (179 ppm) (ii) indicating rapid reduction of the injected [1-13C]-DHA in the tumour, while no observable flux was observed from [1-13C]-A to [1-13C]-DHA (a) [162]. Reproduced with permission.

Figure 7: The Xe biosensor concept: a) 129Xe NMR spectrum of xenon dissolved in water with 5 % DMSO and with 200 mM of the molecular cage cryptophane-A monoacid. Xenon in gas phase is the reference at 0 ppm. Free Xe in solution resonates at ca. 191 ppm whereas bound atoms resonate at ca. 61.8 ppm. Cryoptophane-A is depicted in the top right [198]; b) A targeted, selective, and highly sensitive 129Xe NMR nanoscale biosensor showing strong binding specificity toward targeted lymphoma cells (Ramos line) [125]. Reproduced with permission.

Figure 8: Top: Proton T2-weighted (T2W), apparent diffusion coefficient (ADC), and T2W images with an overlaid pyruvate-to-lactate metabolic flux (kPL) image, with corresponding HP 13C spectral array (SA) for a 52-yr-old prostate cancer patient (A) before therapy and (B) 6 wk after initiation of androgen ablation and chemotherapy. Before treatment, the prostate cancer can be clearly seen (red arrows) as a reduction in signal on the T2W and ADC images, and increased HP lactate and associated kPL flux on HP 13C MRI. After therapy there was a significant reduction in HP lactate and kPL to normal levels, with only a modest treatment effect on prostate volume and ADC [47]. Bottom: Representative 13C images overlaid on grayscale anatomical images in a mid-left ventricle (LV) slice from two healthy volunteers (a-c) and (d-f), respectively. [1-13C]pyruvate is visible mainly in the blood pool within the cardiac chambers (a,d). 13C-bicarbonate appears predominantly in the all of the LV (b,e). [1-13C]lactate (c,f) appeared with a diffuse distribution covering the muscle and chambers [50]. Reproduced with permission.