Zinc Delivery from Non-woven Fibres within a Therapeutic Nipple Shield

Theresa Maiera,b, Rebekah L. Scheuerlea, Daniel Markla, Sylvaine Bruggraberc, 
Axel Zeitlera, Ljiljana Fruka, Nigel K. H. Slatera

a University of Cambridge, Department of Chemical Engineering and Biotechnology, West Cambridge Site, Philippa Fawcett Drive, Cambridge CB3 0AS, United Kingdom

b University of Cambridge, Department of Paediatrics, Cambridge Biomedical Campus, Cambridge CB2 0QQ, United Kingdom

c Medical Research Council Elsie Widdowson Laboratory, 120 Fulbourn Road, 
Cambridge CB1 9NL, United Kingdom

Corresponding author

Name: 			Theresa Maier

Address: 	Department of Chemical Engineering and Biotechnology, 
University of Cambridge

West Cambridge Site, Philippa Fawcett Drive, Cambridge CB3 0AS,
UK

Telephone number: 	+44 (0)7778627858

Fax number:		+44 (0)1223 334796

E-mail address: 	tm520@cam.ac.uk

Abstract

A Therapeutic Nipple Shield (TNS) was previously developed to respond to the global need for new infant therapeutic delivery technologies. However, the release efficiency for the same Active Pharmaceutical Ingredient (API) from different therapeutic matrices within the TNS formulation has not yet been investigated. To address this, in-vitro release of elemental zinc into human milk from two types of Texel non-woven fibre mats of varying thickness and different gram per square meter values, placed inside the TNS was explored and compared to the release from zinc-containing rapidly disintegrating tablets. In-vitro delivery was performed by means of a breastfeeding simulation apparatus, with human milk flow rates and suction pressure adjusted to physiologically relevant values, and release was quantified using Inductively Coupled Plasma Optical Emission Spectrometry (ICP-OES). It was found that a total recovery of 62 - 64 % elemental zinc was obtained after the human milk had passed through the fibre insert, amounting to a 20 - 48% increase compared to previous zinc delivery studies using rapidly disintegrating tablets within the TNS. This indicates that non-woven Texel fibre mats were identified as the superior dosage form for oral zinc delivery into human milk using a TNS.

Keywords: Pediatrics; Oral delivery; Breastfeeding; Human milk; Medication systems; X-ray micro computed tomography

1. Introduction

New infant therapeutic delivery technologies are urgently needed. According to the World Health Organization (WHO), many of the 5.9 million child deaths worldwide before the age of five could be prevented by enabling access to affordable interventions, capable of addressing the current challenges of low-resource settings, such as lack of potable water or sterilisation (IGME, 2015; World Health Organization, 2007). This clearly indicates that novel, affordable and easy-to-use infant therapeutic delivery technologies are urgently required. To address current limitations, a Therapeutic Nipple Shield (TNS) was developed (Gerrard et al., 2012), which enables the delivery of Active Pharmaceutical Ingredients (APIs) during breastfeeding. It consists of a nipple shield device containing a therapeutic insert, for example in the form of an API-loaded tablet or fibrous material. When worn by a mother during breastfeeding, such a device enriches the human milk consumed by the sucking infant with medication or nutrients (see Figure 1) (Gerrard et al., 2012).

The TNS has the potential to dramatically change the way drugs and nutrients are administered to infants worldwide, as it could potentially improve the acceptability of pharmaceutical products in both developed and developing countries (Lopez and Ernest, 2015), and offer an alternative to conventional therapeutic delivery for the 36 % of infants aged 0 - 6 months, who are exclusively breastfed (World Health Organization, 2017).

Zinc is a physiologically important element, responsible for the activity of various proteins in major pathways of the human metabolism (Terrin et al., 2015). Its deficiency has extensive effects on growth and development of infants, as well as the capabilities of the body's immune system (Caulfield et al., 1998; Terrin et al., 2015; Wulf et al., 2013). The intestinal absorption of zinc is independent of zinc status, yet higher uptake is reported for term babies as a result of lower zinc concentration in the blood serum (Terrin et al., 2015). Preterm and Small for Gestational Age (SGA) neonates have an increased risk of zinc deficiency (Wulf et al., 2013), which is believed to occur within the first 1 - 2 months of life (Caulfield et al., 1998). Breastfeeding infants obtain zinc through the consumption of human milk, however, zinc concentration can exhibit significant variations (0.7 - 1.6 mg l-1), and constantly declines postpartum: 8 - 12 mg l-1 zinc can be found in colostrum, the first milk the maternal breast produces after giving birth, 3 - 6 mg l-1 in human milk seven days after birth, and only 1- 3 mg l-1 an additional three weeks later (Terrin et al., 2015). To prevent deficiency, enteral delivery of zinc is recommended, being 0.8 mg kg-1 day-1 for term infants and 3 mg kg-1 day-1 for preterm neonates (Hambidge and Krebs, 2004). If zinc deficiency occurs, it is associated with conditions such as dermatitis, growth retardation, necrotizing enterocolitis, neurologic damage, bronchopulmonary dysplasia, infections, and retinopathy of prematurity (Terrin et al., 2015). It is recognized that not only severe, but also moderate zinc deficiency is adding to “the total burden of disease” in infants leading to about 800,000 child deaths every year (Black, 2003). In addition to zinc’s general health promoting effects, the WHO recommends use of zinc (20 mg zinc sulphate) as an adjunct supplement to oral rehydration salts for the treatment of acute diarrhoea (World Health Organization, 2015). Due to the significance of zinc for the physical development and wellbeing of infants globally, its delivery into human milk using the TNS was investigated in this study.

To aid in-vitro testing of the TNS, a breastfeeding simulation apparatus was developed by Gerrard et al., simulating both the process of lactation and infant feeding by mimicking the average flow rates/patterns of milk during breastfeeding and the infant’s suckling pressure (Gerrard et al., 2013). The apparatus was previously used to investigate release of a Sulforhodamine B dye and of zinc sulphate from rapidly disintegrating tablets into human milk (Gerrard et al., 2012; Scheuerle et al., 2017). The capability of non-woven fibres to release APIs into human milk was previously shown by Gerrard et al., who investigated the delivery of Sodium Dodecyl Sulphate (SDS) from loaded non-woven fibres (Bathfelt, Texel, Québec, Canada) located within a Sinnex filter holder, but without the use of breastfeeding simulation (Gerrard et al., 2012). The following research builds on both studies and explored the fibre-based zinc sulphate delivery from a nipple shield device into human milk using the breastfeeding simulation apparatus. It is the first time that absolute recovery of the same compound using different dosage forms within the TNS has been quantitatively compared.

2. Materials and methods

2.1. Materials

In accordance with previous fibre studies by Gerrard et al. (Gerrard et al., 2012), non-woven fibre mats (35% viscose, 65% polyester), provided from Bathfelt, Texel (Québec, Canada) were used as a delivery matrix. Two different types of fibre mats, placed within the same lip-containing nipple shield device as previously applied for the evaluation of zinc delivery via rapidly disintegrating tablets by Scheuerle et al. (Scheuerle et al., 2017), were investigated: a 235 gram per square meter (g m-2) felt with targeted thickness of 1.8 mm (referred to as fibre mat type A), and a 335 g m-2 felt with targeted thickness of 2.1 mm (referred to as fibre mat type B). An illustration can be found in Figure 2. Zinc sulphate monohydrate was obtained from Sigma Aldrich (Dorset, UK), and preferred over elemental zinc based on its superior solubility and taste characteristics (Scheuerle et al., 2017). Human milk samples were supplied by the Queen Charlotte’s and Chelsea Hospital Milk Bank (Imperial College Healthcare NHS Trust), and characterized as outlined in previous literature (Scheuerle et al., 2017), except that the fat content was calculated using an updated creamatocrit to fat correlation, published by Mitoulas et al. (Mitoulas et al., 2002). Human milk used for experiments in this study had a fat content of 43.52 g l-1, and a protein content of 16.51 g l-1. Both values correspond to previously reported literature data (Emmett and Rogers, 1997; Kent et al., 2006; Saarela et al., 2005). The Cambridge Human Biology Research Ethics Committee at the University of Cambridge approved all human milk sample use.

2.2. Methods
2.2.1. Loading procedure

6.2 mg (standard deviation 0.075 mg, n = 3) and 6.4 mg (standard deviation 0.046 mg, n = 3) of elemental zinc was loaded through pipetting 45 μL of a 49 % zinc sulphate monohydrate solution (wt/wt %) onto type A and B fibre mats, respectively. Values were based on the WHO dose recommendation of zinc sulphate as an adjunct to oral rehydration salts for the treatment of acute diarrhoea (World Health Organization, 2015). All fibre mats were air dried in a petri dish for 24 h at room temperature.

2.2.2. Scanning electron microscope analysis

Fibre mat cut-outs (diameter 10 mm), unloaded as well as loaded with zinc sulphate monohydrate before dissolution respectively, were mounted on 12.5 mm Cambridge stubs with Silver Dag (Taab Ltd), and sputter coated with 15 nm of iridium using a Quorum/Emitech K575X sputter coater. To visualize the fibre mats’ cross-section, samples were frozen in liquid nitrogen prior to being cross-fractured into suitable size by means of a cooled razor blade, and sputter coated. The fibre mats’ surfaces and cross-sections were viewed with a FEI Verios 460L Scanning Electron Microscope at 3 kV and 25 pA using a through lens detector operated in field free mode.

2.2.3. Energy dispersive X-ray analysis

Fibre mat cut-outs (diameter 10 mm), unloaded as well as loaded with zinc sulphate monohydrate before dissolution, were frozen in liquid nitrogen, and cross-fractured into suitable size using a cooled razor blade. The samples were subsequently mounted using carbon DAG, and carbon coated in a Quorum Q150 at 50 nm. Suitable areas for analysis were identified and imaged via SEM analysis as described above. To identify areas of zinc sulphate loading, additional images were taken at 20 kV and 100 pA using a concentric backscattered electron detector (CBS), with which areas of higher atomic number produce a brighter signal in the image. In order to confirm the presence or absence of zinc sulphate, spectra were collected for 50 s using an Ametec/EDAX Energy Dispersive X-ray (EDX) spectrometer with a silicon drift detector.

2.2.4. X-ray micro computed tomography

X-ray micro computed tomography (XµCT) of the entire fibre mat before dissolution was performed using a SkyScan 1172 high-resolution XµCT scanner (Bruker, Antwerp, Belgium), combining a cone beam geometry and a 2D array detector. By rotation of the object over 180° in steps of 0.25°, shadow projections are acquired, from which a 3D image can be obtained following tomographic reconstruction. This imaging method was chosen due to its high spatial resolution and negligible diffraction in the investigated samples. Imaging was performed at an isotropic voxel size of 2.49 µm, 10 images per position were averaged to reduce noise and increase the contrast. The total acquisition time per sample amounted to approximately 3 h, in which 720 images were recorded. Data obtained was reconstructed using NRecon (Bruker, v1.6.8.0), visualized in CTVox (Bruker, v3.3), and processed in Avizo Fire (FEI Company, Hillsboro, Oregon, USA, v8.1). Thresholding was applied to separate the solid material from the pore space for a subvolume (0.998 mm3) of the loaded and unloaded fibre mat. The volume and surface were calculated for the extracted solid material and the porosity was quantified by relating the volume of the pore space to that of the solid material.

2.2.5. Delivery and quantification

Calibration of the breastfeeding simulation apparatus’ pumps prior to any experimental work being undertaken, as well as monitoring of temperature and pressure throughout the breastfeeding simulation was performed. Delivery of zinc was conducted in triplicate for each type of fibre mat by means of the breastfeeding simulation apparatus (Gerrard et al., 2013). Based on literature about breastfeeding physiology (Black et al., 1998; Geddes et al., 2008; Macias and Meneses, 2011; Moral et al., 2010), and in accordance with previous work conducted using the simulation apparatus (Gerrard and Larson, 2013; Scheuerle et al., 2017), a flow rate of approximately 5.0 mL min-1, a suction frequency of 1 suction/s, and a temperature range of 33.7 - 35.7 °C, simulating the likely temperature of human milk in the infant’s mouth, were chosen for all experiments. The pressure range and amplitude were adjusted to values of physiological relevance (Geddes et al., 2008). Peak vacuum values of -22.78 kPa and -22.32 kPa as well as baseline vacuum values of -10.90 kPa and -11.38 kPa for fibre mat type A and B respectively were recorded. Figure 3 illustrates an exemplary average pressure profile for each type of fibre mat. All experiments were conducted in triplicate for each type of mat. The human breast mimic was set to an angle of 30° downwards from the vertical axis. Fractions were collected for a period of 40 s each with a total of 30 fractions, amounting to a total of 20 min. This duration corresponds to double the time of an average breastfeed (Geddes et al., 2008), and was chosen based on previously published breastfeeding simulation by Scheuerle et al. to allow for comparative analysis (Scheuerle et al., 2017). All samples collected by the fraction collector were weighted using a Sartorius analytic balance (Epsom, UK), and subsequently stored at -80°C until further analysis.

Both the overall amount of zinc loaded onto the fibre mats used for experimental procedures, as well as the zinc content in collected fractions during breastfeeding simulation, were evaluated using Inductively Coupled Plasma Optical Emission Spectrometry (ICP-OES) at the MRC Human Nutrition Research Unit (Cambridge, UK). Zinc detection in nine out of the 30 human milk samples was performed according to a protocol by Scheuerle et al., while the zinc concentration within the unmeasured human milk samples was subsequently evaluated by means of linear interpolation (Scheuerle et al., 2017). The amount of elemental zinc loaded onto the non-woven fibre mats was evaluated as follows: Fibre mats were added to a vial containing 1 mL rinse of ultra-high purity water with 0.002% TritonX100. Following two 10-minute rounds of ultra-sonication, all vials were placed on a shaker plate for gentle shaking at night, and 1 mL of rinse was added during the day. This process was performed for a total of three nights/days, following which the samples were held in a 40°C water bath over night. After a 1:10 volumetric dilution in ultra-purity water, samples were diluted 1:20 volumetrically in a diluent, consisting of ultra-high purity water (Sartorius Stedium Biotech Arium pro UV Water polisher) with 0.002 % TritonX100 (Sigma-Aldrich, UK) and a standard of 1 ppm molybdenum (SPEX CertiPrep Metuchen, NJ). All sample solutions were vortexed using a Fisherbrand Whirlimixer (Loughborough, UK), and subsequently analysed according to the ICP-OES protocol as described by Scheuerle et al. (Scheuerle et al., 2017). Data obtained was used to generate a release profile and to evaluate the cumulative zinc release.

3. Results and discussion

3.1. Scanning electron microscope analysis

SEM images of unloaded and zinc sulphate loaded Texel fibre mats type A and type B were obtained to explore the fibre mats morphology (see Figure 4 and Figure 5 for fibre mats’ surfaces, and Figure 6 and Figure 7 for the fibre mats’ cross-sections). It was found that despite their difference in g m-2 and thickness, both fibre mat types have the same fibrous structure and an approximate fibre diameter of 20 µm. Zinc sulphate was identified qualitatively by morphological analysis, indicating its adsorption in localized areas scattered across the fibrous mat network, either in form of flat patches or clumps. It forms a cracked, but mostly uniform coating on the fibre surface, caused by the evaporation of water from the loaded zinc sulphate solution, sometimes covering multiple fibres at once, while also accumulating within the porous fibre network. Due to the porous structure, adsorption within the deeper regions of the fibre mats can be observed.

3.2. Energy dispersive X-ray analysis

EDX results of unloaded and zinc sulphate loaded Texel fibre mats type A and type B, along with visualisation of the analysed area through SEM imaging, are shown in Figure 8 and Figure 9. The EDX analysis detected the elements C and O in loaded and unloaded fibre mats as a result of their material properties (viscose and polyester). Zn and S were only detected in loaded mat samples, confirming the absence of zinc sulphate in the unloaded fibre mats used for experimental procedures.

3.3. X-ray micro computed tomography

X-ray micro computed tomography (XµCT) was used to investigate the fibre mats’ porosity and 3D structure. Figure 10 and Figure 11 illustrate loaded and unloaded Texel fibre mats type A and type B, respectively. Different colours refer to material of different electron density as measured by XµCT, being proportional to true density values whenever the elemental composition of the sample is not changing. Based on the true density values of viscose (1.52 g cm-3) (Koc and Cincik, 2013), polyester (1.38 g cm-3) (Koc and Cincik, 2013), and zinc sulphate (3.54 g cm-3) (FisherScientific, 2014), the presented colour scale was chosen to illustrate unloaded fibre material of comparably lower density in red, and zinc sulphate of comparably higher density in blue and green colour. Figure 10 and Figure 11 depict 3D renderings of the XµCT data for unloaded and loaded fibre mats, respectively. Since the intensity of the XµCT is proportional to the true density of the material, regions of varying densities of the two different samples were highlighted by changing the opacity (see colour maps of the individual rendering). Hereby, images d) - e) depict both fibre material and zinc sulphate loaded onto the fibre mats, indicated by the mats’ red and a blue-red colour, while image f) solely displays loaded zinc sulphate, illustrated in blue and green colour. This becomes particularly apparent in direct comparison with the unloaded fibre mats in images a) - c), in which the visible fibrous material at medium density can only barely be observed as a faint silhouette, with mostly light blue spots of <10 μm and sporadically larger spots of <250 μm size visible. The bottom side of fibre mat type A, unlike fibre mat type B, exhibits a fibrous structure similar in appearance to those of unloaded fibre mats (see Figure 12). In agreement with the SEM analysis we found that the diameter of individual fibres was approximately 20 μm for unloaded or partially loaded fibres. Loading with zinc sulphate appears to result in swelling of individual fibres as well as fusing of adjoined fibres where they physically touch or where they are sufficiently close to come in contact during swelling. As a result, individual fibres are no longer identifiable in areas of high zinc loading. Figure 13 shows examples of representative cross-sectional cuts through a swollen fibre cluster. The volume fraction, porosity, and total surface area were calculated for a cubical region of interest measuring 0.998 mm3 from the fibre mats’ centre (Table 1). The decrease in porosity corresponds to an increase in solid fraction and consequently the volume of the solid increases in loaded fibre mats. The increase in volume is caused by the swelling of individual fibres as well as the deposit of zinc sulphate, which also leads to an increase in the surface area of the total solid material.

|  | Parameter | Unloaded | Loaded | Change [%] |
| --- | --- | --- | --- | --- |
| Fibre mat type A | Porosity [-] | 0.942 | 0.607 | - 36 |
|  | Surface area [mm2] | 19.72 | 32.70 | + 66 |
|  | Volume [mm3] | 0.058 | 0.392 | + 580 |
| Fibre mat type B | Porosity [-] | 0.939 | 0.416 | - 56 |
|  | Surface area [mm2] | 17.30 | 50.90 | + 194 |
|  | Volume [mm3] | 0.061 | 0.583 | + 856 |

Table 1: Overview of porosity, surface area, and volume of a cubical region of interest measuring 0.998 mm3 taken from the loaded and unloaded fibre mats’ centre.

3.4. Release of zinc from non-woven fibres

The average total elemental zinc content loaded onto the fibre mats amounted to 
6.17 ± 0.27 mg for fibre mat type A, and 6.4 ± 0.05 mg for fibre mat type B. The cumulative normalized proportion of zinc released and the normalized proportion of zinc released in individual fractions over time are illustrated in Figure 14. The amount of human milk passed through the TNS, as well as the absolute recovery achieved for both fibre mat types are presented in Table 2. Effectiveness and duration of release for both fibre mat types were comparable.

|  | Recovery achieved [%] | Amount of human milk passed through the device [g] |
| --- | --- | --- |
| Mat type A | 64.00 ± 0.01 | 93.09 ± 1.08 |
| Mat type B | 61.64 ± 0.01 | 98.15 ± 0.97 |

Table 2: Recovery achieved and human milk passed through the TNS for the delivery of zinc from Texel non-woven fibres using the breastfeeding simulation apparatus within the duration of 20 min (fractions of 40 s each with a total of 30 fractions).

Full delivery of zinc into human milk within the duration of one breastfeed, as previously demonstrated to amount to 76.0 ± 12.6 g of human milk for 1-month- to 6-months-old infants (Kent et al., 2006), has not been achieved. This could potentially be caused by milk components accumulating within the porous mat structure, thereby hampering the flow of milk through, and thus the delivery of zinc from the fibre network. The nature and absorption properties of the API significantly affect the release rate, however recent research appears to suggest that the lipid content and distribution within the milk influences the absolute recovery: Previous studies of SDS delivery from Texel non-woven fibres within a Sinnex filter holder into cow’s milk have shown that absolute release was decreased to 30 - 60 % when using non-homogenized milk (5.2 % fat) as opposed to 70 - 90% for use of an homogenized form (3.6 % fat)  (Gerrard et al., 2012). SDS release of approximately 100 % into non-homogenized goat’s milk (4.0 % fat) however, made Gerrard et al. conclude that in addition to the concentration and distribution of lipids, the overall milk composition, including carbohydrates, proteins, and lipids, has an impact on the API release rate from Texel non-woven fibres (Gerrard et al., 2012). Both observations present a challenge for the development of a standardized protocol for therapeutic release via non-woven Texel fibre mats, as human milk is a complex fluid with highly variable properties. To give but one example, absolute fat content of human milk can vary up to three fold, depending on the stage of lactation, dietary requirements and health status of the breastfeeding mother (Vir, 2011). Potential surface modifications, such as plasma treatment, use of the wet chemical method, or graft polymerization of the fibres’ surface (Ikada, 1994; Yoo et al., 2009), could provide a means to address unwanted milk components accumulation while enhancing adsorption properties, yet thereby also increasing manufacturing effort and cost of production. Moreover, the obtained XμCT results also suggest that changes in fibre conformation are likely impacting the release properties of the Texel fibre mats. Given the decrease in porosity as a result of the zinc loading process, flow of human milk through the fibrous structure is restricted following loading, and can further be impacted by the accumulation of milk components. Previously, it was reported that the release from low surface area to volume hydroxypropylmethylcellulose tablets is slower (Reynolds et al., 2002), and similarly, based on literature by Siepmann et al. (Siepmann and Siepmann, 2012), the release from zinc-loaded fibre clusters with their reduced surface to volume ratio is believed to be slowed down. The XμCT data of unloaded fibre mats revealed the presence of fibres of higher density than originally expected based on the densities of viscose and polyester. In addition, fibre accumulation and/or the presence of impurities can be observed as indicated by the 10 – 250 μm sized spots that are visible at medium density values in the XμCT data, believed to be a result of the fibres’ manufacturing process, and/or caused by trapped dust particles. The even distribution of zinc within the fibre mat network of fibre mat type B in contrast to mat type A (Figure 8) is attributed to the larger fibre diameter and different wetting behaviour of the two fibre mats. The liquid absorption of the fibre mats as well as of the individual fibres is driven by capillary action and strongly influenced by the porosity of the samples (Chen et al., 1995). It would also explain the unequal changes in porosity, surface area, and volume of fibre mat type A and B following loading, which however do not lead to an observed difference in release from both mats.

Overall, the absolute release of zinc from Texel non-woven fibres is 20 - 48% superior compared to rapidly disintegrating tablets manufactured by direct compression: In a study by Scheuerle et al. achieved absolute recovery amounted to only 32 - 51% (Scheuerle et al., 2017). It has to be noted that the total elemental zinc content of the rapidly disintegrating tablets (17.81 - 18.21 mg) was higher than the amount of elemental zinc loaded onto the Texel non-woven fibre mats used in this study (6.17 - 6.40 mg) (Scheuerle et al., 2017). Yet, while the main release from non-woven fibres occurred within the first 8 min, the release profile using rapidly disintegrating tables was linear, meaning that after 8 min the estimated elemental zinc content ranged between 2 mg and 4 mg (Scheuerle et al., 2017). This was significantly lower than the release from Texel non-woven fibre mats at 8 min. In addition to their superior release properties, the tested Texel non-woven fibre mats are characterized by enhanced loading feasibility and cost-effectiveness compared to the referenced rapidly disintegrating tablets, supporting their use as a dosage form of the TNS.

Important factors to be taken into consideration prior to in-vivo studies are fibre cohesion in order to exclude the possibility of fibre break-off, palatability, biocompatibility, and long-time stability. Moreover, use of different fibre materials, and different fibre types, such as fast-disintegrating electrospun fibres fabricated into non-woven film, could be investigated in future research.

4. Conclusions

Using a breastfeeding simulation apparatus to mimic the process of lactation and infant feeding, delivery of zinc sulphate from non-woven Texel fibre mats into human milk was investigated. With an absolute elemental zinc recovery of 64% (fibre mat type A) and 62% (fibre mat type B) respectively in 93 g and 98 g of human milk passed through the fibre insert, full delivery within one breastfeed (approximately 76 g) has not been achieved. This can be attributed to the complex properties of human milk and accumulation of milk components within the porous mat structures, as well as structural changes of the fibre networks following loading, thereby hampering the delivery of zinc. In comparison to previous experiments however, the absolute zinc recovery using non-woven Texel fibre mats was identified as superior to the release by means of rapidly disintegrating tablets (Scheuerle et al., 2017). Based on the non-woven fibre mats’ release characteristics and cost-effectiveness, non-woven fibres can be regarded as an advantageous dosage form for oral therapeutic delivery using the TNS. Further research will be required to establish Texel non-woven fibres as a generalized matrix for oral therapeutic delivery, and to further explore alternative fibrous materials with advantageous characteristics for API delivery into human milk.

Acknowledgements

This work was supported by the United States Agency for International Development (USAID), the Government of Norway, the Bill & Melinda Gates Foundation, Grand Challenges Canada and the UK Department for International Development (DFID) through the Saving Lives at Birth Award Scheme [grant number 0454-03]. Theresa Maier (T.M.) was supported through academic scholarships by the WD Armstrong Trust, University of Cambridge, and the German National Merit Foundation, Rebekah Scheuerle (R.L.S.) through a Gates Cambridge Trust scholarship, and Sylvaine Bruggraber (S.F.A.B.) through the Medical Research Council [grant number MC_US_A090_0008/Unit Program number U1059]. We thank Gillian Weaver, manager of the Queen Charlotte’s and Chelsea Hospital Milk Bank (Imperial College Healthcare NHS Trust) for enabling this study by providing human milk samples, Jeremy Skepper and Karin Muller (Cambridge Advanced Imaging Centre) for helping to conduct SEM and EDX analysis, and Stephen Gerrard for having previously built the breastfeeding simulation apparatus and developed the NDSD nipple shield design used in this study.

Competing interests

Co-author Rebekah Scheuerle is an inventor of three provisional patents related to the previous invention of the NSDS nipple shield design, US patent no. 8357117 B2  (US patent applications: No. 62/307,375, 62/337,805, and 62/424,006, all of which are called “Device and Method for Delivering an Agent into Breast Milk while Breastfeeding”). This does not affect the adherence to policies of the International Journal of Pharmaceutics.

References

Black, R.E., 2003. Zinc deficiency, infectious disease and mortality in the developing world. J. Nutr. 133, 1485–1489.

Black, R.F., Jarman, L., Simpson, J., 1998. The Science of Breastfeeding, 3rd ed. Jones and Bartlett Publishers.

Caulfield, L.E., Zavaleta, N., Shankar, A.H., 1998. Potential contribution of maternal zinc supplementation during pregnancy to maternal and child survival. Am. J. Clin. Nutr. 68, 499S–508S.

Chen, Y.-T., Davis, H.T., Macosko, C.W., 1995. Wetting of fiber mats for composites manufacturing: I. Visualization experiments. AIChE J. 41, 2261–2273. doi:10.1002/aic.690411009

Emmett, P.M., Rogers, I.S., 1997. Properties of human milk and their relationship with maternal nutrition. Early Hum. Dev. 49, S7–S28. doi:10.1016/S0378-3782(97)00051-0

FisherScientific, 2014. Safety Data Sheet - Zinc Sulfate. FisherScientific.

Geddes, D.T., Kent, J.C., Mitoulas, L.R., Hartmann, P.E., 2008. Tongue movement and intra-oral vacuum in breastfeeding infants. Early Hum. Dev. 84, 471–477. doi:10.1016/j.earlhumdev.2007.12.008

Gerrard, S.E., Baniecki, M.L., Sokal, D.C., 2012. A nipple shield delivery system for oral drug delivery to breastfeeding infants: Microbicide delivery to inactivate HIV. Int. J. Pharm. 434, 224–234. doi:10.1016/j.ijpharm.2012.05.035

Gerrard, S.E., Larson, A.M., 2013. Reducing infectivity of HIV upon exposure to surfaces coated with N, N-dodecyl, methyl-polyethylenimine. Biotechnol. Bioeng. 110, 2058–2062.

Gerrard, S.E., Orlu-Gul, M., Tuleu, C., Slater, N.K.H., 2013. Modeling the physiological factors that affect drug delivery from a nipple shield delivery system to breastfeeding infants. J. Pharm. Sci. 102, 3773–3783. doi:10.1002/jps.23688

Hambidge, K.M., Krebs, N.F., 2004. Fetal and Neonatal Physiology.

IGME, U., 2015. Levels and Trends in Child Mortality: Report 2015.

Ikada, Y., 1994. Surface modification of polymers for medical applications. Biomaterials 15, 725–736.

Kent, J.C., Mitoulas, L.R., Cregan, M.D., Ramsay, D.T., Doherty, D.A., Hartmann, P.E., 2006. Volume and frequency of breastfeedings and fat content of breast milk throughout the day. Pediatrics 117, 387–395. doi:10.1542/peds.2005-1417

Koc, E., Cincik, E., 2013. An analysis on abrasion resistance of polyester-/viscose-blended needle-punched nonwovens. J. Text. Inst. 104, 852–860.

Lopez, F.L., Ernest, T.B., 2015. Formulation approaches to pediatric oral drug delivery: benefits and limitations of current platforms. Expert Opin. Drug Deliv. 12, 1727–1740. doi:10.1517/17425247.2015.1060218

Macias, M.E.R., Meneses, G.J.S.M., 2011. Physiology of nutritive sucking in newborns and infants. Bol. Med. Hosp. Infant. Mex. 68, 296–303.

Mitoulas, L.R., Lai, C.T., Gurrin, L.C., Larsson, M., Hartmann, P.E., 2002. Efficacy of Breast Milk Expression Using an Electric Breast Pump. J. Hum. Lact. 18, 344–352.

Moral, A., Bolibar, I., Seguranyes, G., Ustrell, J.M., Sebastiá, G., Martínez-Barba, C., Ríos, J., 2010. Mechanics of sucking: comparison between bottle feeding and breastfeeding. BMC Pediatr. 10, 1–8. doi:10.1186/1471-2431-10-6

Reynolds, T.D., Mitchell, S.A., Balwinski, K.M., 2002. Investigation of the effect of tablet surface area/volume on drug release from hydroxypropylmethylcellulose controlled-release matrix tablets. Drug Dev. 28, 457–466.

Saarela, T., Kokkonen, J., Koivisto, M., 2005. Macronutrient and energy contents of human milk fractions during the first six months of lactation. Acta Paediatr. 94, 1176–1181. doi:10.1111/j.1651-2227.2005.tb02070.x

Scheuerle, R.L., Bruggraber, S.F.A., Gerrard, S.E., Kendall, R.A., Tuleu, C., Slater, N.K.H., 2017. Characterisation of zinc delivery from a nipple shield delivery system using a breastfeeding simulation apparatus. PLoS One 12, e0171624. doi:10.1371/journal.pone.0171624

Siepmann, J., Siepmann, F., 2012. Modeling of diffusion controlled drug delivery. J. Control. Release 161, 351–362. doi:10.1016/J.JCONREL.2011.10.006

Terrin, G., Canani, R.B., Chiara, M.D., Pietravalle, A., 2015. Zinc in early life: a key element in the fetus and preterm neonate. Nutrients 7, 10427–10446.

Vir, S.C., 2011. Public Health and Nutrition in Developing Countries. CRC Press.

World Health Organization, 2015. WHO Model List of Essential Medicines for Children 5th list.

World Health Organization, 2007. Promoting safety of medicines for children.

World Health Organization, 2017. Infant and young child feeding, Fact sheet N. 342. http://who.int/mediacentre/factsheets/fs342/en/. (accessed 02.01.16)

Wulf, K., Wilhelm, A., Spielmann, M., Wirth, S., 2013. Frequency of symptomatic zinc deficiency in very low birth weight infants. Klin. Paediatr. 225, 13–17.

Yoo, H.S., Kim, T.G., Park, T.G., 2009. Surface-functionalized electrospun nanofibers for tissue engineering and drug delivery. Adv. Drug Deliv. Rev. 61, 1033–1042.

Figure captions

Figure 1: Working principle of the TNS. The TNS is placed on the mother’s breast during breastfeeding, while the flow of human milk delivers the therapeutic to the sucking infant. (a) Nipple shield device, (b) Active Pharmaceutical Ingredient (API).

Figure 2: Fibres and nipple shield device used in the study. a - b) Illustration of non-woven fibre mat (a) type A and (b) type B, loaded with zinc sulphate. c - d) Lip-containing nipple shield device used for all breastfeeding simulation experiments.

Figure 3: Example pressure profiles during zinc delivery experiments using the breastfeeding simulation apparatus, depicted for 10 s at t = 120 - 130 s. Dosage forms used for the TNS were zinc-loaded fibre mats a) type A, and b) type B. Experiments were conducted using an average flow rate of approximately 5.0 mL min-1.

Figure 4: SEM results for fibre mat type A (fibre mat surface). SEM images illustrating the surface of Texel fibre mat type A (1.8 mm targeted thickness, 235 g m-2) at three different magnifications. Images a) - c) present unloaded fibre mats, images d) - f) fibre mats loaded with zinc sulphate before dissolution.

Figure 5: SEM results for fibre mat type B (fibre mat surface). Illustration of results obtained for SEM analysis of Texel fibre mat type B (2.1 mm targeted thickness, 335 g m-2) at three different magnifications. Images a) - c) present unloaded fibre mats, images d) - f) fibre mats loaded with zinc sulphate before dissolution.

Figure 6: SEM results for fibre mat type A (fibre mat cross-section). SEM images illustrating the cross-section of Texel fibre mat type A (1.8 mm targeted thickness, 235 g m-2) at two different magnifications. Images a) - c) present unloaded fibre mats, images d) - f) fibre mats loaded with zinc sulphate before dissolution.

Figure 7: SEM results for fibre mat type B (fibre mat cross-section). SEM images illustrating the cross-section of Texel fibre mat type B (2.1 mm targeted thickness, 335 g m-2) at two different magnifications. Images a) - c) present unloaded fibre mats, images d) - f) fibre mats loaded with zinc sulphate before dissolution.

Figure 8: EDX spectra and corresponding imaging results for fibre mat type A (fibre mat cross-section) before dissolution. Image a) and d) illustrate the EDX spectra, images b) and e) imaging of the EDX analysed fibre mat area via SEM, images c) and f) imaging of the EDX analysed fibre mat area via a concentric backscatter detector (CBS).

Figure 9: EDX spectra and corresponding imaging results for fibre mat type B (fibre mat cross-section) before dissolution. Image a) and d) illustrate the EDX spectra, images b) and e) imaging of the EDX analysed fibre mat area via SEM, images c) and f) imaging of the EDX analysed fibre mat area via a concentric backscatter detector (CBS).

Figure 10: Images generated from XμCT measurements of a) - c) unloaded, and d) - f) zinc sulphate loaded Texel fibre mat type A (1.8 mm targeted thickness, 235 g m-2). The colour maps were the same for all images and only the opacity was adjusted in order to visualize components of different densities. Red colour indicates low, green colour high density material. The images depict only half of the fibre mats in order to visualize the cross-section at the centre of each sample.

Figure 11: Images generated by XμCT analysis of a) - c) unloaded, and d) - f) zinc sulphate loaded Texel fibre mat type B (2.1 mm targeted thickness, 335 g m-2).

Figure 12: Images generated for XμCT analysis of loaded Texel fibre mats a) type A, b) type B (bottom side facing up).

Figure 13: Cut through zinc-loaded fibres in the y-z plane. Images a) - c) illustrate the resulting cross-sectional images at different position in the x-direction. Zinc-loaded fibres appear swollen and fused with adjoined fibres to form high-density fibre clusters.

Figure 14: Release of zinc sulphate from Texel fibre mats type A (1.8 mm targeted thickness, 235 g m-2) and type B (2.1 mm targeted thickness, 335 g m-2), as well as from rapidly disintegrating tablets by Scheuerle et al. (Scheuerle et al., 2017) into human milk using the breastfeeding simulation apparatus by Gerrard et al. (Gerrard et al., 2012). Experiments were conducted at 33.7 - 35.7°C, and a mean flow rate of approximately 5.0 mL min-1 was applied. a) Cumulative normalized proportion of zinc sulphate released and b) Normalized proportion of zinc sulphate released in each fraction over time. Release is represented as a percentage of the total amount loaded onto the non-woven fibre mats to the total amount released. Each set of points represents the average of three experiments, using the mean of ICP-OES triplicate measurements for each fraction shown.